3GPP TSG-RAN WG2 Meeting #128	R2-2409688

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	8.2.5

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues of AIoT TP2

Document for:	Discussion and Decision

Introduction

In RAN#102, a study item on solutions for Ambient IoT (Internet of Things) in NR has been agreed and in RAN#103, the SID is further refined in [1].

In RAN2#127 meeting, RAN2 has had initial discussion for TP2 in Ambient IoT. The following agreements have been achieved:

Agreements (Topology 2 considerations) - RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2. - A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device. - RAN2 assumes that intermediate UE authorization is performed by upper layers. RAN2 impact can be studied after further SA/RAN3 progress. - RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied) - NW is in control of resources to be used by AIoT air interface. - Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

In RAN2#127bis meeting, based on further discussion, the following agreements have been achieved:

Agreements - Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release. - The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells. - The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

In this paper, we will further discuss the remaining issues of supporting Topology 2 and give our proposals.

Discussion

  - The state of UE reader when performing AIoT operations

In previous meeting, there were some discussion about what the state of UE reader should be when it performs AIoT operations. The views from companies are diverse. More companies think RRC connected mode should be baseline and on need to consider other modes. Meanwhile, some other companies think inactive mode or even idle mode can also be considered.

Per our understanding, a UE reader is a normal UE and can act as a reader after being authorized and selected by core network. So the UE reader should have the capability to operate not only over legacy Uu interface but also over AIoT air interface. However, considering that for either legacy Uu interface or AIoT air interface, there will be both uplink and downlink transmission for a certain service operation (for legacy Uu interface, DL: BS->UE, UL: UE->BS; for AIoT air interface: DL: UE->device, UL: device->UE), and also considering limited UE capability and transmission power, we believe that requiring UE reader to perform full operations (UL and DL) simultaneously on both legacy Uu interface and AIoT air interface may be too high a requirement for a normal UE.

Observation 1a: Considering that for either legacy Uu interface or AIoT air interface, there will be both uplink and downlink transmission for a certain service operation, and also considering limited UE capability and transmission power, to require UE reader to perform full operations (UL and DL) simultaneously on both legacy Uu interface and AIoT air interface may be too high a requirement.

Therefore, we think one of the reasonable assumptions for TP2 is that at any given time, the UE reader can operate on only one of the interfaces, e.g., either legacy Uu interface or AIoT air interface. In other word, the UE reader cannot perform full operations (UL and DL) simultaneously over both legacy Uu interface and AIoT air interface. If needed, RAN2 can further check with RAN1 and/or RAN4 for this aspect.

Proposal 1a: From RAN2 perspective, a reasonable assumption can be that at any given time, a UE reader can work on only one interface, e.g., either legacy Uu interface or AIoT air interface.

Moreover, due to the limited multiplexing capability of devices, the inventory on multiple devices are typically performed via a TDM mode. So even if the AIoT operation performed by UE reader may involve a relatively small number of devices (e.g., smaller than that for a BS reader), the entire inventory process may still take a kind of long time. Thus, if the UE reader does not have any services to be performed in legacy Uu interface, requiring the UE reader to remain in the connected state for only performing AIoT operations might be unnecessary.

Observation 1b: Considering the inventory on multiple devices may take a kind of long time, requiring the UE reader to remain in the connected state for only performing AIoT operations (e.g., no other services in legacy Uu interface) might be unnecessary.

With the above considerations, we see no clear justification to give restriction on RRC state of UE reader when performing AIoT operations. All the connected/inactive/idle modes can be considered:

- If a UE reader is currently in connected mode over legacy Uu interface, when the UE reader is triggered (e.g., based on kind of network control or AIoT resources configuration) to perform AIoT operation, it’s feasible for the UE reader to keep in connected mode and start the AIoT operations over AIoT air interface. After finishing part or all the AIoT operations, the UE reader can include the collected/stored AIoT service responses in the transmission over legacy Uu interface and send them to CN/AF. The following sub-options can be further considered:

  - The network can indicate UE reader to only perform the AIoT operations within the inactive period of connected mode DRX cycle. The UE reader should stop AIoT operations when the inactive period ends regardless of whether all devices have been operated or not.

  - If the current service over legacy Uu interface is delay-tolerant, it’s also feasible for UE reader to request to stop the current data transmission in legacy Uu interface for a while. After confirmed by the network, UE reader can stop the data transmission and/or some of other connected mode tasks, e.g., neighbor cell measurements and start to perform AIoT operations.

    - In order not to miss the notification/(re)configuration/scheduling from the network, UE reader still needs to monitor the DL channel with a certain periodicity (maybe a kind of sparse cycle). Such periodicity can be configured by the network along with triggering UE reader to perform AIoT operations.

- If a UE reader is currently in idle or inactive state over legacy Uu interface and previously has been configured to perform AIoT operations, UE reader can start the AIoT operation over AIoT air interface at certain time point. Based on the capability of UE reader (e.g., for storing the received AIoT service responses), the UE can choose a suitable time point (e.g., after finishing part of or all the service operations) to initiate connection establishment procedure or SDT procedure over legacy Uu interface to send the stored AIoT service responses to CN/AF. Furthermore, in order to ensure the AIoT operation and the AIoT resources at UE reader are always under the control of the network, the following aspects need to be further considered:

  - Even UE reader can be allowed to perform AIoT operation during inactive/idle mode, the UE reader still needs to be configured/activated for this operation by the base station when the UE reader previously was in connected mode. For example, the UE reader in connected mode can be provided with AIoT resources via dedicated message (e.g., RRC re-configuration message, RRC release message) and network allowed such AIoT resources can be used (with conditions) after UE reader is back to inactive or idle modes.

  - Considering that the AIoT operations may take a relatively long time, during AIoT operations, UE reader still needs to monitor the DL channel for monitoring paging on the legacy Uu interface and even perform some measurements. So the base station can also configure kind of sparse cycle for paging monitoring and cell measurement/re-selection to the UE reader.

Proposal 1b: Under the network control, UE reader in any state, e.g., connected/inactive/idle, is allowed to perform AIoT operations over AIoT air interface. In order to minimize the impact on legacy Uu interface, when performing AIoT operations:

- For UE reader in connected state, mechanisms need to be considered to allow the UE reader to intermittently be back to active mode to perform (some) connected mode tasks on legacy Uu interface, e.g., monitoring PDCCH or performing measurements.

- For UE reader in idle or inactive state, mechanisms need to be considered to allow the UE reader to intermittently perform (some) idle mode tasks on legacy Uu interface, e.g., monitoring paging or performing measurements.

  - AIoT air interface resources configuration for UE reader

    - Assistant information for BS to allocate AIoT resources to UE reader

For TP2, it’s easy to understand that the information which needs to be put in the AIoT paging message would also be needed in the legacy Uu interface message for AIoT resources allocation. Taking into account the progress for paging and RA procedures in TP1, the following proposal is given:

Proposal 2a: With reference to the discussion on AIoT air interface for TP1, at least the following information needs to be considered to include in the legacy Uu interface message for AIoT resources allocation:

- The device ID information of the AIoT device(s)/selection criteria

- Radio resources information for D2R transmission and/or random access control information.

For TP1, some assistant information from CN has been agreed for assisting BS reader to allocate AIoT resources. With similar reasons and also considering that the base station controls the AIoT resources allocation in TP2, we assume such assistant information would be also needed for base station in TP2.

Proposal 2b: The following information that has been agreed to be obtained from CN and should be visible to the BS reader in TP1 is also needed to the base station in TP2 to assist AIoT resources allocation to UE reader:

- The service type of A-IoT (e.g. inventory, command). The more finer information on command type (e.g. read/write/disable) would also be useful;

- Targeted for one or more than one devices;

- Approximate number of target devices (if available).

    - Validity of AIoT resources allocated to UE reader

In previous RAN2 meetings, there were much discussion on how to allocate AIoT air interface resources to UE reader(s). The main agreement is that dedicated signaling can be used to give dedicated resource configuration to the UE reader(s).

Furthermore, there were some discussion whether and how to ensure the validity of the AIoT resources at UE reader. RAN2 only agreed that the AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc). The other options as below are still FFS:

- Option-1: the AIoT radio resource is applicable within validity area, e.g. per-cell, per-gNB or multiple gNB area.

- Option-2: the AIoT radio resource is applicable within validity time.

For UE reader performing AIoT operations in the connected state, it is easy to understand that the AIoT resources are typically valid within the current cell, and the validity duration can be the duration for AIoT operation, or the legacy RRC connection duration, or until resources reconfiguration (unless otherwise specified).

- It’s easy to understand such validity area and validity duration are also applicable when a connected mode UE reader is “temporarily” out-of-connection, e.g., RLF, that is, even if the UE reader is “temporarily” out-of-connection, as long as the AIoT resource is in the validity duration, the UE reader still can use it if it needs to perform AIoT operation. But if the UE reader finally re-establish to another new cell, the AIoT resources allocated by the source cell should be released in both UE reader and the source cell.

- Generally, the validity duration can configured by the network explicitly.

Proposal 3a: The validity area of the AIoT resources allocated to a UE reader in connected mode can be the current cell. And the validity duration can configured by the network explicitly. Such validity area and validity duration are also applicable when a connected mode UE reader is “temporarily” out-of-connection, e.g., RLF.

As mentioned in Proposal 1b, we also think UE reader in idle/inactive mode can perform AIoT operation, so it is more necessary to discuss the validity area and validity duration issues for UE reader in idle/inactive modes:

- Regarding the valid area, the straightforward assumption is the last cell, that is, the AIoT resources are only valid when the UE reader is in the last cell where the UE reader was allocated AIoT resources before its connection is released. During previous meeting discussions, some companies argued that due to the small coverage of cells in indoor scenario, it might be necessary to consider that the AIoT resource validity area can include multiple cells. However, it is obvious that supporting a cross-cell resources validity range would require coordination between cells, potentially leading to more standardization work. Hence, it is suggested not to consider this in the current release.

Proposal 3b: The validity area of the AIoT resources allocated to a UE reader in idle/inactive can be the “last cell”. That is, the AIoT resources are only valid when the UE reader is in the last cell where the UE reader was allocated AIoT resources.

- Regarding the validity time, a straightforward option is that the base station can additionally provide a validity duration and/or an epoch time for the AIoT resource when allocating it to the UE reader. The base station and UE reader need to keep a consistent understanding of this validity period and start time. The length of the validity duration can be adapted to the scale of targeted devices needing to perform AIoT operations.

Proposal 3c: The validity time of the AIoT resources allocated to a UE reader in idle/inactive can be represented by a validity duration and/or an epoch time of the AIoT resource which are configured the network explicitly along with AIoT resources allocation.

  - Selection of UE reader

In previous meeting, RAN2 has agreed the assumption that UE reader authorization is performed by upper layers. For this aspect, one straightforward way can be that, UE is authorized/de-authorized to act as reader by CN during registration/de-registration procedure. Furthermore, such authorization/de-authorization should be based on UE relevant capability.

Per our understanding, each time CN/AF initiates an AIoT service request, it can select the involved UE readers from the authorized ones and also be based on the association among CN mode, reader and the devices. However, as the CN may not be able to know timely whether a UE is currently suitable for being a reader, purely relying on the CN to select UE reader may be insufficient. Meanwhile, considering that in some scenarios the base station can have more up-to-date information on the connection existence or radio situation of a UE, or even UE’s preference, it should also allow the base station to involve or adjust UE reader selection.

Proposal 4a: As the base station may have more up-to-date information on the connection existence or radio situation of a UE, or even UE’s preference, it should also allow the base station to involve the UE reader selection.

If the involvement of base station on UE reader selection is allowed, the base station at least needs to know whether a UE has been authorized. So this information may need to be passed from the CN to the base station or reported by the UE to the base station, e.g., via UE assistance information report.

Proposal 4b: The information about whether a UE has been authorized to act as reader may need to be passed from the CN to the base station or reported by the UE to the base station.

Conclusion

This contribution gives the following observations and proposals for supporting TP2 for Ambient IoT:

Proposal 1a: From RAN2 perspective, a reasonable assumption can be that at any given time, a UE reader can work on only one interface, e.g., either legacy Uu interface or AIoT air interface.

Proposal 1b: Under the network control, UE reader in any state, e.g., connected/inactive/idle, is allowed to perform AIoT operations over AIoT air interface. In order to minimize the impact on legacy Uu interface, when performing AIoT operations:

- For UE reader in connected state, mechanisms need to be considered to allow the UE reader to intermittently back to active to perform (some) connected mode tasks on legacy Uu interface, e.g., monitoring PDCCH or performing measurements.

- For UE reader in idle or inactive state, mechanisms need to be considered to allow the UE reader to intermittently perform (some) idle mode tasks on legacy Uu interface, e.g., monitoring paging or performing measurements.

Proposal 2a: With reference to the discussion on AIoT air interface for TP1, at least the following information needs to be considered to include in the legacy Uu interface message for AIoT resources allocation:

- The device ID information of the AIoT device(s)/selection criteria

- Radio resources information for D2R transmission and/or random access control information.

Proposal 2b: The following information that has been agreed to be obtained from CN and should be visible to the BS reader in TP1 is also needed to the base station in TP2 to assist AIoT resources allocation to UE reader:

- The service type of A-IoT (e.g. inventory, command). The more finer information on command type (e.g. read/write/disable) would also be useful;

- Targeted for one or more than one devices;

- Approximate number of target devices (if available).

Proposal 3a: The validity area of the AIoT resources allocated to a UE reader in connected mode can be the current cell. And the validity duration can configured by the network explicitly. Such validity area and validity duration are also applicable when a connected mode UE reader is “temporarily” out-of-connection, e.g., RLF.

Proposal 3b: The validity area of the AIoT resources allocated to a UE reader in idle/inactive can be the “last cell”. That is, the AIoT resources are only valid when the UE reader is in the last cell where the UE reader was allocated AIoT resources.

Proposal 3c: The validity time of the AIoT resources allocated to a UE reader in idle/inactive can be represented by a validity duration and/or an epoch time of the AIoT resource which are configured the network explicitly along with AIoT resources allocation.

Proposal 4a: As the base station may have more up-to-date information on the connection existence or radio situation of a UE, or even UE’s preference, it should also allow the base station to involve the UE reader selection.

Proposal 4b: The information about whether a UE has been authorized to act as reader may need to be passed from the CN to the base station or reported by the UE to the base station.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, RAN#103, March, 2024

[2] 3GPP TR 22.840 v19.0.0: "Study on Ambient power-enabled Internet of Things", 3GPP SA1, Dec. 2023

[3] 3GPP TR 38.848 v18.0.0: "Study on Ambient IoT (Internet of Things) in RAN (Release 18)", RAN#101, Sep. 2023

[4] 3GPP TR 23.700-13 v1.1.0 (2024-10), “Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)”, October. 2024