3GPP TSG-RAN WG2 Meeting #128						R2-2409696

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.5.2

Source:	Samsung

Title:	On-demand SSB SCell Operation

Document for:	Discussion and Decision

# Introduction

The Rel-18 work item on network energy savings for NR specified techniques primarily for RRC Connected UEs such as SSB-less SCell operation, cell DTX/DRX mechanism, mechanisms to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, CHO procedure enhancement(s) etc.

Rel-19 work item aims to specify further network energy savings techniques such as on-demand SSB for SCell operation. WID of Rel-19 network energy saving specifies the following objective for on-demand SSB SCell operation:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

This contribution discusses the details on supporting on-demand SSB on SCell.

# Discussion

In the legacy NR operation, a UE can be configured with SSB based RRM measurement to discover SCell(s). UE reports the RRM measurement results to the gNB on the quality of the measured SSB. The gNB can further provide the SCell configuration by RRC to the UE. At the time of configuration, SCell can be initially activated or deactivated. If the SCell is deactivated, the gNB can activate it by SCell activation command by MAC CE. UE uses SCell’s SSB (or TRS if configured) for AGC and synchronization purpose to be ready for transmitting or receiving data on the activated SCell. After the SCell is activated, the UE can still use the SSB to get re-synchronized with the SCell, by its implementation.

An example of the legacy NR operation of a SCell is shown in Figure 1, wherein the SSB is transmitted by the SCell and is periodic. In case of SSB-less SCell, operation is same except that SSB measurements of a reference serving cell (SpCell or another SCell) are used for the SSB-less SCell.

Figure 1 Illustration of legacy NR procedure for the SCell transmitting SSB

The periodic transmission of SSB for the SCell operation affects the network energy consumption. For network energy savings following are agreed by RAN1 for SSB transmission on SCell:

  - Case #1: No always-on SSB on the cell. Upon on demand SSB trigger, SSBs are periodically transmitted.

  - Case #2: SSBs are periodically transmitted in SSB burst/window. On demand SSB can be additionally provided during the interval between SSB burst/window as and when needed. Note that periodic transmission can be at a longer periodicity to reduce energy consumption but affects performance ((re)sync/AGC delay, etc). Additional on demand SSB can improve performance.

2.1 OD-SSB based RRM measurements

RAN1 has also agreed that for a cell supporting on-demand SSB SCell operation, L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

RAN2#127bis

- Whether always-on SSB and/or OD-SSB are measured in case 2 (Postponed)

    - Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

    - Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

SSBs transmitted during periodic transmission and on demand SSB transmission may be same or different. Considering that periodic transmission can be at a longer periodicity to reduce energy consumption but affects performance ((re)sync/AGC delay, etc), additional on demand SSB can improve performance. So, it may be beneficial to follow option b). However, if the frequency resources for on demand SSB and periodic SSB are different, it may not be possible for UE to measure both. RAN1/RAN4 are discussing this aspect. So RAN2 can wait for RAN1/RAN4 to further discuss whether always-on SSB and/or OD-SSB are measured in case 2.

Proposal 1: Wait for RAN1/RAN4 to further discuss whether always-on SSB and/or OD-SSB are measured in case 2.

On L3 RRM Framework (Postponed) RAN2 discussed the following options in RAN2#127bis:

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

Using only on demand SSB configuration to perform measurement is not sufficient. UE also need to know the measurement reporting configuration. With the legacy RRM measurement frame work, measurement object is linked to measurement reporting are linked by measurement id. So, we prefer to use legacy measurement framework for OD-SSB measurement and reporting.

Currently, for L3 measurement (i.e. RRM), SCell can be configured with a measurement object for always on periodic SSB. It is indicated by field servingCellMO (of type MeasObjectId).

For supporting L3 measurement (i.e. RRM) based on on-demand SSB, SCell can be configured with a measurement object for on demand SSB. It can be indicated by new field onDemandSSBservingCellMO (of type MeasObjectId). UE performs L3 measurement (i.e. RRM) based on onDemandSSBservingCellMO only when on-demand SSB is activated/transmitted.

Proposal 2: For supporting L3 measurement (i.e. RRM) based on on-demand SSB, SCell can be configured with a measurement object for on demand SSB. It can be indicated by new field onDemandSSBservingCellMO (of type MeasObjectId).

2.2Signalling for on demand SSB activation/deactivation:

RAN2#127bis agreements

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Following options can be considered for MAC CE design:

Option 1 (Figure 1):

A field is set to 1 to indicate that on demand SSB for SCell (indicated by Serving Cell ID) is activated/triggered/transmitted. A field is set to 0 to indicate that on demand SSB for SCell is deactivated/not transmitted. Additional information (e.g. SSB pattern info etc) can be included based on RAN1 decision.

Figure 1

Option 2 (Figure 2):

SCells configured with on demand SSB are sequentially indexed in increasing order of ServCellIndex.

A single octet Cj bitmap is included if the total number of SCells for which on demand SSB is configured  is greater than 0 and less than 8; a two octets Cj bitmap is included if the total number of SCells for which on demand SSB is configured is greater than 7 and less than 16; a three octets Cj bitmap is included if the total number of SCells for which on demand SSB is configured is greater than 15 and less than 24; a four octets Cj bitmap is included if the total number of SCells for which on demand SSB is configured is greater than 23;

Cj field is set to 1 to indicate that on demand SSB for jth SCell configured with on demand SSB is activated/triggered/transmitted. Cj field is set to 0 to indicate that on demand SSB for jth SCell configured with on demand SSB is deactivated/not transmitted.

Additional information (e.g. SSB pattern info etc) can be included based on RAN1 decision.

Figure 2

Option 3 (Figure 3 and 4):

Cj field is set to 1 to indicate that on demand SSB for SCell identified by ServCellIndex is activated/triggered/transmitted. Cj field is set to 0 to indicate that on demand SSB for SCell identified by ServCellIndex j is deactivated/not transmitted.

Additional information (e.g. SSB pattern info etc) can be included based on RAN1 decision.

Figure 3 (highest ServCellIndex or SCellIndex for which on demand SSB is configured is < 8)

Figure 4 (highest ServCellIndex or SCellIndex for which on demand SSB is configured is >= 8)

Option 1 is the simplest but cannot activate/deactivate OD-SSB for multiple SCells simultaneously. This may result in more signalling overhead.

Option 3 can activate/deactivate OD-SSB for multiple SCells simultaneously requires. However, two MAC CEs are needed resulting in more specification work, consumption of more eLCID). It also results in more signaling overhead when highest ServCellIndex for which on demand SSB is configured is >= 8 as 4 octets are always included.

Option 2 can activate/deactivate OD-SSB for multiple SCells simultaneously. It requires only one MAC CE. It is also efficient from signalling overhead perspective as number of octets depends on number of SCells which supports OD-SSB.

Proposal 3: RAN2 to agree on option 2 for OD-SSB activation/deactivation MAC CE.

# Conclusion

Proposal 1: Wait for RAN1/RAN4 to further discuss whether always-on SSB and/or OD-SSB are measured in case 2.

Proposal 2: For supporting L3 measurement (i.e. RRM) based on on-demand SSB, SCell can be configured with a measurement object for on demand SSB. It can be indicated by new field onDemandSSBservingCellMO (of type MeasObjectId).

Proposal 3: RAN2 to agree on option 2 for OD-SSB activation/deactivation MAC CE.

# References