3GPP TSG-RAN WG2 Meeting #128      	       	R2-2409710

Orlando, USA, 18th- 22nd Nov 2024

Source:	vivo

Title:	Discussion on Topology 2 related aspects

Agenda Item:	8.2.5

Document for:	Discussion and Decision

- Introduction

Topology 2 with UE as intermediate node under NW control is in the scope of the ambient IoT SID [1]. In the previous meeting RAN2 achieved the following topology 2 related agreements [2]:

RAN2#127bis Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

In this contribution, we will discuss topology 2 related aspects, e.g., validity of resources and UE reader behavior, architecture impacts,etc.

- Discussion

## 2.1 Validity of resources and UE reader behavior

Dedicated AIoT radio resources are configured to the UE via dedicated signaling. As agreed in the previous meeting, “The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.” The validity of AIoT radio resources should be further clarified.For example, resource validity can be considered in twofold:

- Validity timer, period/duration within it the configured resources can be used.

- Area or scope where the configured resources are valid.

Furthermore, we consider whether the AIoT radio resource is allowed to be used when UE Reader is out-of-connection, or is moving across multiple cells.

Regarding the AIoT radio resource validity in temporary out-of-connection, to address the concern of allowing UE reader to continuously occupy/use the dedicated resource, validity time can be introduced. As Figure 1 shows, the UE reader starts a validity timer upon the dedicated AIoT radio resources are configured. The dedicated AIoT radio resources are allowed to be used for AIoT communication within the validity time. This validity time can be predefined in the specification or configured by the network. Furthermore, the dedicated AIoT radio resources can be reconfigured if needed, upon which the validity timer can be restarted. Furthermore, the validity timer is not stopped when entering temporary out-of-connection. In this way, the using of the A-IoT radio resource is under network control.

- A-IoT radio resource validity in Topology 2

Therefore, we propose:

- The AIoT radio resource is applicable within a predefined or configured time period. The UE reader starts/restarts the validity timer upon reception of the (re-)configured AIoT radio resources.

Regarding the resource validity across multiple cells, validity area of the configured dedicated resources can be defined, e.g.:

- Per cell:

In our understanding, it is simple to only use the dedicated resources within the cell which provides the dedicated AIoT resources to the UE reader. The validity area per cell can be supported as baseline.

- Accross multiple cells:

If the dedicated resources can be used across multiple cells, the UE reader can continue the AIoT procedure even after handover or reselect to other cells. Considering the network deployment, these multiple cells may be

  - Intra-gNB cells. Multiple cells under one gNB may allocate a same set of resources for one UE reader, then the gNB allows the UE to use the configured AIoT resources when the UE reader is moving among different cells of the same gNB. This method can improve the continuity of AIoT procedure within the gNB coverage, and avoid redundant resource reconfiguration by the same gNB.

  - Inter-gNB cells. It needs extra coordination between gNBs to allocate AIoT resources for the UE reader, RAN3 study is needed. Further consideration on the AIoT resource deployment among gNBs, e.g., the co-existence of AIoT resources between different gNBs, is required involving RAN1/RAN4.

Then we propose that:

- The AIoT radio resources are applicable within a validity area, including per cell and intra-gNB.

The AIoT service efficiency is an important KPI for user experience, especially the AIoT service performance when UE reader goes out of connection temporarily. Based on validity determination, UE reader can be allowed to continue using the allocated AIoT resources within the validity time or validity area even when it goes to temporarily out of connection, such as a short RLF. If after sending Msg2 UE reader experience RLF, UE reader should be allowed to properly monitor and receive Msg3. During “temporarily” out of connection, it is beneficial that the AIoT AS procedures (if triggered) will not stop but continue, such that the UE reader can complete the AIoT procedures even temporarily out of connection.

- In temporary out-of-connection, UE reader should be allowed/able to perform/continue the AIoT procedure on AIoT interface with AIoT devices if the AIoT resources are valid according to resource validity timer and validity area.

If UE reader continues performing AIoT procedures during temporary out-of-connection, the AIoT data collected by the UE can still be reported to the network as soon as UE recovers from the temporary out-of-connection.

- The UE Reader can resume the AIoT data reporting, if needed, as soon as it recovers from the temporary out-of-connection.

## 2.2 Impacts based on architecture discussions in SA2/RAN3

Another key issue in Topology 2 is the protocol stack architecture and how UE reader transports AIoT upper layer information. In RAN3, potential solutions below are to be studied [3]:

6.3.2.1 Solutions for Topology 2 To support Topology 2, the following solutions are to be studied for conveying AIoT upper layer information: - RRC based solution. With this solution, AIoT CN applies AIoT upper layer information explicitly over XXAP signaling. AIoT upper layer information is then relayed explicitly to/from the AIoT-enabled UE via NR Uu RRC. - NAS based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted over AIoT-enabled UE's NAS. - UP based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted as AIoT-enabled UE's user plane data.

Currently, the basic procedure of RRC based solution is available in TR[3], RAN2 may further study the detailed impacts of RRC based solution based on candidate protocol stack architectures. However, the whole picture of UP and NAS based solutions are still under discussion because direct communication between A-IoT CN and A-IoT-enabled UE for NAS/UP based solution is not consolidated. In SA2, there are still several potential architectures for each solution (RRC/UP/NAS based) on the table for down-selection.

- NAS/UP based solutions and architectures are not ready for RAN2 to study their impact.

Considering UP based solution has less impact on RAN2, we may begin the impact study work from UP and RRC based solution first, but it is not appropriate to analyze the impact of NAS based solution for the time being.

For example, below potential RAN2 related impacts rely on the selected architectures.

- Resource allocation

  - Alt1: UE reader requests for AIoT radio resources. The UE reader needs to request AIoT enabled gNB for AIoT radio resources for this AIoT Service. Then the gNB allocates AIoT radio resources for the UE reader.

  - Alt2: AIoT CN requests the gNB to allocate AIoT radio resources before AIoT transaction.

- AIoT data transport

  - AIoT data transport signaling via RRC, NAS or UP based solutions.

  - the AIoT-enabled UE(s) may send one or multiple inventory reports towards the AIoT CN including the received inventory results.

After the SA2 architecture discussion is stable and RAN3 skeleton description is available, we think it would be feasible for RAN2 to study the impact of all potential architectures. Considering the SI timeline is coming to an end, we may study the RAN2 impact in WI phase. Therefore, we propose that:

- RAN2 to study the impact of architectures in WI phase with high priority on UP/RRC based solutions.

- Conclusion

In this contribution, we have the following observations:

- NAS/UP based solutions and architectures are not ready for RAN2 to study their impact.

Based the above discussions and observations, we have the following proposals:

Validity of resources

- The AIoT radio resource is applicable within a predefined or configured time period. The UE reader starts/restarts the validity timer upon reception of the (re-)configured AIoT radio resources.

- The AIoT radio resources are applicable within a validity area, including per cell and intra-gNB.

- In temporary out-of-connection, UE reader should be allowed/able to perform/continue the AIoT procedure on AIoT interface with AIoT devices if the AIoT resources are valid according to resource validity timer and validity area.

- The UE Reader can resume the AIoT data reporting, if needed, as soon as it recovers from the temporary out-of-connection.

Architecture Impact

- RAN2 to study the impact of architectures in WI phase with high priority on UP/RRC based solutions.

- Reference

- RP-240826 Study on solutions for Ambient IoT (Internet of Things) in NR.

- RAN2#127bis Chairman Notes.

- R3-244837 (TP for TR 38.769) RAN architecture aspects. Huawei.