3GPP TSG RAN WG2 #128                                                                                   R2-2409716

Orlando, USA, Nov 18th – Nov 22nd, 2024

Source:	CATT

Title:	Discussion on LCM for UE-sided model for BM use case

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#128 meeting, RAN2 discussed the LCM for UE-sided model for beam management use case and achieved the agreements as follows [1].

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

After RAN2#127 meeting, RAN2 sent an LS to RAN1 to provide RAN2’s agreements and asked specific questions that RAN2 needs to enable further progress. In this contribution, we will focus on the issues that are not dependent on RAN1’s reply and issues we haven’t yet discussed (e.g. necessary signalling/protocols to configure the UE for training, etc).

- Discussion

  - Non-applicable functionalities reporting

In RAN2#127bis meeting, RAN2 discussed and agreed that UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). In RAN2#127 meeting, RAN2 agreed that UE reports applicable functionality upon change of applicable functionality via UAI. Based on the agreement, when an applicable functionality becomes non-applicable, the applicable functionality set change occurs. Thus applicable functionality reporting is triggered to inform network the new applicable functionality set and the UAI is used as the reporting signaling. Based on the reporting, network can get to know which functionality becomes non-applicable from applicable functionality compared with previous reported applicable functionality set implicitly.

Proposal 1: When an applicable functionality becomes non-applicable, it can be reported to network implicitly by triggering the applicable functionality reporting via UAI.

Another issue is whether it is regarding an active functionality, i.e., whether only when active functionality becomes non-applicable functionality can be reported to network. We understand any applicable functionality becomes non-applicable can be reported to network, so that the network can get to know which functionality can be configured with inference configuration in Step 5.

Proposal 2: When any applicable functionality becomes non-applicable, it can be reported to network, i.e., not only the active functionality.

After RAN2#126 meeting, during the email discussion [2], an issue was discussed that whether to define available functionalities separately from applicable functionalities, but it is still FFS. In [3], the related issue was discussed that whether UE can include non-applicable functionalities in Step 4 (i.e., along with reporting applicable functionalities), which is that UE can report the functionalities for which a model is available in device, but is not currently applicable. From our perspective, the intention of reporting the non-applicable functionalities is not clear, e.g., it is uncertain that reporting the non-applicable functionalities in step 4 is for the network to provide the configuration (such as the NW-side additional condition) to make the non-applicable functionalities change to applicable functionalities. If this is the purpose, RAN2 needs to check whether it is feasible.

Proposal 3: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities with available model to network, e.g., for network to update NW-side additional condition.

In addition, if there is no model available in device for a functionality, whether the UE can also report the functionality without an available model to network for e.g., model transfer needs to be discussed, which is also related to model transfer discussion.

Proposal 4: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities without available model to network, e.g., for model transfer.

A related issue is if it is supported that the UE can report the non-applicable functionalities, should it provide the non-applicable functionalities when it doesn’t have applicable functionality that can be reported upon a change in applicable functionality or upon receiving a network request for applicable functionalities, or should the UE report the non-applicable functionalities as long as there are non-applicable functionalities? This can be discussed further based on Proposal 3 and Proposal 4.

Another related issue is what behavior of UE should be when it doesn’t have applicable functionality that can be reported upon a change in applicable functionality or upon receiving a network request for applicable functionalities. Should it report empty content or should some other assistant information (such as non-applicable functionalities) be sent to the network for model transfer or providing NW-side additional condition to UE.

Proposal 5: RAN2 to discuss the UE behaviors when no applicable functionality can be reported upon change of applicable functionality or upon receiving the network request, e.g., report empty content or reporting other assistant information (e.g., non-applicable functionalities).

  - Neighbouring cell’s applicable functionality reporting

In [3], an issue was proposed by companies that whether neighbouring cell’s applicability needs to be considered in applicable functionality reporting to serving cell. Based on the agreements in RAN2#127 meeting, it seems that the NW-side additional condition needs to be provided to UE from network for determining the applicable functionalities in UE side, it can be e.g., based on associated ID to indicate the NW-side additional condition. The relevant issues had been added in LS to RAN1 and we can wait for RAN1’s reply.

In addition, in RAN1#118 meeting, the agreement was achieved as follow.

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Based on the highlighted in yellow part, the associated ID is in cell level, and FFS whether it can applicable for other cells, RAN1 will continue to discuss the FFS issue which has the impact on neighbouring cell’s applicable functionality reporting.

Considering that the NW-side additional condition related issue is under RAN1 discussion, it is too early to discuss the neighbouring cell’s applicable functionality reporting issue. Therefore, we suggest that RAN2 to postpone the discussion on neighbouring cell’s applicable functionality reporting until the NW-side additional condition is clear.

Proposal 6: RAN2 to postpone the discussion on neighbouring cell’s applicable functionality reporting until the NW-side additional condition is clear.

  - Configuration for training data collection

In last meeting, RAN2 discussed the issue and agreed that data collection initiation and configuration is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)). For UE-side model, we understand UE knows more model related information than network and it is reasonable to allow UE to request the data collection configuration for training. Also, the network can reject to provide the configuration to UE if it does not expect it.

Proposal 7: For BM use case for UE-side model, UE can request data collection configuration for training from the network.

Proposal 8: For BM use case for UE-side model, network can accept or reject the data collection configuration for training request from UE.

In RAN1#116bis meeting, RAN1 discussed and agreed the following agreement for UE-side model(s) developed (e.g., trained, updated) at UE side.

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

Based on the description highlighted in yellow, Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a solution for resolving the consistency for functionality. In step A, i.e., training data collection configuration procedure, network needs to signal the data collection related configuration(s) and associated ID(s) to UE.

Proposal 9: For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID can be included in training data collection configuration.

For the data collection related configuration(s), e.g., measurement resources configuration etc., we understand it is similar as the inference configuration except for it does not include the inference results reporting configuration. The detail contents in the data collection related configuration(s) can wait for RAN1’s input for inference configuration.

Proposal 10: For BM use case for UE-side model, the details contents of data collection related configuration(s) is left to RAN1.

- Conclusion

According to the discussion in section 2, we propose:

Non-applicable functionalities reporting

Proposal 1: When an applicable functionality becomes non-applicable, it can be reported to network implicitly by triggering the applicable functionality reporting via UAI.

Proposal 2: When any applicable functionality becomes non-applicable, it can be reported to network, i.e., not only the active functionality.

Proposal 3: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities with available model to network, e.g., for network to update NW-side additional condition.

Proposal 4: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities without available model to network, e.g., for model transfer.

Proposal 5: RAN2 to discuss the UE behaviors when no applicable functionality can be reported upon change of applicable functionality or upon receiving the network request, e.g., report empty content or reporting other assistant information (e.g., non-applicable functionalities).

Neighbouring cell’s applicable functionality reporting

Proposal 6: RAN2 to postpone the discussion on neighbouring cell’s applicable functionality reporting until the NW-side additional condition is clear.

Configuration for training data collection

Proposal 7: For BM use case for UE-side model, UE can request data collection configuration for training from the network.

Proposal 8: For BM use case for UE-side model, network can accept or reject the data collection configuration for training request from UE.

Proposal 9: For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID can be included in training data collection configuration.

Proposal 10: For BM use case for UE-side model, the details contents of data collection related configuration(s) is left to RAN1.

- References

- Draft_RAN2_127b_Meeting_Report_v1.docx

- [POST126][032][AIML PHY] LCM (Intel, Samsung) – phase1

- [POST126][032][AIML PHY] LCM (Intel, Samsung) – phase2