3GPP TSG-RAN WG2 Meeting #128                                                                    R2-2409719

Orlando, U.S.A, Novermber 18th – 22nd, 2024

Source:	CATT

Title:	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting, RRM relaxation was discussed and the following work assumption was achieved:

- Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

In this contribution, we provide our further considerations on RRM relaxation and offloading in RRC_IDLE/INACTIVE.

Discussion

Relationships among conditions

According to RAN4 agreements, the following cases were agreed:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Until now, RAN2 has discussed LP-WUS monitoring conditions, MR RRM measurement relaxation (i.e. case #3 Relaxed cased b in RAN4) conditions, and MR serving cell fully offloading (i.e. case #1 Fully offloading case in RAN4) conditions.

For LP-WUS monitoring conditions, we have agreed:

- Entry condition: based on MR and optional LR.

- Exit condition: based on LR.

For MR serving cell fully offloading conditions, we have agreed:

- Entry condition: based on MR and optional LR.

- Exit condition: based on LR.

If conditions for these cases, including LP-WUS monitoring, MR RRM measurement relaxation and MR serving cell fully offloading, are configured in a cell, we would like to clarify their relationships at first. It is straightforward that conditions for MR RRM measurement relaxation should be better or equal to those for LP-WUS monitoring, and conditions for MR serving cell fully offloading should be better than those for MR RRM measurement relaxation, as shown in Figure 1.

Figure 1 One example of relationships among different conditions

Proposal 1: Conditions for MR RRM measurement relaxation should be better or equal to those for LP-WUS monitoring, and conditions for MR serving cell fully offloading should be better than those for MR RRM measurement relaxation.

MR RRM measurement relaxation

- MR serving cell measurement relaxation

Similar as LP-WUS conditions and MR serving cell fully offloading conditions, discussions on conditions for MR serving cell measurement relaxation are separated into entry condition and exit condition.

For entry condition of MR serving cell measurement relaxation, the possible options can be:

- Option 1: Based on MR and optional LR.

- Option 2: Based on both MR and LR.

- Option 3: Based on LR and optional MR.

- Option 4: Based on MR only.

- Option 5: Based on LR only.

According to existing agreements, both entry condition of LP-WUS monitoring and that of MR serving cell fully offloading are based on MR and optional LR. Entry condition of MR serving cell measurement relaxation can follow the same principle.

Proposal 2: Entry condition of MR serving cell measurement relaxation is based on MR and optional LR.

For exit condition of MR serving cell measurement relaxation, the possible options can be:

- Option 1: Based on LR.

- Option 2: Based on LR or MR, but MR is optional configured.

- Option 3: Based on LR or MR, but LR is optional configured.

- Option 4: Based on LR or MR.

- Option 5: Based on MR.

Option 1 is similar to exit conditions for both LP-WUS monitoring and MR serving cell fully offloading. But different from LP-WUS monitoring and MR serving cell fully offloading, the need to perform cell reselection based on MR serving cell and MR neighbor cell should be considered during MR serving cell measurement relaxation. It is doubtful whether the network can always configure a suitable LR threshold for the exit condition of MR serving cell measurement relaxation to ensure that MR serving cell measurement result is good enough before the exit condition of MR serving cell measurement relaxation is fulfilled. If it is possible that MR serving cell measurement result maybe not good enough before the exit condition of MR serving cell measurement relaxation is fulfilled, it is safer to consider MR serving cell measurement result as an optional configured exit condition of MR serving cell measurement relaxation. When both LR and MR are configured as exit conditions of MR serving cell measurement, no matter LR or MR condition is fulfilled, the exit condition of MR serving cell measurement is fulfilled. Therefore, we propose RAN2 to discuss which option is adopted for MR serving cell measurement relaxation, i.e., Option 1: Based on LR or Option 2: Based on LR or MR, but MR is optional configured.

Proposal 3: RAN2 discuss which option is adopted for exit condition of MR serving cell measurement relaxation:

- Option 1: Based on LR;

- Option 2: Based on LR or MR, but MR is optional configured.

- MR neighbor cell measurement relaxation

In RAN2#127bis meeting, we had the working assumption that RAN2 do not define additional MR-based criterion over the R16 criteria for neighbor cell measurement relaxation for UEs capable of LP-WUS. But different from R16 criteria, MR serving cell may be relaxed as well. MR serving cell measurement results cannot be updated timely during MR serving cell relaxation. Therefore, for neighbor cell measurement relaxation for UEs capable of LP-WUS, LR needs to be considered together.

Proposal 4: Apart from MR, LR needs to be considered for neighbor cell relaxation for UEs capable of LP-WUS.

And similar as LP-WUS monitoring conditions and MR serving cell fully offloading conditions, conditions for neighbor cell relaxation can be:

- Entry condition: based on MR and optional LR.

- Exit condition: based on MR or LR.

Proposal 5: Conditions of MR neighbor cell measurement relaxation for UEs capable LP-WUS:

- Entry condition: based on MR and optional LR.

- Exit condition: based on MR or LR.

MR Serving cell fully offloading

- MR neighbor cell measurement

In RAN4#111 meeting, RAN4 agreed the MR serving cell fully offloading case as follows.

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON.

According to the agreed case#1 fully offloading case in RAN4, it is not required to perform MR neighboring cell measurement either. In our understanding, MR neighbor cell measurements on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority are not required when MR serving cell measurement is offloading to LR. But whether MR neighbor cell measurements on NR inter-frequency cells of higher priority, or inter-RAT frequency cells of higher priority can be stopped depends on RAN4’s further discussion.

Proposal 6: When the entry condition for serving cell measurement offloading is fulfilled (i.e., there is no serving cell measurement by MR), it is not required to perform measurement for MR neighbor cell on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

- Relationship between MR Serving cell fully offloading and MR neighbor cell measurement off in legacy

In legacy, if the MR serving cell is good enough (e.g. in cell center), the UE may not perform neighbor cell measurements including [1]:

-	If the serving cell fulfills Srxlev > SIntraSearchP (i.e. s-IntraSearchP broadcast in SIB2) and Squal > SIntraSearchQ (i.e. s-IntraSearchQ broadcast in SIB2), the UE may not perform intra-frequency measurements.

-	If the serving cell fulfills Srxlev > SnonIntraSearchP (i.e. s-NonIntraSearchP broadcast in SIB2) and Squal > SnonIntraSearchQ (i.e. s-NonIntraSearchQ broadcast in SIB2), the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

In RAN2#127 meeting, RAN2 agreed that entry conditions for MR serving cell fully offloading (i.e., there is no serving cell measurement by MR) can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. Entry conditions for serving cell measurement offloading should not be worse than conditions for legacy MR neighbor cell measurement off on intra-frequency, NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority, considering serving cell measurement offloading should be careful. Therefore, MR RSRP threshold in entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchP and s-NonIntraSearchP, as shown in Figure 2. And before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for MR neighbor cell measurement off no matter if MR serving cell measurement is relaxed or not, i.e. if the MR serving cell fulfills Srxlev > SIntraSearchP (i.e. s-IntraSearchP broadcast in SIB2) and Squal > SIntraSearchQ (i.e. s-IntraSearchQ broadcast in SIB2), the UE may not perform MR intra-frequency measurements; If the MR serving cell fulfills Srxlev > SnonIntraSearchP (i.e. s-NonIntraSearchP broadcast in SIB2) and Squal > SnonIntraSearchQ (i.e. s-NonIntraSearchQ broadcast in SIB2), the UE may choose not to perform MR measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

Proposal 7: MR RSRP threshold in the entry conditions for serving cell measurement offloading (i.e., there is no serving cell measurement by MR) should be better than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 8: Before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Figure 2: One example of the serving cell measurement offloading coverage

Conclusion

According to the analysis in Section 2, we propose:

Relationships among conditions:

Proposal 1: Conditions for MR RRM measurement relaxation should be better or equal to those for LP-WUS monitoring, and conditions for MR serving cell fully offloading should be better than those for MR RRM measurement relaxation.

MR RRM measurement relaxation:

Proposal 2: Entry condition of MR serving cell measurement relaxation is based on MR and optional LR.

Proposal 3: RAN2 discuss which option is adopted for exit condition of MR serving cell measurement relaxation:

- Option 1: Based on LR;

- Option 2: Based on LR or MR, but MR is optional configured.

Proposal 4: Apart from MR, LR needs to be considered for neighbor cell relaxation for UEs capable of LP-WUS.

Proposal 5: Conditions of MR neighbor cell measurement relaxation for UEs capable LP-WUS:

- Entry condition: based on MR and optional LR.

- Exit condition: based on MR or LR.

MR Serving cell fully offloading:

Proposal 6: When the entry condition for serving cell measurement offloading is fulfilled (i.e., there is no serving cell measurement by MR), it is not required to perform measurement for MR neighbor cell on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Proposal 7: MR RSRP threshold in the entry conditions for serving cell measurement offloading (i.e., there is no serving cell measurement by MR) should be better than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 8: Before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Reference

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.