3GPP TSG-RAN WG2 Meeting #128	R2-2409745

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Impacts on the random access by the evolution of duplex operation

Document for: Discussion and Decision

# 1	Introduction

In the RAN2#127 meeting, RAN2 achieved the following working assumptions and agreements about the impacts on random access operation by the evolution of duplex operation:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. - RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. - RAN2 to strive for a common SBFD CBRA framework independent of RRC state. - FFS whether/how early indication is used during a SBFD RA procedure. - RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

And in the RAN2#127 bis meeting, the following agreements were achieved:

- RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. - From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. - Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. - The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration - For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. - The RACH configuration for SBFD is transmitted via SIB1. - FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

In this contribution, we will further analyse the RAN2 impacts to support random access in SBFD symbols by UEs in RRC CONNECTED mode and in RRC_IDLE/INACTIVE mode.

# 2	Discussion

## 2.1 RACH events

In the RAN2#127 meeting, there was one working assumption on the appliable RACH events for SBFD resource. Considering that random access in SBFD symbols aims to reduce the random access latency, to reduce the PRACH collision probability and to improve the coverage of PRACH and Msg3 transmission, we do not see strong reason to drop any of the RACH events and all of them should benefit from the advantages of RACH operation in SBFD resource.

Proposal 1: Confirm the working assumption "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events".

## 2.2 RACH Configuration

In the last RAN2 meeting, there was agreement about RACH configuration with detailed signalling FFS. It is understood that RAN2 can start working on the signalling design while RAN1 is progressing on the RACH resource configuration.

We notice that in previous RAN1 meeting, there were some agreements about the framework of SBFD RACH configuration as following:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

And in the RAN1#118bis meeting, there was conclusion about SBFD RACH resource configuration achieved:

Conclusion There is no consensus in RAN1 to support the following in Rel-19: - Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

Based on the above RAN1 conclusion, it is understood that enabling both options at the same time from network side is not to be supported. Considering that, though UE is not required to support both options, it is possible that one UE could support both option 1 and option 2 based on the related UE capability. The UE behaviour in different scenarios can be summarized as following:

Table 1. UE behaviour in different scenarios regarding RACH resource configuration options

|  | gNB uses Option 1 only | gNB uses Option 2 only |
| --- | --- | --- |
| UE supports Option 1 only | Follow Option 1 config | No SBFD RACH config |
| UE supports Option 2 only | No SBFD RACH config | Follow Option 2 config |
| UE supports both Option1, 2 | Follow Option 1 config | Follow Option 2 config |

We understand for SBFD-aware UEs supporting Option 1 configuration only, if the UE receives RACH configuration with Option 2, it will ignore the SBFD RACH resource configured by Option 2; For SBFD-aware UEs supporting Option 2 configuration only, if the UE receives RACH configuration with Option 1, it will ignore the SBFD RACH resource configured by Option 1. For the UE supporting both configurations, it will follow the SBFD RACH resource configuration used by gNB.

Proposal 2: As RAN1 agrees network not to configure both Option 1 and Option 2 in a cell, RAN2 confirms the understanding that the above Table 1 is the UE behaviour depending on UE capability regarding supporting Option 1 and Option 2.

## 2.3 RACH resource selection

In the last meeting, RAN2 discussed the RACH resource selection issue, and achieved the following agreements:

- Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

For the first agreement, whether there is need for an indication from network is FFS. we think this indicator is unnecessary due to the duplicated control by network.

First there is no much point that the network configures SBFD RACH resource and also indicates to not use the SBFD RACH resource, i.e. OFF indicator is not necessary.

Then for network sends ON indicator and asks all UE to use SBFD RACH resource. In this case, we think it is better to let UE to determine whether it is suitable to use SBFD RACH resource based on the UE's knowledge on its surrounding environment, considering that not all the UEs in the cell will benefit from using the SBFD RACH resource. Again, there is no much point to "force" a UE to use SBFD RACH resource even when this UE is experiencing e.g. high CLI.

Besides, as network can always configure the condition/prioritization to the UE meanwhile the network is aware of UE behaviour regarding RACH resource type selection based on condition/prioritization, there is no need to have one separate/additional indication due to the extra signalling overhead/complexity. For example, the network can configure a specific condition (e.g. very high/very low threshold value) such that the UE always select a specific RACH resource type.

Proposal 3: No need to have one separate/additional indication from network on UE RACH resource type selection.

Regarding on the details of RACH selection, conditions/prioritizations should be further clarified. We prefer configured condition over simple prioritization, in order to maximize resource efficiency and minimize RACH failure. It is beneficial that the SBFD-aware UEs can select the PRACH resource based on configured condition. For example, as illustrated in Figure 2, a coverage limited UE (typically at cell-edge) can select the legacy PRACH in non-SBFD symbols so that its PRACH detection performance can be guaranteed. Additionally, since coverage-limited UEs typically transmit PRACH at high power levels, restricting these UEs from selecting PRACH resources in SBFD symbols can help prevent excessive UE-to-UE CLI. A non-coverage-limited UE (typically at cell-center) can select the new PRACH in SBFD symbols since it has more spare power to combat gNB-to-gNB CLI.

Regarding using which parameter for the condition based RACH resource type selection, we prefer to use SSB RSRP against a configured threshold, as SSB RSRP is already commonly used in legacy RACH procedure for the selection between 2-step RA type and 4-step RA type, for the selection between the NUL carrier and the SUL carrier, as well as for the selection for Msg1 repetition number. It is straightforward to use SSB RSRP without further specification work and involved complexity.

Proposal 4: Support the SBFD-aware UE to select PRACH resource in non-SBFD symbols when the SSB RSRP smaller than a configured threshold, otherwise to select PRACH resource in SBFD symbols.

Figure 2. PRACH resource selection for SBFD-aware UEs

If the network doesn’t configure the threshold, we think the SBFD UE can have one default behavior as to prioritize to select the certain type of RACH resource, e.g. the SBFD RACH resource. We do not support only use priority-based RACH resource type selection, because as discussed above, it is not always beneficial for SBFD-aware UE to select SBFD resource especially when the UE is at the cell border and when the CLI interference is high.

Proposal 5: If the network doesn't configure the condition, the SBFD-aware UE prioritizes to select the SBFD RACH resource.

## 2.4 Msg3 early indication

In the last RAN2 meeting, Msg1 resource-based early identification was agreed, whether Msg3 based EI is needed is to be further discussed. We think it is still useful for network to know the UE capability earlier, when Msg1 based EI is not used, so that network can schedule subsequent transmissions/receptions in SBFD symbols. For Msg3 early indication solution, the reserved LCID can be allocated to the SBFD aware UEs to indicate the supporting of SBFD, which will have no impact to legacy UE. As there are plenty of reserved LCID, the concern on the limited LCID shall not be critical.

Proposal 6: Msg3 based early identification solution is supported, e.g., using reserved LCID to indicate supporting SBFD operation.

## 2.5 RACH-RNTI calculation

In the last RAN2 meeting, some contributions mentioned the RA-RNTI calculation issue caused by SBFD RACH resource configuration.

In the RAN1#116bis meeting, the configuration related to legacy-ROs and additional-ROs was discussed with the following agreements:

Agreement For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives: - Alt 2-3: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols - Alt 2-4: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs. For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs.

In the RAN1#118 meeting, there was further agreement about the RACH configuration:

Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

According to these agreements, we understand there shall be no need to enhance the calculation of RACH-RNTI. For RACH configuration Option 1, the legacy ROs and additional ROs share the same space of RO index, hence no RA-RNTI collision issue. For RACH configuration Option 2 with Alt2-3, the additional ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs, i.e., the "valid" SBFD ROs and legacy RO cannot overlap at the same time point. Thus there would be no RA-RNTI collision issue as well. Anyway, we can wait the conclusion of RAN1 regarding RA-RNTI conflict and the need to revise RA-RNTI calculation formula.

Proposal 7: RAN2 to wait for RAN1 conclusion regarding RA-RNTI conflict and the need to revise RA-RNTI calculation formula.

# 3	Conclusion

Based on the above discussion we have the following observation and proposals:

RACH events

Proposal 1: Confirm the working assumption "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events".

RACH Configuration

Proposal 2: As RAN1 agrees network not to configure both Option 1 and Option 2 in a cell, RAN2 confirms the understanding that the above Table 1 is the UE behaviour depending on UE capability regarding supporting Option 1 and Option 2.

RACH resource selection

Proposal 3: No need to have one separate/additional indication from network on UE RACH resource type selection.

Proposal 4: Support the SBFD-aware UE to select PRACH resource in non-SBFD symbols when the SSB RSRP smaller than a configured threshold, otherwise to select PRACH resource in SBFD symbols.

Proposal 5: If the network doesn't configure the condition, the SBFD-aware UE prioritizes to select the SBFD RACH resource.

Msg3 early indication

Proposal 6: Msg3 based early identification solution is supported, e.g., using reserved LCID to indicate supporting SBFD operation.

RACH-RNTI calculation

Proposal 6: RAN2 to wait for RAN1 conclusion regarding RA-RNTI conflict and the need to revise RA-RNTI calculation formula.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)