3GPP TSG-RAN WG2 Meeting #128	R2-2409749

# Orlando, Florida, USA, November 18-22 2024

Source: 			Qualcomm Incorporated

Title: 	Network signalling of maximum number of UL segments - RRC processing delay

Document for: 	Decision

Agenda Item: 	6.1.3.2

# Introduction

RAN2#127bis agreed to introduce an enhancement for UL segmentation for UE capability enquiry procedure as follows.

Agreements 1) Agree on the signalling solution for or RRC message segmentation of UECapabilityInformation where: - The UE indicates its support for network-requested maximum number of UL segments. - The network indicates the maximum number of UL segments the UE is allowed to use. - The UE generates UECapabilityInformation ensuring the total size of the message does not exceed the maximum allowed size according to the maximum number of UL segments the UE is allowed to use. 2) The proposed new signalling solution is introduced from release-17. 3) The RRC processing delay requirement for UE capability enquiry / transfer procedure when the network indicates the maximum allowed number of UL segments is TBD ms. 4) We will send an LS to RAN3 saying that we plan to specify this feature and that we have a need for CN to indicate the support to the RAN.

In this document, we discuss about the RRC processing delay requirement.

# Discussion / Proposal

We make the following observations on this issue.

Observation 1:	UE capability enquiry procedure is a rare event and latency performance of the procedure is not a key factor.

Observation 2:	Delay requirement still needs to be specified to allow the network to have a proper error handling (e.g. how long it should wait until it declares procedure failure).

Observation 3:	The current UE implementation may be optimized for one segment (legacy behaivour starting from Rel-15).

Observation 4:	For the current UL segmentation (Rel-17 onwards), the UE does not have to employ sophisticated strategy in reducing the UE capability size simply because the allowed UE capability size is large enough.

Observation 5:	The new enhancement solution will require the UE to employ different strategies in reducing UE capability size (e.g. skip certain features, bands or fallback band combinations) on the spot depending on the allowed number of UL segments.

Observation 6:	The new enhancement solution will need to be implemented and deployed rather quickly to address the field issue and there are many uncertainties in how good UE’s processing delay can be.

Based on the observations, we propose the RRC processing delay requirement to be 640ms.

Proposal:	Specify 640ms as the RRC processing delay requirement for the case UECapabilityEnquiry message includes the field rrc-MaxCapaSegAllowed.