3GPP TSG-RAN WG2 Meeting #128		R2-2409761

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_IDLE/INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

- Introduction

In previous meetings, some agreements for LP-WUS/WUR in RRC_IDLE/INACTIVE were made as below.

The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

In this contribution, we will further discuss the issues for RRC_IDLE/ INACTIVE procedures with LP-WUR, including the LP-WUS configuration, the entry and exit condition, other wake up procedure, subgrouping, UE capability, and RAN1 LS.

- Discussion

  - LP-WUS configuration

In RAN2#126 meeting, it was agreed that the entry/exit condition for LP-WUS monitoring is included in LP-WUS configuration. However, it is still FFS whether the entry/exit condition for LP-WUS monitoring is always configured in SIB. Besides, RAN1 concluded that it is up to RAN2 discussion [1].

It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

Some companies thought if the MR and LR have the same coverage, the entry/exit condition for LP-WUS monitoring seems not always need to be configured in RAN2#125bis meeting.

According to the WID [2], the target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for Msg3. However, it depends on the real deployment on whether the coverage of LP-WUS and LP-SS is full coverage or partial coverage. For example, if the size of a cell is small, the coverage of LP-WUS and LP-SS can be full coverage.  On the contrary, if the size of a cell is large, the coverage of LP-WUS and LP-SS can be partial coverage. In our understanding, the entry/exit condition should be mandatorily configured to make UE correctly check whether the entry/exit condition for LP-WUS monitoring is met or not.

Some companies may argue it should be optional to leave the flexibility to network configuration. But our understanding is network could either provide a very small value in case the network always allows UE to perform LP-WUS monitoring or provide the same value as the threshold of condition for RRM relaxation/offloading, which provides enough flexibility for network configuration.

Proposal 1: The entry/exit condition for LP-WUS monitoring is mandatorily configured if the network is intended to support the LP-WUS feature.

In RAN2#125bis and RAN2#126 meeting, it was agreed that the LP-WUS related configuration includes LP-SS configuration, LP-WUS configuration, and the entry/exit condition for LP-WUS monitoring. Besides, In RAN2#127 meeting, RAN2 agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. If the network supports the OOK-based WUR, the LP-SS configuration should be configured in the LP-WUS related configuration. Otherwise, after UE with OOK-based WUR enters LP-WUS monitoring, OOK-based WUR cannot receive LP-WUS without the synchronization from LP-SS.

Proposal 2: If the network supports the OOK-based WUR, the LP-SS configuration should be always configured.

  - Entry/exit condition of using LP-WUS for OFDM-based LR

In RAN2#127 meeting, RAN2 agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Besides, RAN1 agreed that overlaid OFDM sequence(s) is supported for LP-SS as below in RAN1#118bis meeting.

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences.

Then, the entry/exit thresholds of using LP-WUS for OFDM-based WUR monitoring the overlaid OFDM sequence(s) needs to be discussed and decided, i.e., the entry/exit thresholds could be the same as either the entry/exit thresholds for OFDM-based WUR or entry/exit thresholds for OOK-based WUR.

In our understanding, RAN1 agreed that the design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. Thus, the entry/exit thresholds and requirements for OFDM-based WUR monitoring the overlaid OFD sequence(s) should be the same as the entry/exit thresholds and requirements for OOK-based WUR. Considering the corresponding requirements should be decided in RAN4, while RAN2 needs to determine the entry/exit threshold of using LP-WUS.

Proposal 3: The entry/exit thresholds of using LP-WUS for OFDM-based WUR monitoring the overlaid OFDM sequence(s) are the same as that for OOK-based WUR.

  - Other wake up conditions

As legacy PEI design, the MR in ultra-deep sleep/monitoring LP-WUS should also wake up in case there is UL data/signalling arrival.

Proposal 4: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT/RNA update/TA update, due to UL data and signaling arrival when monitoring LP-WUS.

  - LP-WUS Subgrouping

Regarding the subgrouping method for LP-WUS, RAN2 agreed that the PEI subgrouping method is taken as baseline, i.e. CN assigned and UE_ID based subgrouping. In the following part, we will analyse the specific detail for subgrouping methods.

The first issue is whether a network indication should be introduced to indicate the UE to monitor LP-WUS and PEI or not.

At the network side, PEI could be configured in the case that the coverage of LP-SS/LP-WUS is the partial coverage. According to legacy design, if there is PEI configuration, all UE supporting PEI could monitor PEI, which means that network needs to always provide PEI if it indicates supporting PEI. Similarly, if network supports LP-WUS and provides the LP-WUS configuration, all UE supporting LP-WUS could monitor LP-WUS, which means that network needs to always provide LP-WUS if it indicates support LP-WUS.

At the UE side, there is a case that PEI and LP-WUS are applicable at one UE when the entry condition of  LP-WUS monitoring is fulfilled. In RAN1#116 meeting, RAN1 made the following agreement:

Agreement For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

It could be observed that, when UE supports PEI and PEI is configured by the gNB, it is up to UE implementation whether to monitor PEI or not after the UE receives LP-WUS indicating wake-up.

Observation 1: According to the RAN1 agreement, when UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Based on the above design at network side, regardless of whether the UE monitors PEI or not after receiving LP-WUS indicating wake-up, the network would provide PEI and LP-WUS in case it provides the PEI and LP-WUS configuration. Thus, there is no additional effort for network to provide both PEI and LP-WUS. In this way, there is no need for network to provide an indication to indicate the UE to monitor both PEI and LP-WUS or not.

Proposal 5: No need to introduce a network indication to indicate the UE to monitor both LP-WUS and PEI or not.

With the above analysis, the UE may monitor PEI and LP-WUS together. But the network does not know whether the UE monitors LP-WUS and PEI or one of them. Thus, in case LP-WUS and PEI are used together for one UE, it is expected that UE could achieve more power saving gain via two levels of subgrouping. According to the following Figure 1 and Figure 2, assuming there are 1024 UEs supporting both LP-WUS and PEI subgrouping sharing the same PO and the subgrouping number of LP-WUS and PEI are both 4, in case UE_ID#0 is paged:

- In Figure 1, LP-WUS subgrouping and PEI subgrouping are totally same, 256 UEs will be woken up by LP-WUS to monitor PEI, and all of them will be indicated by PEI to monitor the following PO.

- In Figure 2, LP-WUS subgrouping and PEI subgrouping are orthogonal, 256 UEs will be woken up by LP-WUS to monitor PEI, while only 64 UEs will be indicated by PEI to monitor the corresponding PO.

Figure 1: LP-WUS and PEI subgrouping are same

Figure 2: LP-WUS and PEI subgrouping are diverse/ orthogonal

It could be observed that if the subgroup ID of LP-WUS and PEI are the same, there is no benefit for PEI monitoring, instead it leads to more power consumption as UE has to monitor PEI after LP-WUS. However, if the subgroup ID of LP-WUS and PEI are independent, the false alarm rate could be further reduced with PEI.

Observation 2: False alarm rate could be further reduced if the subgrouping ID of LP-WUS and PEI are independent.

Proposal 6: In order to allow UE to use LP-WUS together with PEI to reduce paging false alarm rate, it is expected independent subgroup ID is applied for a UE.

To achieve additional power saving gain as analyzed above and considering both LP-WUS and PEI support UE_ID based subgrouping and CN assigned subgrouping, the following options could be considered to achieve more power saving gain:

- For CN assigned subgrouping, it is expected that the CN could assign different subgrouping ID for LP-WUS and PEI for the same UE as much as possible. However, it is up to the SA2/CT1 design and up to network implementation to assign same or different subgrouping ID for the same UE.

- For UE_ID based subgrouping, a different formula could be used as UE_ID for LP-WUS and PEI subgrouping.

- For case CN assigned subgrouping is used for one feature (i.e. LP-WUS or PEI), while UE_ID based subgrouping is used for another feature, considering different subgrouping methods are used, it is expected that independent subgrouping ID is assigned/calculated for LP-WUS and PEI, naturally. It could be achieved by the restriction that the number of CN assigned subgrouping is the same within the registration area, which is up to network implementation.

For the UE_ID based subgrouping, RAN2 agreed that the similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. Besides, the formula for PEI is as follows.

SubgroupID = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID), where: - N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 - Ns: number of paging occasions for a PF - UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 - subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

With this same logic, considering the UE may monitor LP-WUS and PEI together, the subgrouping for LP-WUS should be considered on top of the subgrouping for PEI. Thus, a parameter of Np (i.e. the number of subgroupsNumForUEID for PEI) is introduced in the formula as below.

Proposal 7: For UE_ID based subgrouping, the formula is defined as below for LP-WUS subgrouping,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

- UE_ID is related to 5G-S-TMSI,

- N is the number of total paging frames in one DRX cycle,

- Ns is the number of the PO for a PF,

- Np is the number of subgroupsNumForUEID for PEI, if configured; otherwise, Np is 1,

- subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Regarding UE_ID, similar as PEI design, it could be derived from 5G-S-TMSI. More specific, if eDRX is not configured, UE_ID=5G-S-TMSI mode (8192* max(subgroupsNumForUEID)).

Proposal 8: For LP-WUS UE_ID based subgrouping, UE_ID = 5G-S-TMSI mode (8192* max(subgroupsNumForUEID)).

Considering UE's LP-WUS subgroup can be either assigned by CN or formed based on UE_ID, the UE needs to know which kind of subgrouping method is used for LP-WUS when camping on a cell. In Rel-17 PEI, a cell can indicate its capability of CN assigned subgrouping and UE_ID based subgrouping to UE by the configuration. Specifically, the detail is as follows.

- If  PEI is configured and both subgroupsNumPerPO and subgroupsNumForUEID are equal to 1, the cell supports PEI without subgrouping, i.e., there is only one subgroup and all UEs supporting PEI belong to the same subgroup.

- If PEI is configured and subgroupsNumForUEID has the same value (except 1) as subgroupsNumPerPO, the cell only supports the UE_ID based subgrouping.

- If PEI is configured and subgroupsNumForUEID is absent, the cell only supports the CN assigned subgrouping.

- If PEI is configured and subgroupsNumForUEID < subgroupsNumPerPO, the cell supports both CN assigned subgrouping and UE_ID based subgrouping.

Correspondingly, according to TS 38.304 [3], the UE behaviors for the PEI subgroup are captured as follows.

- If UE only supports UE_ID based/CN assigned subgrouping and the cell supports both CN assigned subgrouping and UE_ID based subgrouping, UE uses the UE_ID based/CN assigned subgrouping.

- If the cell only supports UE_ID based/CN assigned subgrouping and the UE supports both CN assigned subgrouping and UE_ID based subgrouping, UE uses the UE_ID based/CN assigned subgrouping.

- If both UE and cell support CN assigned subgrouping and UE_ID based subgrouping, UE uses the CN assigned subgrouping.

- If both UE and cell only support UE_ID based/CN assigned subgrouping, UE uses the UE_ID based/CN assigned subgrouping.

- If both UE and cell only support CN assigned subgrouping and UE has no CN assigned subgroup ID, UE monitors the legacy PO.

- If the cell supports PEI without subgrouping, UE uses the UE_ID based subgrouping.

- If UE only supports one kind of subgrouping and the cell only supports another kind of subgrouping, UE monitors the legacy PO.

For LP-WUS subgrouping, we think it is straightforward to reuse the PEI mechanism. That is, the network provides the capability on supporting LP-WUS subgrouping method(s) based on the configuration. For UE supporting both UE_ID based and CN assisgned subgrouping, it is up to network configuration to use UE_ID based or CN assisgned subgrouping, similar to the PEI mechanism.

Proposal 9: The network provides the capability on supporting LP-WUS subgrouping method(s) based on the configuration, similar as PEI design.

Proposal 10: For UE supporting both UE_ID based and CN assisgned subgrouping, it is up to network configuration to use UE_ID based or CN assisgned subgrouping, similar as PEI design.

  - UE capability

For CN/RAN paging, the network needs to know the UE capability for paging to decide which kind of paging mechanism is used, e.g. PEI/LP-WUS or legacy paging. In Rel-17 PEI, separate  UE capabilities of CN assigned  subgrouping and UE_ID based subgrouping is introduced. Besides, the UE capability to indicate supporting PEI and UE_ID based subgrouping are same, i.e., if UE supporting PEI it will also support the UE_ID based subgrouping, and this combined UE capability is reported to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message. While UE’s capability of supporting the CN assigned subgrouping is reported to CN by NAS signalling during the registration request or TAU procedure.

Observation 3: For UE capability design in Rel-17 PEI, the UE capability supporting PEI is also used to indicate the capability supporting UE_ID based subgrouping.

Observation 4: For UE capability design in Rel-17 PEI, the UE capabilities for UE_ID based subgrouping and CN assigned subgrouping are separate.

Considering LP-WUS is also used for CN/RAN paging, UE capability for LP-WUS should also be reported to the network like PEI. Different from PEI, besides two kinds of subgrouping methods, there are two kinds of LR receivers for LP-WUS, i.e. OOK-based WUR and OFDM-based WUR. Since RAN2 has agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs,  correspondingly, it is reasonable to introduce separate UE capabilities for OFDM-based WUR and OOK-based WUR. But the final decision should be determined in RAN1.

Proposal 11: RAN2 assumes separate UE capabilities for different LR types, i.e. OFDM-based WUR and OOK-based WUR, should be introduced and reported to the network. Final decision is up to RAN1.

Regarding the UE capability for CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, it is straightforward to reuse the capability design for PEI. That is, the UE capabilities on supporting CN assigned subgrouping and UE_ID based subgrouping are separately indicated. And no need to introduce additional UE capability for UE_ID based subgrouping, while using the UE capability supporting the LP-WUS to indicate whether supporting the UE_ID based subgrouping. Though there are separate UE capabilities for different types of LR, i.e., the UE capability supporting OFDM-based WUR and OOK-based WUR, we think no matter UE indicates which LR type among these two capabilities, it will also support the UE_ID based subgrouping.

Proposal 12: Introduce separate UE capabilities for CN assigned subgrouping and UE_ID based subgrouping, similar as PEI design.

Proposal 13: UE capability for supporting LP-WUS (assuming capability for OFDM-based WUR or OOK-based WUR), indicates supporting of UE_ID based subgrouping, i.e. capability supporting LP-WUS and UE_ID based subgrouping is one capability, similar as PEI design.

Regarding the capability reporting for LP-WUS, we think the similar reporting mechanism for PEI could be reused directly. In detail, UE’s capability of supporting the CN assigned  subgrouping is reported to CN by NAS signalling during the registration request or TAU procedure. And for the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping is reported to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message, and gNB will report this UE capability to AMF transparently for storage.

Proposal 14: The capability reporting deisgn for PEI is reused for LP-WUS, i.e. UE reports capability of supporting the CN assigned subgrouping to CN by NAS signalling during the registration request or TAU procedure; UE reports the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

After the AMF acquired the UE capabilities for LP-WUS, for RRC_IDLE UEs, the CN will include these UE capabilities in NG paging message to inform the gNB. For RRC_INACTIVE UEs, the CN will inform the gNB about these UE capabilities via NGAP core Networking assistance information IE, then the last serving gNB could store this UE capabilities. After the gNB acquire the UE capabilities for LP-WUS, it could determine whether to send LP-WUS with CN assigned subgrouping ID or UE_ID based subgrouping ID and which kind of WUR is used based on the included UE capability information.

Proposal 15: In the CN paging message, AMF carries UE’s OFDM/OOK-based WUR capability.

  - RAN2 impacts on RAN1 LS

In RAN1#118 meeting, RAN1 sent an LS to RAN2 which is shown as follows [4]:

# 1 Overall description In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. # 2 Actions To RAN2 and RAN4: ACTION: RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

According to the LS, it could be observed that the scenario that the UE’s MR is on one band, e.g. band#1, which couldn’t support the LP-WUS. In this case, some one may argure that the LP-WUS feature couldn’t be used when UE camps on this band. However, in our understanding, the intention of the LS sent by RAN1 is to trigger RAN2/RAN4 to identify the coresonding impacts if LP-WUS is to be supported in this case.

Observation 5: The intention for RAN1 LS is to check RAN2/RAN4 impacts on supporting LP-WUS in case UE may not support LP-WUS on all the bands supported by the MR.

In our view, the following alternatives could be considered to support LP-WUS in this case:

- Alt1: UE camps on the band supporting LP-WUS with higher priority.

- Alt2: Supporting the scenario that UE camps on one band, LP-WUS monitoring on another band.

To achieve Alt1, the frequency priority should be set based on whether supporting LP-WUS in cell reselection. That is to say, the frequency priority of the band supporting LP-WUS should be higher for UE supporting LP-WUS. It could be observed that the Alt1 will impact the legacy UE cell reselection procedure. Besides, since all UEs supporting LP-WUS will camp on the band(s) supporting LP-WUS and the number of UEs supporting LP-WUS may be large, the cell supporitng LP-WUS will have high load in this case, which will impact the network performance.

Observation 6: Alt1, i.e. UE camps on the band supporting LP-WUS with higher priority, has the following RAN2 impacts:

- Cell reselection: The frequency supporting LP-WUS should have higher priority for UE supporting LP-WUS.

- Network load: The frequency supporting LP-WUS will have high load for UEs supporting LP-WUS.

To achieve Alt2, the LR measurement should always be considered for the entry condition for LP-WUS monitoring, since MR measurement couldn’t refelect the LR measurement well in case MR and LR are on different bands. However, since RAN2 has agreed the LR threshold could be configured for the entry condition for LP-WUS monitoring, this impact is small. Besides, the network needs to broadcast configuration(s) for LP-WUS monitoring on the separate band.

Observation 7: Alt2, i.e. supporting the scenario that UE camps on one band, LP-WUS monitoring on another band, has the following RAN2 impacts:

- LR threshold is mandatorily configured for entry condition for LP-WUS monitoring.

- Network needs to broadcast configuration(s) for LP-WUS monitoring on separate band.

With the above, we think Alt2 should be considered as it has less RAN2 impacts from RAN2 point of view, considering Alt1 will impact the cell reselection principle largely due to one feature and impact the network performance. Anyway, this could be further discussed after RAN1 makes the decision on whether/how to support this case. Regarding the reply LS, we should provide the corresponding RAN2 imapcts.

Proposal 16: Reply LS to RAN1 to include the solutions to supporting LP-WUS in case UE may not support LP-WUS on all the bands supported by the MR and the corresponding impacts.

A draft reply LS is provided in Annex.

Proposal 17: RAN2 to consider the draft reply LS in Annex.

- Conclusion

In this contribution, we discuss the LP-WUS configuration, the entry and exit condition, other wake up procedure, subgrouping, UE capability, and RAN1 LS. Based on the discussion, we have the following observations and proposals.

Observations：

Observation 1: According to the RAN1 agreement, when UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Observation 2: False alarm rate could be further reduced if the subgrouping ID of LP-WUS and PEI are independent.

Observation 3: For UE capability design in Rel-17 PEI, the UE capability supporting PEI is also used to indicate the capability supporting UE_ID based subgrouping.

Observation 4: For UE capability design in Rel-17 PEI, the UE capabilities for UE_ID based subgrouping and CN assigned subgrouping are separate.

Observation 5: The intention for RAN1 LS is to check RAN2/RAN4 impacts on supporting LP-WUS in case UE may not support LP-WUS on all the bands supported by the MR.

Observation 6: Alt1, i.e. UE camps on the band supporting LP-WUS with higher priority, has the following RAN2 impacts:

- Cell reselection: The frequency supporting LP-WUS should have higher priority for UE supporting LP-WUS.

- Network load: The frequency supporting LP-WUS will have high load for UEs supporting LP-WUS.

Observation 7: Alt2, i.e. supporting the scenario that UE camps on one band, LP-WUS monitoring on another band, has the following RAN2 impacts:

- LR threshold is mandatorily configured for entry condition for LP-WUS monitoring.

- Network needs to broadcast configuration(s) for LP-WUS monitoring on separate band.

Proposals：

LP-WUS configuration:

Proposal 1: The entry/exit condition for LP-WUS monitoring is mandatorily configured if the network is intended to support the LP-WUS feature.

Proposal 2: If the network supports the OOK-based WUR, the LP-SS configuration should be always configured.

Entry/exit condition of using LP-WUS for OFDM-based LR

Proposal 3: The entry/exit thresholds of using LP-WUS for OFDM-based WUR monitoring the overlaid OFDM sequence(s) are the same as that for OOK-based WUR.

Other wake up procedure:

Proposal 4: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT/RNA update/TA update, due to UL data and signaling arrival when monitoring LP-WUS.

LP-WUS Subgrouping:

Proposal 5: No need to introduce a network indication to indicate the UE to monitor both LP-WUS and PEI or not.

Proposal 6: In order to allow UE to use LP-WUS together with PEI to reduce paging false alarm rate, it is expected independent subgroup ID is applied for a UE.

Proposal 7: For UE_ID based subgrouping, the formula is defined as below for LP-WUS subgrouping,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

- UE_ID is related to 5G-S-TMSI,

- N is the number of total paging frames in one DRX cycle,

- Ns is the number of the PO for a PF,

- Np is the number of subgroupsNumForUEID for PEI, if configured; otherwise, Np is 1,

- subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Proposal 8: For LP-WUS UE_ID based subgrouping, UE_ID = 5G-S-TMSI mode (8192* max(subgroupsNumForUEID)).

Proposal 9: The network provides the capability on supporting LP-WUS subgrouping method(s) based on the configuration, similar as PEI design.

Proposal 10: For UE supporting both UE_ID based and CN assisgned subgrouping, it is up to network configuration to use UE_ID based or CN assisgned subgrouping, similar as PEI design.

UE capability:

Proposal 11: RAN2 assumes separate UE capabilities for different LR types, i.e. OFDM-based WUR and OOK-based WUR, should be introduced and reported to the network. Final decision is up to RAN1.

Proposal 12: Introduce separate UE capabilities for CN assigned subgrouping and UE_ID based subgrouping, similar as PEI design.

Proposal 13: UE capability for supporting LP-WUS (assuming capability for OFDM-based WUR or OOK-based WUR), indicates supporting of UE_ID based subgrouping, i.e. capability supporting LP-WUS and UE_ID based subgrouping is one capability, similar as PEI design.

Proposal 14: The capability reporting deisgn for PEI is reused for LP-WUS, i.e. UE reports capability of supporting the CN assigned subgrouping to CN by NAS signalling during the registration request or TAU procedure; UE reports the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

Proposal 15: In the CN paging message, AMF carries UE’s OFDM/OOK-based WUR capability.

RAN2 impact on RAN1 LS:

Proposal 16: Reply LS to RAN1 to include the solutions to supporting LP-WUS in case UE may not support LP-WUS on all the bands supported by the MR and the corresponding impacts.

Proposal 17: RAN2 to consider the draft reply LS in Annex.

- References

- Draft_Minutes_report_RAN1#116b_v030

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO

- TS38.304 v18.3.0

- R2-2407921/R1-2407559, LS on LP-WUS operation in IDLE/INACTIVE mode

Annex – [draft] reply LS to RAN1 on LP-WUS operation

Title:	[Draft] reply LS on LP-WUS operation in IDLE/INACTIVE mode

Response to:	R2-2407921/R1-2407559

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	TBD [to be RAN2]

To:	RAN1

Cc: 	RAN4

Contact Person:

Name:	              TBD

E-mail Address:	 TBD

1. Overall Description:

RAN2 would like to thank RAN1 for the LS on LP-WUS operation in IDLE/INACTIVE mode in [R2-2407921/R1-2407559].

RAN2 has discussed this issue, and has identified the following solutions on how to support it and the corresponding RAN2 impacts [To be updated based on the discussion conclusion]:

Alt1: UE camps on the band supporting LP-WUS with higher priority, the corresponding RAN2 impacts include:

- Cell reselection: The frequency supporting LP-WUS should have higher priority for UE supporting LP-WUS.

- Network load: The frequency supporting LP-WUS will have high load for UEs supporting LP-WUS.

Alt2: Supporting the scenario that UE camps on one band, LP-WUS monitoring on another band, the corresponding RAN2 impacts include:

- LR threshold is mandatory configured for entry condition for LP-WUS monitoring.

- Network needs to broadcast configuration(s) for LP-WUS monitoring on separate band.

2. Actions:

To RAN WG 1:

RAN2 kindly request RAN1 to take the above information into account and provide feedback, if any.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #129		17th Feb. – 21st Feb. 2025		Athens, Greece

TSG RAN WG2 Meeting #129bis		7th Apr. – 11th Apr. 2025		             TBD, China