3GPP RAN WG2 Meeting #128 	R2-2409800

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.9.3

Source:	InterDigital, Inc.

Title:	EDT/PUR enhancements.

Document for:	Discussion

# Introduction

A revised WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]: - Msg3 transmission without msg1/RAR - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#126, the following was agreed:

Agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements

In RAN2#127, the following was agreed:

Agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

In RAN2#127bis, the following was agreed:

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

.

# Overview of existing functionality

## 2.1	MO-EDT

It was agreed that both CP and UP solutions, NB-IoT and eMTC, are in the scope of the study, but only EPS (and not 5GC).

The following show MO-EDT procedures for the CP and UP EPS optimizations.

The MO-EDT procedure for Control Plane CIoT EPS optimisation is illustrated in Figure 7.3b- of 36.300 [2]

Figure 7.3b-1: MO-EDT for Control Plane CIoT EPS Optimisation

The MO-EDT procedure for User Plane CIoT EPS optimisation is illustrated in Figure 7.3b-2 of 36.300 [2]

Figure 7.3b-2: MO-EDT for User Plane CIoT EPS Optimisation

## 2.2	Transmission using PUR

The procedure for transmission using PUR for the Control Plane CIoT EPS optimisation and for the Control Plane CIoT 5GS optimisation is illustrated in Figure 7.3d-2 of 36.300 [2].

Figure 7.3d-2: Transmission using PUR for the Control Plane CIoT EPS/5GS Optimisations

The procedure for PUR configuration request and PUR configuration is common to the Control Plane CIoT EPS/5GS optimisations and the User Plane CIoT EPS/5GS optimisations and is illustrated in Figure 7.3d-1 of 36.300 [2].

Figure 7.3d-1: PUR Configuration Request and PUR Configuration

# Msg3 transmission without msg1/RAR

In the previous meeting the following key agreements were made for contention based EDT transmission:

- PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

- PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission (to be confirmed by RAN1)

FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

While the purpose of repetitions is to improve coverage, the purpose of DSA is to improve contention failure. It needs to be carefully considered how these 2 features will work together because using a large number of repetitions to ensure coverage is already demanding on the resource usage, and adding duplication on top may result in an inefficient use of resources.

In Release 13, the maximum TBS for NPUSCH transmission is 1000 bits and the maximum delay k0 that can be set is 64 ms. The maximum number of RUs (NRU ) and repetitions (NRep) are 10 and 128. Therefore, depending on the duration of the RUs which can be up to 32ms depending on subcarrier spacing and number, the maximum duration for a NPUSCH transmission is 10 × 128 × 32 ms = 40.96s. Transmitting just one duplicate would use 256 repetitions in total increasing that transmission to 81.92s.

For legacy preamble/PRACH using repetitions, it is possible for the network to configure number of repetitions per CE level. If the transmission fails using the selected CE level (i.e. maximum number of repetitions are transmitted without response) then the UE retries using the next CE level (so increasing the number of repetitions).

If CB-EDT is configured in the network, it would make sense that the configuration is per CE level. This is already implied by the current agreement, however what is not clear (it is FFS) is the configuration of DSA and how to use 4 step RACH.

These issues can simply be solved by allowing DSA and CB-EDT configuration per CE level. In order words, if CB-EDT is used it can be enabled per CE level therefore removing the need for separate thresholds for CB-EDT/4 step RACH (the existing CE level thresholds can apply), and similarly DSA can be enabled and configured per CE level (at least the number of replicas as well as the resource selection criteria). This allows the network flexibility to configure the resources accordingly, in order to address the resource usage trade-off between repetition numbers per CE level with the number of replicas, depending on whether coverage or contention needs to be improved.

As an example, consider the following configuration:

CE level 0: CB-EDT enabled, DSA enabled – 2 PUSCH repetitions, and 4 DSA replicas ( = 8 PUSCH TBs in total)

CE level 1: CB-EDT enabled, DSA enabled – 4 PUSCH repetitions, and 2 DSA replicas ( = 8 PUSCH TBs in total)

CE level 2: CB-EDT disabled – UE uses 4 step RACH.

Allowing the possibility to configure whether CB-EDT is used per coverage level allows to configure the lowest coverage level(s) with PRACH by default. A large number of repetitions to target deep coverage implies high resource usage, and the main issue is coverage rather than contention - contention can be resolved by using 4 step RA in this case and use of PRACH  repetitions rather than PUSCH repetitions limits resource overhead.

In relatively good coverage, UEs can be configured with a relatively low number of repetitions. And to take advantage of this, the number of replicas can be increased to improve contention.

The failure handling is covered by default – re-using the legacy (PRACH) approach, by selecting the next coverage level down automatically selects a more reliable mechanism, and there would be no need to define any new “fallback” mechanism because if the network decides to configure the lowest (or multiple) coverage levels without CB-EDT then PRACH will be used by default.

Based on this, we propose:

Proposal 1: CB-EDT configuration per CE level includes whether it is enabled (if not enabled then the existing PRACH based configuration will be used for that coverage level).

Proposal 2: DSA configuration is per coverage level (FFS exactly what the configuration contains e.g. number of replicas, and resource configuration)

Proposal 3: Re-use the existing per-CE level PRACH failure handling – if transmission fails using the selected CE level configuration, then select the next CE level configuration and retry. No need to introduce separate failure handling.

In terms of resource selection, in case of legacy PRACH selection the UE selects a preamble randomly. For the case of selecting a PUSCH resource, we need to decide how the UE picks a resource (or set of resources in case of DSA and/or repetitions). One option is simply to define random selection. In case of failure and retry, if we assume that the UE selects the next coverage level, then the UE will pick from a different set of resources, hence choosing a different resource or set of resources implies another random selection. Random selection can be done either using a random number or by a pseudo-random method such as using a HASH function of UE-ID.

Proposal 4: UE selects a PUSCH resource for transmission of CB-EDT randomly (either using random number or UE-ID) from the set of resources configured for the selected CE level. FFS the exact parameters associated with the selected resources (e.g. repetitions, replica pattern, OCC configuration, etc)

Proposal 5: In case of failure, UE selects again (using the same process) from the set of resources configured with the next selected CE level.

# Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

As can be seen, only the MO-EDT procedure for CP solution uses “RRCEarlyDataComplete”. User data is attached to Msg3 and Msg4 during the RACH procedure. The procedure is terminated by Msg4.

The MO-EDT procedure for UP solution transfers data in RRC_CONNECTED, after completing random access (5 steps) and the procedure is terminated using RRCConnectionRelease.

The WI objective clearly states that the intention is to reduce overhead of Msg4/RRCEarlyDataComplete, so it is clear that this part of the objective is only applicable to the CP solution.

Proposal 6: Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete only applies to the C-plane solution.

As can be seen, if PUR is used, it is already possible to terminate EDT without using RRCEarlyDataComplete. As described in 36.300 [2] :

7a	If the (ng-)eNB is aware that there is no pending downlink data or signalling, the (ng-)eNB can send a Layer 1 ACK optionally containing a Time Advance Adjustment to the UE to update the TA and terminate the procedure.

7b	If the (ng-)eNB is aware that there is no further data or signalling, the (ng-)eNB can send a Time Advance Command to update the TA and terminate the procedure.

Observation 1: The existing PUR feature can terminate the EDT procedure without using RRCEarlyDataComplete, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

The main question as to whether early termination of EDT works in practise is “how does eNB know that there is no pending downlink data from the application layer?“

Proposal 7: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

The UE can transmit an indication (L1Ack) in PURConfigurationRequest, which informs the eNB a preference for using RRCEarlyDataComplete. Note that, although the figure states “L1 ACK” this is named “rrc-ACK” in the RRC specification and it indicates RRC response message is preferred by the UE for acknowledging the reception of a transmission using PUR. This is an indirect way to specify that the UE can send some information about whether it expects downlink data in response to the uplink. It is then up to the eNB implementation what (if anything) to do with that knowledge. The main issue with terminating EDT early is when there is some downlink application layer data to send in response. If the procedure is terminated early then the network must page the UE in order to transmit this, which is even more inefficient than waiting and terminating using RRC.

Observation 2: With the existing PUR feature it is possible for the UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRCEarlyDataComplete.

Given that we have established above that UE can already indicate a preference for RRC response, and eNB can already terminate EDT without RRC response, the question is how this can possible be further improved or whether the existing feature is in fact sufficient.

There are several potential deficiencies with the existing EDT termination feature with PUR in the context of NTN.

- The current PUR feature only applies if the UE remains in the same cell – i.e. PURConfigurationRequest, and RRCConnectionRelease containing PURConfiguration is needed in the cell on which transmission using PUR is used. For NTN, the cell can change frequently such that the overhead in performing this is excessive.

- The UE can only send an indication “rrc-ACK” in the PUR configuration request, which would apply to all subsequent PUR occurrences configured with PURConfiguration. If we enable dedicated PUR resources which are applicable in multiple cells, or if we enable shared (contention based) PUR in a cell, then there is currently no way for the UE to indicate “rrc-ACK”. There is also currently no network signalling to transfer this information from one eNB to another.

- As a minimum, a L1 ACK needs to be sent in the downlink. There is currently no way to terminate EDT without downlink signalling, or without sending downlink signalling to all UEs always.

- As it has been agreed to use shared PUR resources, there is a possibility that there is contention, even if DSA is used. Therefore, a contention resolution procedure is needed and it may not be possible to do this without transmission of a contention resolution MAC CE. Even though it was agreed that the RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission, this is not sufficient to resolve any potential contention.

To address these deficiencies, some possible enhancement can be considered.

- Enabling PUR configuration information across cells. This may imply that the information sent by the UE (i.e. the rrc-ACK indication) needs to be transferred between eNBs using network signalling)

- Indication by the UE in RRCEarlyDataRequest an “rrc-ACK” parameter – which is an indication whether the UE expects a downlink application layer response. This would enable the UE to indicate to the cell on which the transmission using PUR is initiated, and indicate per transmission. (This could work whether dedicated or shared PUR resources are used).

- Enabling EDT termination without any downlink ACK

- Using a common ACK for multiple transmissions or UEs (E.g. using a group common signalling)

- Use of contention resolution MAC CE to terminate the procedure

Proposal 8: RAN2 to consider whether the following enhancements are beneficial:

- Introducing network signalling so that PUR configuration request information to be transferred across cells.

- Indication by the UE in RRCEarlyDataRequest an “rrc-ACK” parameter (whether UE expects a downlink application layer response)

- Enabling EDT termination without any downlink ACK

- Using a “common” ACK for multiple transmissions or UEs

- Use of contention resolution MAC CE to terminate the procedure

# Conclusion

In this contribution we discuss uplink capacity enhancements and make the following proposals for discussion:

Msg3 transmission without msg1/RAR

Proposal 1: CB-EDT configuration per CE level includes whether it is enabled (if not enabled then the existing PRACH based configuration will be used for that coverage level).

Proposal 2: DSA configuration is per coverage level (FFS exactly what the configuration contains e.g. number of replicas, and resource configuration)

Proposal 3: Re-use the existing per-CE level PRACH failure handling – if transmission fails using the selected CE level configuration, then select the next CE level configuration and retry. No need to introduce separate failure handling.

Proposal 4: UE selects a PUSCH resource for transmission of CB-EDT randomly (either using random number or UE0=-ID) from the set of resources configured for the selected CE level. FFS the exact parameters associated with the selected resources (e.g. repetitions, replica pattern, OCC configuration, etc)

Proposal 5: In case of failure, UE selects again (using the same process) from the set of resources configured with the next selected CE level.

Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

Proposal 6: Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete only applies to the C-plane solution.

Observation 1: The existing PUR feature can terminate the EDT procedure without using RRCEarlyDataComplete, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

Proposal 7: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

Observation 2: With the existing PUR feature it is possible for the UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRCEarlyDataComplete.

Proposal 8: RAN2 to consider whether the following enhancements are beneficial:

- Introducing network signalling so that PUR configuration request information to be transferred across cells.

- Indication by the UE in RRCEarlyDataRequest an “rrc-ACK” parameter (whether UE expects a downlink application layer response)

- Enabling EDT termination without any downlink ACK

- Using a “common” ACK for multiple transmissions or UEs

- Use of contention resolution MAC CE to terminate the procedure

# References

- RP-242397 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”