3GPP TSG-RAN WG2 Meeting#128                                	R2-2409822

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.9.2

Source: 	Google

Title: 	Discussion on the S&F operation indication

Document for:	Discussion and Decision

# Introduction

In Rel-19, the benefits of enabling the Store & Forward operation for IoT-NTN are identified and the following scopes are included in the WID of Rel-19 IoT-NTN [1].

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]. - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

The Rel-19 IoT-NTN discussion was kicked off in the RAN2#125bits meeting, and some progress was made in RAN2#127bis meeting, with regard to the S&F indication provided by the network [2].

RAN2 Agreements in RAN2#127bis: - The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed) - RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided. - An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed - If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure - When present, the “S&F operation” indication has two possible settings: ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell) ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In this paper, we discuss the impact of the S&F operation indication on the UE, especially on the UE with limited S&F capability. Based on the discussion, several way forwards are proposed in this paper.

# Discussion

## Access control for the UEs with limited S&F capability

It was agreed in RAN2#127bis [2] that a dynamic indication “the cell is operating in S&F mode” is broadcast in the system information for the cell operating in S&F mode (i.e., having no feeder link available). While the value of this dynamic indication determines directly whether the Rel-19 UE supporting S&F can access the cell or not, it has no impact to the Rel-19 UE not supporting S&F as well as to the pre-R19 UEs. The Rel-19 UEs not supporting S&F and pre-R19 UEs still rely on the legacy NTN barring bit (i.e., cellBarredNTN) to determine whether to access a cell or not. Therefore, it is possible for the UEs not supporting S&F (including R19 and pre-R19 UEs) to camp on a cell operating in the S&F mode (i.e., broadcasting the S&F operation indication), if the legacy NTN barring bit indicates “notBarred” in that cell.

The UE not supporting S&F may be allowed to camp on the cell providing the S&F operation indication, depending on the legacy barring bit.

In certain network deployment option (e.g., full-EPC onboard), the network may allow the UEs not supporting S&F to camp on a cell operating in the S&F mode, but may reject the access attempts from these UEs due to the lack of the feeder link. We think the Rel-19 UE may support the S&F operation in different ways. Some UEs may have full support for the S&F operation including the support from the NAS layer while other UEs may have only limited support (e.g., only RRC support) for the S&F operation. The access control for the UE with limited S&F support should be less restrict than the UE not supporting S&F, but more restrict than the UE with full S&F support. Therefore, if a Rel-19 UE can report its S&F capability in details at the earliest timing during the access attempt, the network can better judge whether to accept the access attempt based on the UE capability and also based on the deployment option (e.g., split-MME or full-EPC onboard) employed by the network for supporting the S&F operation.

A Rel-19 UE can inform eNB of its S&F capability in details (e.g., full S&F support including the NAS-layer support, or only limited S&F support in the RRC layer) via MSG3.

As unsuccessful access attempts may consume considerable radio resources, it is beneficial if at least the Rel-19 UEs with limited S&F support (e.g., only RRC-level support) can refrain themselves from attempting to access under certain circumstance (e.g., the S&F operation indication is present and cellBarredNTN = ‘Barred’). It is even more beneficial if such UEs can know in advance when the feeder link will resume (and hence the S&F operation indication will disappear) in the cell operating in the S&F mode, so that these UEs will not even need to check the S&F operation indication, or can even freeze their NAS layer before the time when the feeder link is resumed.

It is beneficial for the Rel-19 UE with limited S&F support to know when the feeder link will resume, while camping on the cell operating in the S&F mode.

On the other hand, while camping on a cell operating in normal mode (i.e., not broadcasting the S&F operation indication), it is also beneficial for the Rel-19 UE with limited S&F support to know when the feeder link will break. With this knowledge, the Rel-19 UE with limited S&F support can determine whether to initiate/trigger an access attempt based on whether there is sufficient time to fulfill the access purpose.

It is beneficial for the Rel-19 UE with limited S&F support to know when the feeder link will break, while camping on the cell not operating in the S&F mode.

Therefore, to provide better access control for the Rel-19 UEs with limited S&F support, a remaining duration associated with the current feeder link status can be broadcast by the cell supporting the S&F operation. If the remaining duration is signaled together with the S&F operation indication, the remaining duration indicates when the feeder link will become available; if the remaining duration is signaled but the S&F operation indication is absent, the remaining duration indicates when the feeder link will become unavailable.

A remaining duration associated with the current feeder link status can be broadcast by the cell supporting the S&F operation.

When the S&F operation indication is present, the remaining duration indicates when the feeder link will become available.

When the S&F operation indication is absent, the remaining duration indicates when the feeder link will become unavailable.

As the remaining duration associated with the current feeder link status is a value that would decrease every time when it appears in the system information, the changes of the remaining duration (associated with the current feeder link status) should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1. The UE should only need to acquire the remaining time once and then maintain and elapse the value automatically by itself.

## Impact on the cell reselection

For the UEs with limited S&F support, the principle is to make these UEs stay in the cells operating in the normal mode for as long as possible, even if they are allowed to camp on the cell operating in the S&F mode. Therefore, while camping on the cell operating in the normal mode but indicating when the feeder link will become unavailable, the Rel-19 UE with limited S&F support should trigger the intra-/inter-frequency measurement before the feeder link becomes unavailable. Similar to the case of t-service in Rel-18, the UE should trigger the intra-/inter-frequency measurement on the neighbouring cells regardless of other triggering conditions (e.g., the serving cell quality, the distance to the reference location).

While camping on the cell indicating when the feeder link will become unavailable, the Rel-19 UE can trigger the intra-/inter-frequency measurement before the feeder link becomes unavailable.

To make it easier for the UE to detect, measure, and reselect a cell operating in the normal mode, the network can provide a list of cell (PCIs) that are not operating in the S&F mode as part of the cell reselection information/configuration.  The UE can then prioritize the cells in the list while making the cell reselection decision.

Rel-19 UE can be provided with a list of cells that are not operating in the S&F mode.

Even for the Rel-19 UE with full S&F support, camping on the cell having a feeder link is better than camping on the cell with no feeder link available, if these cells are similar in terms of cell quality. This is because camping on the cell with feeder link means the UE is always able to receive the paging as well as the ETWS and CMAS notification in real-time regardless of the network deployment option (e.g., split-MME or full CN on board), and the MT or MO data will have shorter latency (as there is no feeder link interruption).

Even for the Rel-19 UE with full S&F support, camping on the cell with feeder link is better than camping on the cell with no feeder link available, if these cells are similar in terms of cell quality.

Therefore, we think it is beneficial if the Rel-19 UE with full S&F support can also trigger the intra-/inter-frequency measurement, upon detecting the presence of the S&F operation indication. The prerequisite of triggering the intra-/inter-frequency measurement is that the UE has been provided with the list of cells not operating in the S&F mode, otherwise the UE may end up wasting powering if all the neighboring cell are operating in the S&F mode.

If being provided with the list of cells not operating in the S&F mode, the Rel-19 UE supporting the S&F operation can trigger the intra-/inter-frequency measurement, upon detecting the presence of the S&F operation indication.

## The suspension of UE’s RRC and NAS in split-MME payload option

While connecting to a cell operating in the S&F mode, the UE with full S&F support may first establish its RRC connection with the cell and then initiate a NAS procedure by transmitting a NAS message. For the split-MME payload option or for the scenario when only the full eNB is onboard, it may take long time for the UE to receive the response to the NAS message, most likely after the UE has become out of coverage of the connected satellite and returned to the coverage again of the satellite carrying the NAS response. Therefore, during the lifetime of a NAS transaction, the UE may have moved between different RRC states multiple times. As these transitions between different RRC states may clear/reset the NAS progress especially if the UE has moved to RRC_IDLE once, we should avoid UE moving to the idle state (RRC_IDLE as well as ECM_IDLE) whenever the feeder link or the service link becomes unavailable if there is an on-going NAS procedure. For this purpose, when the feeder is or becomes not available, eNB can suspend UE’s RRC connection by sending an RRC Connection Release message to the UE with full S&F support, where the RRC Connection Release message may include an rrc-InactiveConfig IE or set the releaseCause as ‘rrc-Suspend’. Upon receiving the RRC Connection Release message, the UE RRC indicates the suspension of the RRC to the NAS layer, which may result in the suspension to the NAS layer as well.

Since the existing RRC_INACTIVE state and RRC_IDLE state with RRC suspension is only applicable to the UE that has already attached/registered to a network, it can be further discussed if a new RRC suspension state/mode is needed (i.e., a new release cause is needed) to support the initial ATTACH scenario.

When the feeder-link is not available, eNB can suspend UE’s RRC and NAS by transmitting an RRC Connection Release message to the UE with full S&F support. FFS whether a new release cause is needed.

# Conclusion

In this paper, we discuss the impact of the new S&F operation indication on the access control and cell reselection, and have the following observations.

Observation 1	The UE not supporting S&F may be allowed to camp on the cell providing the S&F operation indication, depending on the legacy barring bit.

Observation 2	It is beneficial for the Rel-19 UE with limited S&F support to know when the feeder link will resume, while camping on the cell operating in the S&F mode.

Observation 3	It is beneficial for the Rel-19 UE with limited S&F support to know when the feeder link will break, while camping on the cell not operating in the S&F mode.

Observation 4	Even for the Rel-19 UE with full S&F support, camping on the cell with feeder link is better than camping on the cell with no feeder link available, if these cells are similar in terms of cell quality.

Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	A Rel-19 UE can inform eNB of its S&F capability in details (e.g., full S&F support including the NAS-layer support, or only limited S&F support in the RRC layer) via MSG3.

Proposal 2	A remaining duration associated with the current feeder link status can be broadcast by the cell supporting the S&F operation.

Proposal 3	When the S&F operation indication is present, the remaining duration indicates when the feeder link will become available.

Proposal 4	When the S&F operation indication is absent, the remaining duration indicates when the feeder link will become unavailable.

Proposal 5	While camping on the cell indicating when the feeder link will become unavailable, the Rel-19 UE can trigger the intra-/inter-frequency measurement before the feeder link becomes unavailable.

Proposal 6	Rel-19 UE can be provided with a list of cells that are not operating in the S&F mode.

Proposal 7	If being provided with the list of cells not operating in the S&F mode, the Rel-19 UE supporting the S&F operation can trigger the intra-/inter-frequency measurement, upon detecting the presence of the S&F operation indication.

Proposal 8	When the feeder-link is not available, eNB can suspend UE’s RRC and NAS by transmitting an RRC Connection Release message to the UE with full S&F support. FFS whether a new release cause is needed.

# Reference

RP-234007, New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

Draft Report of 3GPP TSG RAN WG2 meeting #127bis, ETSI MCC.