3GPP TSG-RAN WG2 Meeting #128                                                        R2-2409854

Orlando, USA, 18th – 22nd Nov 2024

Source:	CATT

Title:	Discussion on Enabling TX/RX for XR during RRM Measurements

Agenda Item:	8.7.3

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting, RAN2 started the first discussion on RRM measurement gaps/restrictions related enhancements. And the following agreements were achieved.

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

In this contribution, we give our further considerations on it.

Discussion

Impact on DSR

The behavior of MAC entity during an activated measurement gap is captured in TS 38.321 [1]  below.

- 5.14 Handling of measurement gaps During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH.

According to the above description, the MAC entity of UE considers the TX/RX restriction during measurement gap per serving cell. For example: If per FR1 gap is configured, the MAC entity of UE considers the TX/RX restriction on the serving cell(s) in the FR1 range during per FR1 gap.

Observation 1: The MAC entity of UE considers the TX/RX restriction during measurement gap per serving cell.

As it is known to all that DSR is triggered for an LCH when the remaining time before discard of any buffered PDCP SDU goes below a configured threshold and used to provide the network with delay status per LCG, including remaining time and so on. Therefore, considering granularities are different, there is mismatch between measurement gap and DSR triggering/reporting. If DSR triggering/reporting is enhanced with measurement gap, some inaccuracy information may be introduced. For example: An LCH is configured to trigger DSR when the remaining time before discard of any buffered PDCP SDU goes below a configured threshold while per FR1 gap is configured to the UE. For the data belongs to the LCH, it is possible they are transmitted via any serving cell. In this case, the UE cannot determine if the remaining time is shortened by the activated per FR1 gap or not as during per FR1 gap as the data belongs to the LCH may be transmitted via FR2 serving cell. Then if the remaining time is always shortened by the activated per FR1 gap (i.e. assuming the activated per FR1 gap always interrupts TX/RX), the enhanced remaining time is shorter than the actual remaining time before discard of the PDCP SDU. Then DSR triggered for the LCH maybe earlier if DSR triggered is enhanced with measurement gap, or DSR reporting for the LCG is not accurate if remaining time in DSR reporting is enhanced with measurement gap. If TX/RX for XR during measurement gap with DCI mechanism is considered together, DSR enhancement(s) are more complex as the network doesn’t know if UE has already received the DCI with measurement gap or not when DSR is triggered or reported. Considering complexity and accuracy, we propose not to consider remaining DSR enhancement (including DSR triggering and reporting) with measurement gap.

Proposal 1: Not to consider DSR enhancement (including DSR triggering and delay status calculation) specific for measurement gap.

DRX impact

Measurement gap/restriction may be overlapped with the Active Time in a DRX group or not.

If measurement gap/restriction is overlapped with the Active Time in a DRX group, the network can use DCI to skip measurement gap/restriction if data delay is critical. No additional impact on the Active Time in a DRX group with measurement gap/restriction is introduced.

And if measurement gap/restriction is not overlapped with the Active Time in a DRX group, the network cannot use DCI to skip measurement gap/restriction. But during the inactive time in a DRX group, there may be CG or SPS occasions with stringent latency requirements overlapped with measurement gap/restriction, as shown in Figure 1. The corresponding CG or SPS occasions are not expected to be disabled due to measurement gap/restriction. As the network cannot use DCI to skip measurement gap/restriction, the existing agreements on enabling TX/RX for XR during measurement gap/restriction cannot handle the case.

Figure 1 Example of CG occasions overlapped with measurement gap and DRX

In LTE, the issue of DRX and DL SPS was discussed [2] and the following agreement was reached:

R2-131173 DRX and DL SPS Renesas Mobile Europe Ltd Disc REL-10 LTE-L23, TEI10 - Samsung think most eNB will schedule DL SPS during On Duration, but also ok with Alt-3. But Samsung is not happy with Alt-2. NSN think Alt-3 is already discussed in Rel-8 discussion. RIM point out that this was discussed in RAN2#65, and Alt-1 is enough. LG also think that DL SPS and On Duration is aligned, and Alt1 should be enough. LG think even if DL SPS and On Duration are not aligned, the DL SPS retransmission can be transmitted at next On Duration. CATT point out that the UE shall process DL SPS only in Active Time. AsusTek point out that R2-091481 already discussed the same issue, and there was a common understanding that the UE can process DL SPS outside of Active Time. RIM clarified that UE may choose to process DL SPS outside of Active Time, but there is no requirement. - Renesas think if companies think Alt-1 is enough, it should be captured in MAC specification. => Alt-1 is enough. - eNB implementation based solution, i.e. eNB always configure the DL SPS occasion align with the onDurationTimer (according to the long DRX cycle), in this case, eNB could make sure that all the configured DL assignments also happen during active time so UE will start and stop the timer correctly.

Observation 2: In LTE, eNB could make sure that all the configured DL assignments also happen during active time of DRX.

We wonder if the same principle is applied to NR, i.e. NG-RAN node could make sure that all the SPS/CG occasions happen during active time of the DRX group. If it is, the case that out of the Active Time in a DRX group CG/SPS occasions are overlapped with measurement gap/restriction is a corner case. We don’t need to consider solutions to handle it. Otherwise, RAN2 need to discuss the corresponding solutions.

Proposal 2: RAN2 discuss whether there is a need to consider the case that out of the Active Time in a DRX group CG/SPS occasions are overlapped with measurement gap/restriction.

UE assistant information

In RAN2#117 meeting, RAN1 discussed UE assistance information for enabling TX/RX during RRM measurements. And the following agreement was made:

Agreement RAN1 agrees to send an LS to RAN4 (CC: RAN2) to convey the following information about UE assistance information (including the conclusion on UAI): 1. Overall Description: RAN1 discussed UE assistance information related to measurements occasions and related to channel conditions, traffic, UE mobility. The following agreement and conclusions were agreed: Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable. 2. Actions: To RAN4: ACTION: RAN1 kindly asks RAN4 to consider the above information into account and decide whether or not to introduce any UE assistance information related to measurement occasions.. Final LS is approved in R1-2405736.

Considering UE assistance information related to measurements occasions may impact on RRM performance, RAN1 kindly asked RAN4 to decide whether or not to introduce any UE assistance information related measurement occasions [3]. In RAN4#112 meeting, RAN4 started the discussion on the objective and no agreement was made on UE assistance information related to measurements occasions. RAN2 can wait for RAN4’s conclusions on UE assistance information related to measurements occasions.

Proposal 3: RAN2 waits for RAN4 conclusions on UE assistance information related to measurements occasions before further discussion.

Conclusion

According to the analysis in section 2, it is proposed:

Observation 1: The MAC entity of UE considers the TX/RX restriction during measurement gap per serving cell.

Proposal 1: Not to consider DSR enhancement (including DSR triggering and delay status calculation) specific for measurement gap.

Observation 2: In LTE, eNB could make sure that all the configured DL assignments also happen during active time of DRX.

Proposal 2: RAN2 discuss whether there is a need to consider the case that out of the Active Time in a DRX group CG/SPS occasions are overlapped with measurement gap/restriction.

Proposal 3: RAN2 waits for RAN4 conclusions on UE assistance information related to measurements occasions before further discussion.

Reference

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

- R2-131173, DRX and DL SPS, Renesas Mobile Europe Ltd, RAN2#81bis

- R1-2405736, LS on UE assistance information, RAN1#117