3GPP TSG-RAN WG2 Meeting #128                                                        R2-2409860

Orlando, USA, 18th – 22nd Nov, 2024

Source:	CATT

Title:	Discussion on the Control Plane Procedures

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting, the following agreements on control plane procedure were reached:

The following connections are assumed as a baseline to be needed: - From last Relay UE perspective: A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED. - From intermediate relay UE perspective (including first relay UE): A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED. FFS what RRC states are supported for the intermediate relay UE. - From U2N Remote UE perspective: A direct (non-relayed) PC5 connection with Intermediate Relay UE, and An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED. The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18). The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

After RAN2#127, a long email discussion on control plane [4] was initiated and in our understanding Approach 1 is preferred. Hence, in this contribution, we will further discuss the details of the control plane procedures based on Approach 1.

Discussion

Whether integrated connection setup is supported?

In Rel-18 U2U relay, regarding to the relationship between discovery and connection setup, there are two alternatives:

- Alt 1:  Separate discovery and connection setup procedure;

- Alt 2:  Integrated discovery and connection setup procedure.

In TR23.700-03 [2], the following conclusions were reached:

- Standalone discovery & link setup/management is considered and both Model A and Model B are supported. - When Model A discovery is performed, the Remote UE may choose the Intermediate Relay based on the announcement message sent by the Intermediate Relay. - When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay. To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list of User Info ID of Relays in the selected path. The Remote UE sends the selected path information to the Intermediate UE-to-Network Relay for communication setup.

Based on the above conclusions, it is obvious that SA2 already agreed that standalone discovery and link setup is considered and Remote UE is responsible for the intermediate relay selection. Considering there is no need to establish the E2E PC5-RRC connection between the remote UE and the U2N relay UE connected with network via Uu, hence, integrated relay discovery and connection setup procedure is not considered in Rel-19.

Proposal 1:  Separate discovery and connection setup procedure is used for Rel-19 multi-hop relay.

How to establish the E2E connection between remote UE and network?

In Rel-17 U2N relay, the E2E connection setup procedure is as below:

Figure-1 Procedure for L2 U2N Remote UE connection establishment (Rel-17)

In case of multi-hop, according to the discussion in [4], the baseline connection establishment procedure was captured. But it only captured the main procedure, not every detail. Hence, in the following, a more detailed procedure was given as below:

Figure-2 Procedure for L2 U2N Remote UE connection establishment (Rel-19)

For the above Figure-2, most of the steps are similar as Rel-17 U2N relay. Our discussion mainly focuses on step 3, step 4, step 5 and step 9.

Step 3

For step 3, in Rel-17 U2N relay, specified PC5 RLC channel can be used to transmit SRB0 between Remote UE and U2N relay. The same principle can be reused in Rel-19.

Proposal 2:  Specified PC5 RLC channel can be used to transmit E2E SRB0 between remote UE and intermediate relay UE.

Step 4

After step 3, when the first relay UE receives the SL-RLC0, its next behavior depends on its connection state and RRC state.

Based on the solutions listed in TR23.700-03 [2], SA2 made the following conclusions [3]:

SA2 would like to inform SA3 that for Rel-19 FS_5G_ProSe_Ph3 study has concluded, and the solutions were documented in TR 23.700-03. For the support of multi-hop UE-to-Network Relay operations (KI#1 of TR 23.700-03), it was concluded that: - for Model A based discovery and corresponding link establishment/management, Solution#1 (as documented in clause 6.1 of TR 23.700-03) would be used as basis for normative work; - for Model B based discovery and corresponding link establishment/management, Solution#2 and #7 (as documented in clause 6.2 and 6.7 of TR 23.700-03) would be used as basis for normative work.

For relay discovery model A, solution #1 in TR23.700-03 [2] is baseline and for relay discovery model B, solution #2 and solution#7 is baseline.

For Relay discovery model A, the following was captured in solution #1 in TR23.700-03 [2]:

To act as an Intermediate UE-to-Network Relay, e.g. forwarding the Model A discovery message or replying to the Model B Discovery Response message, the relay UE needs to have a valid connection towards the network via the 5G ProSe UE-to-Network Relay indicated by the Root Relay Info IE.

Based on the above, it can be deduced that relay discovery model A, there should be PC5-RRC connections between intermediate relay UE and its parent relay. But considering there may be no data transmission requirement during discovery procedure, both intermediate relay UE and U2N relay can be in RRC_IDLE/INACTIVE. That is to say, there may be no E2E connection between intermediate relay and network.

But for relay discovery model B, there is no requirement on PC5-S/PC5-RRC connection in TR23.700-03[2].

Proposal 3: RAN2 confirms that when the first relay UE receives the E2E SRB0 from the remote UE:

- For relay discovery Model A, the intermediate relay UE (including the first relay UE) should have PC5-S/PC5-RRC connection with its parent relay UE;

- For relay discovery Model B, the intermediate relay UE (including the first relay UE) may have no PC5-S/PC5-RRC connection with its parent relay UE.

Proposal 4: RAN2 confirms that when the first relay UE receives the E2E SRB0 from the remote UE, it is possible that both the intermediate relay UE (including the first relay UE) and the last relay UE are in RRC_IDLE/INACTIVE state.

Based on Proposal 3/4, upon receiving E2E SRB0 from remote UE, if the first relay UE has no PC5-S/PC5-RRC connection with its parent relay UE, it should trigger the PC5-S/PC5-RRC connection setup with its parent relay UE. Once the PC5-S/PC5-RRC connection is setup, it should further establish the E2E RRC connection with the gNB in order to send SUI to gNB to request the local ID for the remote UE.

Proposal 5: Upon receiving E2E SRB0 from the remote UE, the first relay UE should establish the PC5-S/PC5-RRC connection with its parent relay UE (if not exists) and establish the E2E RRC connection with the gNB (if not exists).

If Proposal 5 is agreed, once the E2E connection between the first relay UE and gNB is established, it should further discuss which relay UE send sidelinkUEInformationNR to network to request the local L2 ID of remote UE and request the Egress RLC channel configuration for routing SL-RLC0. There are two alternatives:

- Alternative 1: The first relay UE sends sidelinkUEInformationNR to network;

- Alternative 2: The last relay UE sends sidelinkUEInformationNR to network.

No matter which alternative is used, network should aware the list of all relay UEs on the multi-hop path in order to provide the proper SRAP configuration for each relay UE. If Alternative 1 is used, the network can aware the list of all relay UEs on the multi-hops since each intermediate relay UE establish the E2E connection with network through its parent relay. But if Alternative 2 is used, the network cannot aware the list of all relay UEs and enhancement is needed. For example, the last relay should get the list of all relay UEs and report it to network via sidelinkUEInformationNR. This will bring specification effort. Hence, Alternative 1 is slightly preferred.

Proposal 6: Upon receiving SL-RLC0 from remote UE, the first relay UE can send SidelinkUEInformationNR to network.

Step 5

Based on sidelinkUEInformationNR sent by intermediate relay UE, network can allocate the local L2 ID for remote UE and at least configure the mapping between SRB0 and its Egress RLC channel for each intermediate relay and U2N relay.

Proposal 7: Upon receiving SidelinkUEInformationNR from the first relay UE, network can allocate Local L2 ID for remote UE and at least configure the mapping between SRB0 and its Egress RLC channel for each relay UE in the multi-hop path.

Step 9

After remote UE receiving the RRCSetup, it should further discuss which PC5 RLC channel can be used for transmission the E2E SL-SRB1. In legacy Rel-17 U2N relay, default SL-RLC1 can be used. The same principle can be used for multi-hop U2N relay.

Proposal 8:  Default PC5 RLC channel can be used to transmit E2E SRB1 between remote UE and the first relay UE.

QoS split for Multi-hop relay

In Rel-17, for L2 U2N relay, the network is responsible for QoS split between the Uu and PC5 interface. And only one parameter needs to be split, that is PDB.

In Rel-19, with the introduction of multi-hop relay, in order to meet the E2E QoS requirement, it should also discuss which node is responsible for QoS split and which QoS parameter needs to be split.

Based on Proposal 5, if there is E2E RRC connection between each relay UE and network, it is feasible to reuse the QoS split method in Rel-17 L2 U2N relay. That is the network can decide how to split the QoS for each hop and the network should inform the split result to remote UE and each relay UE. The parameter needs to be split is PDB.

Proposal 9: The network is responsible for the QoS split for each hop.

Proposal 10: Same as Rel-17 L2 U2N relay, the parameter needs to be split is PDB.

System information forwarding

The legacy Rel-17, system information forwarding procedure is as below：

Figure-3 System information forwarding for Rel-17 U2N Relay

In Rel-19, with the introduction of multi-hop, the similar procedure can be reused. That is remote UE can send the request SIB(s) to the first relay UE, and then, the first relay UE can forward the request to its parent relay UE, and its parent relay UE can further forward it until the last relay UE receives it. The only difference compared with Rel-17 is that when each intermediate relay UE (including the first relay UE) performs the forwarding, it can include the SIB(s) request by both its child UE(s) and itself.

Proposal 11: For multi-hop relay, upon receiving the request SIB(s) from its child UE(s), the intermediate relay UE (including the first relay UE) can forward the requested SIB(s) from its child UE(s) and the request SIB(s) of itself to its parent relay UE.

Paging forwarding

In legacy ReL-17, IDLE/INACTIVE remote UE can notify its UE ID (e.g., 5G-S-TMSI or I-RNTI) to the U2N relay UE. And the U2N relay UE will monitor the corresponding PO and forwarding the paging to the remote UE if the paging message contains the Remote UE ID.

In Rel-19, with the introduction of multi-hop, the similar procedure can be reused. That is the immediate relay UE forward the remote UE ID to the first relay UE, and the first relay UE performs the PO monitoring and paging forwarding.

Proposal 12: For multi-hop relay, the intermediate relay UE can notify the remote UE ID(s) to the first relay UE and the first relay UE can monitor the PO for the served remote UE(s).

RLF handling

With the introduction of multi-hop, there are three kinds of links as below:

- Sidelink 1: The sidelink between Remote UE and first relay UE;

- Sidelink 2: The sidelink between relay UEs;

- Uu link: The Uu link between the last relay UE and gNB.

Figure-4  Multi-hop relay

When analyzing the RLF handling, we should discuss based on the above three kinds of links:

Table-1 RLF handling for multi-hop relay

| Failed link | Legacy Rel-17 behaviors | Rel-19 behaviors |
| --- | --- | --- |
| Sidelink 1 | Remote UE detected PC5 RLF: - Initiate RRC connection re-establishment procedure. Relay UE detected PC5 RLF: - Send SUI to gNB. | Remote UE detected PC5 RLF: - Initiate RRC connection re-establishment procedure (same as Rel-17). First relay UE detected PC5 RLF: - Send SUI to gNB (same as Rel-17). |
| Sidelink 2 | N/A | First relay UE detected PC5 RLF: - Send NotificationMessageSidelink message to remote UE with new indicationType (e.g., “relay UE-PC5-RLF”) Last relay UE detected PC5 RLF: - Send SUI to gNB (same as Rel-17). |
| Uu link | - Send NotificationMessageSidelink message to remote UE. | - Send NotificationMessageSidelink message to its child relay UE and its child relay UE should forward it to the Remote UE. |

In the above table, the red part is new UE behaviors introduced by multi-hop.

Proposal 13: For multi-hop, when remote UE detects PC5 RLF, it can initiate RRC connection re-establishment procedure (same as Rel-17).

Proposal 14: For multi-hop, when the first relay UE detects PC5 RLF between the remote UE and the first relay UE, it should send SUI to gNB (same as Rel-17).

Proposal 15: For multi-hop, when the first relay UE detects PC5 RLF between the first relay UE and immediate relay/last relay UE, it can send NotificationMessageSidelink to the remote UE with new indicationType of “relayUE-PC5-RLF”.

Proposal 16: For multi-hop, when the last relay UE detects PC5 RLF, it can send SUI to gNB (same as Rel-17).

Proposal 17: For multi-hop, when the last relay UE detects Uu RLF, it can send NotificationMessageSidelink to its child relay UE, and its child relay UE should forward it to the remote UE.

Conclusion

According to the analysis in section 2, it is proposed:

RRC connection establishment procedure

Proposal 1:  Separate discovery and connection setup procedure is used for Rel-19 multi-hop relay.

Proposal 2:  Specified PC5 RLC channel can be used to transmit E2E SRB0 between remote UE and intermediate relay UE.

Proposal 3: RAN2 confirms that when the first relay UE receives the E2E SRB0 from the remote UE:

- For relay discovery Model A, the intermediate relay UE (including the first relay UE) should have PC5-S/PC5-RRC connection with its parent relay UE;

- For relay discovery Model B, the intermediate relay UE (including the first relay UE) may have no PC5-S/PC5-RRC connection with its parent relay UE.

Proposal 4: RAN2 confirms that when the first relay UE receives the E2E SRB0 from the remote UE, it is possible that both the intermediate relay UE (including the first relay UE) and the last relay UE are in RRC_IDLE/INACTIVE state.

Proposal 5: Upon receiving E2E SRB0 from the remote UE, the first relay UE should establish the PC5-S/PC5-RRC connection with its parent relay UE (if not exists) and establish the E2E RRC connection with the gNB (if not exists).

Proposal 6: Upon receiving SL-RLC0 from remote UE, the first relay UE can send SidelinkUEInformationNR to network.

Proposal 7: Upon receiving SidelinkUEInformationNR from the first relay UE, network can allocate Local L2 ID for remote UE and at least configure the mapping between SRB0 and its Egress RLC channel for each relay UE in the multi-hop path.

Proposal 8:  Default PC5 RLC channel can be used to transmit E2E SRB1 between remote UE and the first relay UE.

QoS split

Proposal 9: The network is responsible for the QoS split for each hop.

Proposal 10: Same as Rel-17 L2 U2N relay, the parameter needs to be split is PDB.

System information forwarding

Proposal 11: For multi-hop relay, upon receiving the request SIB(s) from its child UE(s), the intermediate relay UE (including the first relay UE) can forward the requested SIB(s) from its child UE(s) and the request SIB(s) of itself to its parent relay UE.

Paging forwarding

Proposal 12: For multi-hop relay, the intermediate relay UE can notify the remote UE ID(s) to the first relay UE and the first relay UE can monitor the PO for the served remote UE(s).

RLF handling

Proposal 13: For multi-hop, when remote UE detects PC5 RLF, it can initiate RRC connection re-establishment procedure (same as Rel-17).

Proposal 14: For multi-hop, when the first relay UE detects PC5 RLF between the remote UE and the first relay UE, it should send SUI to gNB (same as Rel-17).

Proposal 15: For multi-hop, when the first relay UE detects PC5 RLF between the first relay UE and immediate relay/last relay UE, it can send NotificationMessageSidelink to the remote UE with new indicationType of “relayUE-PC5-RLF”.

Proposal 16: For multi-hop, when the last relay UE detects PC5 RLF, it can send SUI to gNB (same as Rel-17).

Proposal 17: For multi-hop, when the last relay UE detects Uu RLF, it can send NotificationMessageSidelink to its child relay UE, and its child relay UE should forward it to the remote UE.

Reference

- RP-241609   New WID on NR sidelink multi-hop relay

- TR23700-03   Study on system enhancement for Proximity based Service (ProSe) in the 5G System (5GS) Phase 3

- S2-2407207   LS on security considerations for 5G ProSe multi-hop relays

- Report of [POST127][402][Relay] Multi-hop relay control plane