3GPP TSG-RAN WG2 Meeting #128	                                          R2-2409867

Orlando, USA, 18 – 22, Nov, 2024

Source: 	Xiaomi

Title:  	Simulation assumptions on event/RLF/SLS and model generalization

Agenda Item:	8.3.5

Document for:	Discussion and Decision

Introduction

In addition to RRM, this SI also includes event and RLF prediction. Furthermore, system level simulation (SLS) is expected to evaluate the performance gain. In this contribution, we discuss the simulation assumptions for event, RLF and SLS.

Discussion

Event prediction

Regarding following simulation parameters for FR2 intra-frequency temporal domain case A, some discussion is needed on the OW length. In case A, continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. OW length can be up to UE implementation e.g. can be optimized to achieve the best prediction accuracy. Companies can report OW length used in simulation.

| Parameters | Recommended value |
| --- | --- |
| A3 event offset (db) | 2 |
| TTT (ms) | 160 |
| UE speed (km/h) | 30 |
| OW length (ms) | 400 |
| PW length (ms) | 200 |
| Max ETD (ms, note1) | 40 |

Proposal 1: For FR2 intra-frequency temporal domain case A, companies can pick and report what they are using for OW length.

For Case B, one discussion point is related to how L3 RSRP filtering is performed considering that in some time instances, actual measurement results are not available. Take following as example:

Figure 2.1.3-1: Temporal domain case B example 2

The 3 options also mean:

- Filtering option 1: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T2;

- Filtering option 2: L3 cell result at T3 is filtered based on the L1 filtered result at T3; in other words, not consider old L3 filtered result.

- Filtering option 3: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T1 (i.e. the old L3 filtered cell result from last actual measurement);

L3 RSRP filtering for Case B is a common issue for both RRM prediction and measurement event prediction when indirect method is used. RAN2 has discussed this aspect for RRM prediction in RAN2#127, with the following agreement:

In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement

It is suggested that L3 RSRP filtering option for Case B is aligned for RRM prediction and measurement event prediction.

Proposal 2: L3 RSRP filtering option for Case B is aligned for RRM prediction and measurement event prediction.

RLF prediction

In the email discussion, several metrics are proposed to evaluate the RLF prediction accuracy. It’s proposed to reuse the event prediction evaluation metrics, i.e. precision, recall and F1 score. However, we want to point out the impact to NW of miss RLF detection and false RLF detection is not the same. If AI miss RLF detection, there is no harm, since the consequence is the same as legacy, i.e. RLF. If AI false predict RLF, it may harm the NW. Because NW may decide to handover the UE or UE would trigger RRC reestablishment due to false prediction, which would result in additional interruption. Precision is used to evaluate the positive prediction accuracy. So, we should prioritize the evaluation of precision.

Proposal 3: RAN2 to prioritize the precision for RLF prediction.

It’s discussed how to determine the serving signal of RLM. Majority companies agreed to use the serving beam. However, it’s not clear how to determine the serving beam. We don’t need to simulate the beam management procedure. The strongest beam, i.e. with highest RSRP, can be assumed as serving beam.

Proposal 4: The strongest beam, i.e. with highest RSRP, can be assumed as serving beam.

Although RAN2 has agreed HO is not simulated in RLF simulation, we understand the RLF data may not be practical. Because the UE is expected to handover in the SLS, so most of the predicted RLF would not occur. No gain would be observed. We understand the RLF may be caused by sudden radio channel condition change in practice. In the simulation, we can introduce artificial blockage, which results in sudden radio channel condition change. How to model the blockage can be up to companies. In the SLS, the same blockage can be maintained, the predicted RLF can assist NW to avoid the RLF. NW can handover UE to other frequencies if available.

Proposal 5: Artificial blockage can be introduced to accelerate RLF. How to model the blockage can be up to companies.

SLS

HO model shall be aligned between companies. However, we observed there are multiple understanding from companies. In our understanding, HO modelling for case A and event prediction can be as following,

- NW can start the HO preparation after receiving the event prediction.

- After prediction, UE would still perform measurement to evaluate A3 event and eventually trigger MR if event is fulfilled.

- Upon receiving the real MR, NW sends HO command if HO preparation is finished. Otherwise, NW sends HO command after HO preparation is finished.

The advantage of this modelling is the false prediction would not trigger early HO. On the other hand, NW can always save the HO preparation time if the prediction is correct. There is no side affect due to prediction. Maybe some inter-node signaling would be wasted due to false prediction. But not much inter-node signaling is needed.

Proposal 6: RAN2 agree following HO modelling for case A and event prediction,

- NW can start the HO preparation after receiving the event prediction.

- After prediction, UE would still perform measurement to evaluate A3 event and eventually trigger MR if event is fulfilled.

- Upon receiving the real MR, NW sends HO command if HO preparation is finished. Otherwise, NW sends HO command after HO preparation is finished.

For case B, the HO modelling in [1] can be reused. The MR event is evaluated based on both real and predicted measurement results. We don’t see the motivation to evaluate the MR only based on real measurement results, which makes prediction useless.

Proposal 7: For case B, the HO modelling in [1] can be reused. The MR event is evaluated based on both real and predicted measurement results.

Model generalization

It’s necessary to study the model generalization. However, we understand the AI model can’t be generalize across the cases defined as following,

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Especially for the high prioritized cases, the model structure and input/output pattern may not be the same. Case 2 and 4 are temporal prediction, while case 3 is frequency domain prediction. In case 4, the inputs are consecutive on temporal domain. While, in case 2, two options are supported as following,

Figure 1 Example 1 of intra-frequency temporal domain case B

Figure 2 Example 2 of intra-frequency temporal domain case B

The example 2 is not aligned with the pattern of case 4. Therefore, it’s difficult to generalize the model across cases.

Proposal 8: RAN2 only evaluates model generalization within each case, i.e. case A, case B or inter-frequency.

In AI air, following methods are supported to evaluate the generalization:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Notes: Companies to report the ratio for dataset mixing. Number of the multiple scenarios/configurations can be larger than two. - The following case for generalization verification, can be optionally considered by companies: - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Companies to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance.

Case 1 is used as baseline. Case 2 and 3 are used to evaluate the performance degradation due to generalization. We can reuse the methods and leave to companies to select and report. Note there is case 2A. However, more options would bring more difficulty to converge. We suggest to focus on case 1/2/3.

Proposal 9: Reuse the following model generalization evaluation methods from [2]

-	Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

-	Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

-	Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

-	Notes: Companies to report the ratio for dataset mixing. Number of the multiple scenarios/configurations can be larger than two.

Then we need to identify the detail of the scenario/configuration for AI mobility. We observe the following parameters are configurable in the simulation assumptions,

| Parameters | Value |
| --- | --- |
| Frequency | FR1/FR2 |
| UE speed (km/h) | 30/60/90 for FR1, 60/90/120 for FR2 |
| OW/PW | Company report |
| Spatial consistency | A/B |
| LOSsoft | Yes/No |
| Trajectory boundary handling | 1/2/3 |
| Trajectory model | 1/2/3 |
| L1/L3 filtering | Sliding/non-sliding |
| Number of TX/RX beam for FR2 | Company report |
| Filtering coefficient factor | Company report |

It can be up to companies to select parameter to evaluate the generalization performance and report. However, the potential configurable parameters are too many. The simulation result may not be enough if companies select different parameters. We suggest RAN2 agrees a minimum parameter set. All companies shall at least evaluate the generalization performance regarding the minimum parameter set. In our understanding, the minimum parameter set can be UE speed, filtering coefficient factor and trajectory model.

Proposal 10: It can be up to companies to select parameter to evaluate the generalization performance and report.

Proposal 11: All companies shall at least evaluate the generalization performance regarding minimum parameter set, i.e. UE speed, filtering coefficient factor and trajectory model.

Conclusions

According to the discussion above, we have the following observations and proposals:

Event prediction

Proposal 1: For FR2 intra-frequency temporal domain case A, companies can pick and report what they are using for OW length.

Proposal 2: L3 RSRP filtering option for Case B is aligned for RRM prediction and measurement event prediction.

RLF prediction

Proposal 3: RAN2 to prioritize the precision for RLF prediction.

Proposal 4: The strongest beam, i.e. with highest RSRP, can be assumed as serving beam.

Proposal 5: Artificial blockage can be introduced to accelerate RLF. How to model the blockage can be up to companies.

SLS

Proposal 6: RAN2 agree following HO modelling for case A and event prediction,

- NW can start the HO preparation after receiving the event prediction.

- After prediction, UE would still perform measurement to evaluate A3 event and eventually trigger MR if event is fulfilled.

- Upon receiving the real MR, NW sends HO command if HO preparation is finished. Otherwise, NW sends HO command after HO preparation is finished.

Proposal 7: For case B, the HO modelling in [1] can be reused. The MR event is evaluated based on both real and predicted measurement results.

Model generalization

Proposal 8: RAN2 only evaluates model generalization within each case, i.e. case A, case B or inter-frequency.

Proposal 9: Reuse the following model generalization evaluation methods from [2]

-	Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

-	Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

-	Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

-	Notes: Companies to report the ratio for dataset mixing. Number of the multiple scenarios/configurations can be larger than two.

Proposal 10: It can be up to companies to select parameter to evaluate the generalization performance and report.

Proposal 11: All companies shall at least evaluate the generalization performance regarding minimum parameter set, i.e. UE speed, filtering coefficient factor and trajectory model.

Reference

[1] TR 36.839

[2] TR 38.843