3GPP TSG-RAN WG2 Meeting #128                              	  R2-2409874

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda Item:	8.6.3

Source: 	OPPO

Title:  	Discussion on event-triggered L1 measurement reporting

Document for:	Discussion and Decision

Introduction

In last meeting, RAN2 has discussed the issues related to event-triggered L1 measurement reporting and the following agreements were reached:

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

In this contribution, we discuss the remaining open issues on event-triggered L1 measurement reporting, including the event configuration as well as the reporting content.

Discussion

Measurement reporting content

In last meeting, RAN2 has reached the following initial agreements on the content of event triggered MR MAC CE:

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

One of the agreement is that UE may report the measurement result of current beam in the triggered MAC CE to NW. The question is that whether R18 indication(i.e. spCellInclusion) on current SpCell measurement result reporting can be reused.

R18 LTM CSI report allows UE to report the serving cell quality if it is configured as an LTM candidate cell, which is aiming to allow NW to have a better understanding of the radio condition of both serving cell and candidate cells. And there is one restriction on supporting SpCell CSI reporting due to the UCI field limitation of CSI report, which is the current SpCell should be configured as an SpCell of an LTM candidate configuration.

For event triggered L1 measurement reporting, MAC CE is used to carry the measurement results and we see no need to follow same restriction of Rel-18 CSI report.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration.

MR MAC CE can include the measurement result associated to the current beam regardless the current SpCell is configured as an SpCell of an LTM candidate configuration or not.

Another question is whether the MAC CE can contain the measurement result of the other candidate RS in addition to triggered RS. During early discussion on the applicable scenario for event triggered L1 measurement, companies have reached consensus that the triggered event is used for NW to select the target cell/beam to early sync procedure and LTM cell switch procedure. Both scenarios require NW to have a full knowledge of the candidate cells/beams. Besides triggered RS, we see no reason to prohibit UE to report the measurement result of other candidate beams.

Except the triggered beam, the measurement report can contain the measurement results of other candidate beams.

Event evaluation

In RAN2#126 meeting, RAN2 made a working assumption that the RS type used for both serving and candidate cell for event evaluation should be same.

4.    Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

And in last RAN2 meeting, RAN2 has confirmed this working assumption. It is questionable that how to guarantee the same RS type for both serving and candidate cell for L1 event evaluation.

In last RAN1 meeting, the following agreement were reached. They defined the way to derive the serving cell RS for event evaluation. The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell.

In general, we may face following cases when perform the L1 event evaluation:

| Cases | QCL RS of Indicated TCI state | Candidate RS type | How to guarantee same RS type |
| --- | --- | --- | --- |
| Case 1 | SSB(not supported) | SSB | The QCL RS of indicated TCI state does not support SSB according to existing QCL rule. |
| Case 2 | SSB(not supported) | CSI-RS | The QCL RS of indicated TCI state does not support SSB according to existing QCL rule. |
| Case 3 | CSI-RS | CSI-RS | The RS for current beam is the QCL RS in the indicated TCI state. |
| Case 4 | CSI-RS | SSB | The RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. |
| Case 5 | TRS | SSB | The RS for current beam is SSB QCLed with the QCL RS in the indicated TCI state. |
| Case 6 | TRS | CSI-RS | The RS for current beam is SSB QCLed with the QCL RS in the indicated TCI state; The RS for candidate beam is SSB QCLed with the candidate beam. |

For case1 and case2, as SSB is not supported to be a QCL RS for TCI state indication. The two cases are naturally inexistence.

For case 3, same RS type of measurement RS is configured for both current beam and candidate beam, UE can use QCL RS in the indicated TCI state as current beam to perform the L1 measurement and evaluate the event accordingly.

For case4, if the QCL RS in the indicated TCI state is CSI-RS while the measurement RS of candidate beam associated with the event configuration is SSB, UE can use SSB which is QCLed with the QCL RS in the indicated TCI state for L1 measurement;

For case 5 and case 6, the QCL RS of indicated TCI state is TRS. Currently, TRS as measurement RS of current beam for determining L1-RSRP TRS is not supported, to be simple, we can avoid indicating TRS as QCL RS of indicated beam. Otherwise, the SSB QCLed with the TRS in the indicated TCI state can be based for L1 measurement and event evaluation. For example, if the QCL RS in the indicated TCI state is TRS while the measurement RS of candidate beam associated with the event configuration is SSB, UE can use SSB which is QCLed with the QCL RS in the indicated TCI state for L1 measurement and event evaluation; if the QCL RS in the indicated TCI state is TRS while the measurement RS of candidate beam associated with the event configuration is CSI-RS, UE can use SSB which is QCLed with the QCL RS in the indicated TCI state and the SSB which is QCLed with candidate beam for L1 measurement and event evaluation.

According to the discussion and analysing above, we have the following proposals:

If the QCL RS in the indicated TCI state is CSI-RS and the measurement RS of candidate beam associated with the event configuration is CSI-RS, UE uses the QCL RS in the indicated TCI state for event evaluation.

If the QCL RS in the indicated TCI state is CSI-RS while the measurement RS of candidate beam associated with the event configuration is SSB, UE uses SSB which is QCLed with the QCL RS in the indicated TCI state for event evaluation.

If the QCL RS in the indicated TCI state is TRS while the measurement RS of candidate beam associated with the event configuration is SSB, UE uses SSB which is QCLed with the indicated TCI state for event evaluation.

If the QCL RS in the indicated TCI state is TRS while the measurement RS of candidate beam associated with the event configuration is CSI-RS, UE uses SSB which is QCLed with the QCL RS in the indicated TCI state and the SSB which is QCLed with candidate beam for event evaluation.

For P3-P6, RAN2 further study whether to rely on NW indication to guarantee same RS type is used for both serving beam and candidate beam or we capture the UE behaviour explicitly.

Send LS to RAN1 to check the feasibility of the proposals above.

Another FFS from last meeting is about the granularity of TTT operation for a candidate cell, whether the TTT is maintained per cell or per beam. For L3 measurement event, the TTT timer is maintain per triggered event, i.e.  consider the event to be fulfilled if the event is fulfilled during corresponding TTT. In L1 event triggered MR, similar principle can be followed. That is, the TTT is maintained per triggered event, and the event is considered to be fulfilled if the event is fulfilled during corresponding TTT.

Regarding the configuration granularity, we understand per event configuration TTT config is sufficient.

TTT is maintained per triggered event.

TTT is configured per event configuration.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	MR MAC CE can include the measurement result associated to the current beam regardless the current SpCell is configured as an SpCell of an LTM candidate configuration or not.

Proposal 2	Except the triggered beam, the measurement report can contain the measurement results of other candidate beams.

Proposal 3	If the QCL RS in the indicated TCI state is CSI-RS and the measurement RS of candidate beam associated with the event configuration is CSI-RS, UE uses the QCL RS in the indicated TCI state for event evaluation.

Proposal 4	If the QCL RS in the indicated TCI state is CSI-RS while the measurement RS of candidate beam associated with the event configuration is SSB, UE uses SSB which is QCLed with the QCL RS in the indicated TCI state for event evaluation.

Proposal 5	If the QCL RS in the indicated TCI state is TRS while the measurement RS of candidate beam associated with the event configuration is SSB, UE uses SSB which is QCLed with the indicated TCI state for event evaluation.

Proposal 6	If the QCL RS in the indicated TCI state is TRS while the measurement RS of candidate beam associated with the event configuration is CSI-RS, UE uses SSB which is QCLed with the QCL RS in the indicated TCI state and the SSB which is QCLed with candidate beam for event evaluation.

Proposal 7	For P3-P6, RAN2 further study whether to rely on NW indication to guarantee same RS type is used for both serving beam and candidate beam or we capture the UE behaviour explicitly.

Proposal 8	Send LS to RAN1 to check the feasibility of the proposals above.

Proposal 9	TTT is maintained per triggered event.

Proposal 10	TTT is configured per event configuration.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx