3GPP TSG-RAN WG2 Meeting #128	 R2-2409902

Orlando, USA, Nov 18-22, 2024

Agenda Item:	8.4.2

Source: 	NEC

Title:  	LP-WUS in RRC_IDLE INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#127b meeting, the following agreements are made:

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements.

For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion.

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

RAN2 inform this conclusion to SA2/CT1/RAN3. This LS is approved in R2-2409225

In this contribution, we further discuss the remaining issue for LP-WUS applied in RRC_IDLE/INACTIVE.

- 2 Discussion

- 2.1 Configuration aspects

There could be some cases to also restrict UE to perform LP-WUS monitoring besides entry/exit condition, e.g., buffered UL transmission, other feature (e.g., SDT/MBS) is ongoing during RRC_IDLE/INACTIVE, or latency requirement (due to long MR transition duration), etc. If these exceptional cases are operating by the UE, the UE should not be allowed to enter LP-WUS monitoring mode or not allowed to evaluate LP-WUS monitoring entry/exit condition. In a word, entry/exit condition could be the general one, but more exceptional cases to restrict UE entering LP-WUS monitoring mode can be FFS.

Proposal-1: In addition to LP-WUS monitoring entry/exit condition, some exceptional cases which may impact on whether the UE can enter LP-WUS monitoring mode should be considered.

Even though the system information is promising to provide LP-WUS configuration including entry/exit condition, but one question is that whether a unified entry/exit condition broadcasted by system information is suitable for all UEs in one cell considering different UE may have different LR capability, therefore UE-specific entry/exit condition of LP-WUS monitoring by dedicated RRC signalling can also be considered as an optional feature, here, since this is intended for RRC_IDLE/INACTIVE UE, RRCRelease is more suitable.

Furthermore, if the LP-WUS configuration is only provided by system information, it means all UEs who support LP-WUS feature may use LP-WUS monitoring mode, which means there is no explicit indication to guide UE behaviour. One enhancement is that based on NW implementation, RRCRelease is used to explicitly indicate which UE could use LP-WUS monitoring mode. In this way, NW can coordinate UE between LP-WUS and paging monitoring so that avoid large amount of UEs monitoring LP-WUS which could reduce power saving gain.

Proposal-2: support dedicated RRC signalling (i.e., RRCRelease) to provide the UE-specific LP-WUS configuration, e.g., the UE-specific entry/exit condition of LP-WUS monitoring.

Proposal-3: support dedicated RRC signalling (i.e., RRCRelease) to indicate whether the UE can perform LP-WUS monitoring or not.

- 2.2 Subgrouping

Last meeting, RAN2 has discussed how to make sure that the UEs belonging to the same LP-WUS/PEI subgroup are assigned to different PEI/LP-WUS subgroup regarding UE_ID based subgrouping. And several companies proposed a “new/similar” subgrouping calculation formula as the following [1]:

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

When the UE choose to monitor both LP-WUS and PEI based on implementation, by adding a new parameter related to PEI subgroup number, the UE belonging to the same PEI subgroup could be separately assigned to different LP-WUS subgroup, which could further reduce the paging false alarm. In our understanding, this is a way forward for false alarm issue and can be supported. However some details should be noted that, this formula is to link the PEI parameter with LP-WUS subgrouping calculation, but the UE is not mandatory to always monitor both PEI and LP-WUS. For some UEs who is not using PEI, question is that whether these UEs still need to use Np to calculate its LP-WUS subgroup ID. Have said this, there is no beneficial to differentiate PEI monitoring UE and non-PEI monitoring UE, applying the same LP-WUS calculation formula is enough, otherwise mismatch between UE and NW could happen. But a NOTE is needed for the explanation of Np, for example, to say even though the LP-WUS monitoring UE chooses to not monitor PEI, the UE still need to consider Np when calculating UE_ID based LP-WUS subgroup ID. We think this NOTE is needed to avoid ambiguity, otherwise some misunderstanding could be caused for non-PEI monitoring UE.

Proposal-4: for UE_ID based LP-WUS subgrouping, if the new/similar formula as shown below is adopted, some clarification/NOTE is needed:

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured by network; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

NOTE: even though the LP-WUS monitoring UE chooses to not monitor PEI, the UE still need to consider Np (if configured) when calculating UE_ID based LP-WUS subgroup ID.

Furthermore we believe that paging false alarm issue could happen in not only UE_ID based subgrouping, but also CN based subgrouping and mix of CN and UE_ID based subgrouping. For purely CN based subgrouping (i.e., CN based subgrouping is used for both LP-WUS and PEI), it could be easily up to CN implementation. But for mix of CN and UE_ID based subgrouping, Question is that how CN is aware of UE-ID based subgrouping information, for example, if UE-1 and UE-2 are using UE-ID based subgrouping for PEI, when CN assign LP-WUS subgroup ID for UE-1 and UE-2, does CN know UE-1 and UE-2 are assigned for the same LP-WUS subgroup or not? It could be FFS.

Proposal-5: If both CN based and UE-ID based subgrouping are used simultaneously, FFS how CN is aware of UE-ID based subgrouping information.

- 2.3 LS from RAN1

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE.

When we look at the historical RAN1 discussion, the issue is raised about whether LR and MR can camp on different carrier/band. From RAN2 perspective, since RRC_IDLE/INACTIVE UE need to camp on initial downlink BWP, which means the MR better to work on this BWP, therefore, it is promising that LR should also work on this same BWP. In fact, if different carrier/band between LR and MR are assumed, it would bring big impact on what we are discussing currently, for example, we are discussing LP-WUS monitoring entry/exit condition, LP-WUS specific measurement relaxation/offloading, if LR is camping on different carrier, the LR result somehow cannot be used for determining some operations related to MR. Therefore, at least in this release, it is recommended to only support that LR and MR work on the same carrier/band during RRC_IDLE/INACTIVE, but the final decision could be determined by other WGs.

Proposal-6: from RAN2 perspective, it is recommended to only support that LR and MR work on the same carrier/band for RRC_IDLE/INACTIVE at least in Rel-19, but the final decision is up to other WGs.

- 2.4 SI/PWS reception

It has been agreed that “From RAN2 perspective, no new procedure is introduced for SI reception/updates”, which means if there is short message coming, the NW would firstly use LP-WUS to trigger UE to wake up, then the UE receive paging short message as legacy way. Basically this is same mechanism as PEI solution, but there is some difference, i.e., now we need to consider that MR and LR transition is involved.

Based on the current agreement, if the UE receive LP-WUS indication and turn on its MR, realising this paging is for short message (e.g., SI update or PWS reception). What is UE behaviour after SI/PWS reception? For normal UE, the UE will keep monitoring PEI/PO just by using MR (i.e., no MR transition is involved), but for LP-WUS capable UE, if there is no unicast paging, after reception of SI/PWS, keeping MR turned on is unnecessary, the LP-WUS monitoring UE should go back to LP-WUS monitoring mode again for power saving reason, i.e., turn off MR and keep using LR to monitoring LP-WUS.

Proposal-7: After SI acquisition / PWS reception during MR-ON, the LP-WUS monitoring UE should go back to LP-WUS monitoring mode, i.e., turn off MR and keep using LR to monitoring LP-WUS.

- 2.5 Mobility

Regarding mobility (i.e., cell reselection), since measurement results for both serving cell and neighbour cell can be derived during LP-WUS monitoring mode, it is expected that cell reselection can be supported for a LP-WUS capable UE during LP-WUS monitoring mode. And one of the reasons why mobility during LP-WUS monitoring mode should be supported is that we are expecting the UE could monitor LP-WUS as much as possible among cells as long as there is no paging/data/signal arrival even though the cell has already been changed, staying LP-WUS monitoring mode as much as possible is beneficial for UE power saving.

Fig.1 Cell reselection for LP-WUS capable UE

One example for intra frequency is that, in legacy the UE perform cell reselection based on the best cell. However if the best cell is not supporting LP-WUS feature meanwhile some other cells which support LP-WUS feature are also good enough for camping, by legacy way, the LP-WUS capable UE need to exist from LPWUS monitoring mode and camp on the best cell, which is not friendly for power saving. Therefore some solutions could be considered to let UE perform cell reselection to select one cell which supports LP-WUS feature as much as possible. For example, neighbour cell information can be provided by serving cell, which can inform UE of the information about which cell is providing LP-WUS configuration, i.e., LP-WUS supporting cell. Then the UE can, based on the neighbour cell information, optimize its cell reselection decision, of which details is FFS.

Proposal-8: RAN2 is suggested to consider optimization on cell reselection, e.g., introduce neighbour cell information to facilitate LP-WUS monitoring UE to camp on LP-WUS supporting cell as much as possible.

- 3 Conclusions

Proposal-1: In addition to LP-WUS monitoring entry/exit condition, some exceptional cases which may impact on whether the UE can enter LP-WUS monitoring mode should be considered.

Proposal-2: support dedicated RRC signalling (i.e., RRCRelease) to provide the UE-specific LP-WUS configuration, e.g., the UE-specific entry/exit condition of LP-WUS monitoring.

Proposal-3: support dedicated RRC signalling (i.e., RRCRelease) to indicate whether the UE can perform LP-WUS monitoring or not.

Proposal-4: for UE_ID based LP-WUS subgrouping, if the new/similar formula as shown below is adopted, some clarification/NOTE is needed:

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured by network; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

NOTE: even though the LP-WUS monitoring UE chooses to not monitor PEI, the UE still need to consider Np (if configured) when calculating UE_ID based LP-WUS subgroup ID.

Proposal-5: If both CN based and UE-ID based subgrouping are used simultaneously, FFS how CN is aware of UE-ID based subgrouping information.

Proposal-6: from RAN2 perspective, it is recommended to only support that LR and MR work on the same carrier/band for RRC_IDLE/INACTIVE at least in Rel-19, but the final decision is up to other WGs.

Proposal-7: After SI acquisition / PWS reception during MR-ON, the LP-WUS monitoring UE should go back to LP-WUS monitoring mode, i.e., turn off MR and keep using LR to monitoring LP-WUS.

Proposal-8: RAN2 is suggested to consider optimization on cell reselection, e.g., introduce neighbour cell information to facilitate LP-WUS monitoring UE to camp on LP-WUS supporting cell as much as possible.

- 4 References

[1] R2-2408114	Vivo		RAN2#127b