3GPP TSG-RAN WG2 Meeting #128	R2-2409937

Orlando, US, Nov. 18th – Nov. 22th, 2024

Source: 	NTU

Title:  	Further considerations on retransmission and finite buffer for DSA

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting, the following agreements are made:

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this proposal, we further discuss the performance and implementation consideration of DSA, particularly focusing on retransmission mechanisms and finite buffer issues.

Retransmission and finite buffer considerations

During RAN #127bis meeting, RAN2 decided to introduce DSA to CB-Msg3 transmission. To ensure a quality of service (e.g. reliability of 0.9 to 0.99), retransmission scheme is necessary. However, when the UE is executing retransmission and backoff, it is possible that new packet arrives and UE cannot deal with it immediately. In real case, these newly arriving packets will be placed into a finite buffer and wait for the previous transmission to end. If the buffer is full, these new packets will be blocked from the buffer and dropped directly, leading to packet loss.

Observation 1: When a UE executes backoff procedures, any newly arriving packets are placed into a finite buffer. If the buffer is full, the new packets will be blocked, leading to packet losses.

To ensure quality of service, it is unavoidable to implement retransmission despite higher delay. In satellite communication scenarios, the value of delay is large due to large RTT (round trip time) and can be several seconds. [1] In high traffic load case, new packets arrive frequently and the handling time for a single packet is long, which causes the system overload and large number of packets are blocked. In this case, increasing retransmission count cannot enhance PLR (packet loss ratio) and system throughput anymore, because most of packet loss is due to blocking rather than transmission failure.

To analyse the system performance with retransmission and finite buffer, we need to first discuss when UEs can transmit packets. For each packet, UE needs an EDT occasion and an RTT (round trip time) for contention-based random access and waiting for the satellite feedback. Then, UE should wait for the next EDT occasion start for the next attempt. Thus, after a transmission attempt, the UE needs to wait at least 3 EDT occasion period for a new attempt.

Figure 1:Transmission delay on different cases

Figure 1 shows three different cases of transmission delay: no retransmission, retransmission without random backoff and retransmission with random backoff. Each retransmission without random backoff costs 3-EDT occasion period delay, and the delay will be higher if the random backoff mechanism comes into account. Assuming random backoff is not applied, the delay experienced by a successfully delivered message can be approximated as:

Dn= nTEDT +n-12TEDT

where TEDT is the EDT occasion period and n denotes the number of transmission attempt.

Observation 2: Each retransmission attempt costs at least 3 EDT occasion period delays and may be longer with random backoff mechanism.

Simulation result and discussion

In the remainder, we simulate the SA and DSA system with retransmission and finite buffer. The parameters considered in the study are reported in Table 1.

| Parameter | Value |
| --- | --- |
| Number of EDT resources (subcarriers) per occasion, Nsc | 48 |
| Number of EDT occasions, N | 4 |
| Duration of an EDT occation, Tedt | 96 ms |
| Replicas for DSA | 2 and 3 |
| Round trip time, RTT | 510 ms |
| Transmission error probability | 0.01 |
| Buffer size | 10 packets |
| Maximum retransmission number | 5 |
| Random backoff value | 3 |

Table 1: Simulation setup

In our simulation setup, we consider a retransmission scenario with an upper limit, along with a random backoff mechanism. A finite buffer is considered in the simulation.

First, we compare the performance of using SA and DSA with 2 or 3 replicas. (N represents the number of replicas. When N = 1, it implies that slotted ALOHA is adopted.)

Figure 2: SA and DSA performance comparison

In Figure 2, we observe that retransmission can reduce the packet loss rate (PLR) to nearly zero at low traffic loads. However, once the traffic load exceeds a threshold—approximately 17 kbps—the PLR sharply increases beyond acceptable levels. This rise is accompanied by a significant increase in delay over a similar range. These results suggest that when traffic load surpasses a critical threshold, the system becomes overloaded and is unable to handle the heavy demand. In this situation, most UE continuously attempts retransmission, resulting in frequent packet blockage, high delays, and high PLR.

In Figure 3, as the buffer size increases, the PLR remains low under slightly higher traffic loads. However, it still rises sharply once it reaches a certain threshold. The average delay does not significantly vary with buffer size, except in the no-buffer case. In this case, the average delay is lower because most packets are blocked rather than transmitted when the traffic load is high.

Figure 3: Performance comparison of different buffer size

There are two main reasons leading to packet loss: blockage and transmission failure. In Figure 4, we observe that when traffic load exceeds 15 kbps, most packet loss results from blockage rather than transmission failure. Although retransmission mechanism can help reduce transmission failure, it cannot mitigate blockage loss. In fact, it introduces higher delays and leads to additional blockage loss. Thus, retransmissions cannot bring the PLR within an acceptable range under high traffic load conditions.

Figure 4: the percentage of blockage loss within total packet loss

Observation 3: When traffic load surpasses a certain threshold, most packet loss results from blockage rather than transmission failures. In this situation, increasing the retransmission count does not improve the PLR.

Proposal 1: RAN2 should focus on low-traffic load scenarios, where packet loss due to blockage is minimal and retransmissions can effectively enhance PLR to an acceptable reliability requirement.

Figure 5: Performance comparison of different retransmission count and buffer size

In Figure 5, we compare the effects of the maximum allowed number of retransmissions (k) with different buffer size. We observe that while increasing k can reduce the PLR, improvements become minimal once k exceeds a certain threshold. This threshold depends on the buffer size. Larger buffers allow more effective retransmission attempts. Consequently, an upper limit for retransmission attempts can be set based on the buffer size. If this limit is reached, the system may switch to alternative uplink schemes (e.g., 4-step random access or other methods).

Observation 4: When the maximum allowed number of retransmissions exceeds a certain threshold, it does not lead to further enhancements in PLR. The threshold varies depending on the buffer size.

Proposal 2: Set the maximum number of retransmissions based on buffer size.

When implementing retransmissions, a random backoff mechanism is commonly used to reduce collisions during subsequent attempts. In general random backoff schemes, the UE randomly selects a backoff time before the next attempt. However, in CB-Msg3 EDT transmission, the UE can initiate contention-based uplink transmission only within each EDT period. Therefore, the UE selects a random number of EDT occasions from a limited range for backoff. This number must remain relatively small, as each EDT period lasts approximately 0.4 seconds; waiting through too many EDT periods would result in significant delays.

In the following analysis, we compare performance under varying traffic loads with different numbers of replicas and maximum random backoff values.

Figure 6: Performance comparison of different replica number and random backoff values

In figure 6, we compare DSA performance with different numbers of replicas (2 or 3) and varying random backoff values r (0, 1, 2). When r = 0, the scheme becomes 1-persistent, with all UEs attempting transmission at the next EDT occasion without any additional backoff.

The simulation results indicate three distinct traffic load intervals, each showing different performance characteristics.

- Low Traffic Load (below 15 kbps): At this load, nearly all packets are successfully transmitted with retransmission. In this interval, more replicas and fewer EDT periods for backoff perform better, resulting in lower delay.

- Medium Traffic Load (15-25 kbps): Here, both the PLR and delay increase sharply. In this range, extending backoff periods helps to reduce collisions during retransmission attempts, which improves both PLR and delay. Fewer replicas further enhance system performance in this interval by minimizing collision.

- High Traffic Load (above 25 kbps): At this load, no configuration can achieve a PLR below 0.1, as the system becomes overloaded and unstable. In this case, fewer replicas and shorter backoff periods perform best, as they help reduce delay and, consequently, the blocking probability—identified as the main contributor to PLR at this load.

Combining these observations with previous findings (indicating that the system begins to destabilize around a traffic load of 17 kbps), we conclude that system stability has a critical influence on performance across different traffic loads.

Observation 5: Under varying traffic load conditions (low, medium, high), different configurations of replica count and random backoff values yield optimal performance in terms of PLR and delay.

Proposal 3: Adjust the number of replicas and random backoff values according to traffic load conditions (low, medium, or high) to enhance system performance.

Conclusions

This contribution for Rel-19 IoT NTN gives the following observations and proposals.

Observation 1: When a UE executes backoff procedures, any newly arriving packets are placed into a finite buffer. If the buffer is full, new packets will be blocked, leading to packet losses.

Observation 2: Each retransmission attempt costs at least 3 EDT occasion period delays and may be longer with random backoff mechanism.

Observation 3: When traffic load surpasses a certain threshold, most packet loss results from blockage rather than transmission failures. In this situation, increasing the retransmission count does not improve the PLR.

Observation 4: When the maximum allowed number of retransmissions exceeds a certain threshold, it does not lead to further enhancements in PLR. The threshold varies depending on the buffer size.

Observation 5: Under varying traffic load conditions (low, medium, high), different configurations of replica count and random backoff values yield optimal performance in terms of PLR and delay.

Proposal 1: RAN2 should focus on low-traffic load scenarios, where packet loss due to blockage is minimal and retransmissions can effectively enhance PLR to an acceptable reliability requirement.

Proposal 2: Set the maximum number of retransmissions based on buffer size.

Proposal 3: Adjust the number of replicas and random backoff values according to traffic load conditions (low, medium, or high) to enhance system performance.

Reference

[1] R2-2408547 “Repetitions and Delay Considerations about SA and DSA”

[2] Chairman_notes of 3GPP TSG RAN WG2 meeting #127bis