3GPP TSG-RAN WG2 Meeting #128	R2-2409959

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.7.6

Source:	Apple

Title:	Views on MAC Signalling for XR Rate Control

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

During RAN #105, the following new objective has been approved for Rel-19 WI on XR for NR Phase 3 [1]:

-	Specify uplink congestion signalling [RAN2]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In RAN2 #127bis, we have started the discussions on this topic, and we have reached the following agreements [2]:

FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - - Extend the Bit Rate field - - Define a new bit rate table to provide sufficient granularity for XR traffic - - Introduce new values for the bitRateMultiplier

This paper presents our views about XR rate control.

# Discussions

## Recommended Bit Rate Indication Granularity

The XR rate control signalling allows the Application to adjust the source coding scheme based on the instantaneous RAN status (e.g. channel quality, network loading etc), in order to maintain the operation without severely impacting user experience. The objective of this WI focusses on DL signalling, which targets to enable adaptation of Application at the UE side.

XR services may comprise multiple different traffic flows, including video, audio, and haptic feedback. It is possible that some of these traffic flows are mapped to the same DRB (and hence the same LCH) due to their similar QoS requirements, but the recommended bit rates for these traffic flows may be different. Consequently, we may end up increasing or decreasing the data rate for some of the traffic flows inappropriately, as the existing Recommended Bit Rate MAC CE is unable to differentiate multiple QoS flows mapped to this LCH. Furthermore, since such rate recommendation mechanism is supposed to complement L4S which is also based on QoS flow, we think indication per QoS flow can enable better inter-working with L4S.

Proposal 1: For the indication of XR rate control, the granularity based on QoS flow is more suitable (e.g. for inter-working with L4S).

## Flexible Indication of Bit Rate Multiplier

Regardless of the indication granularity is per DRB or per QoS Flow, we think it is helpful if the rate multiplier can be signalled in a more flexible manner. Currently, for the Recommended Bit Rate MAC CE, a bitRateMultiplier value chosen from the set {x40, x70, x100, x200} is configured for a logical channel. Then, the indicated rate for the concerned logical channel should be multiplied by the configured bitRateMultiplier value to obtain the actual bit rate recommendation. Due to the dynamic nature of XR applications and network congestion status, we think the multiplier value could be dynamically signalled in the MAC CE, rather than fixed to a pre-configured value.

In particular, each logical channel or QoS flow may be configured with a specific set of possible rate multiplier values, and the MAC CE can be used to indicate one particular multiplier value out of this configured set. This naturally provides a flexible framework to cope with bit rate recommendation in dynamically changing environments.

Proposal 2: RAN2 can consider the direction based on dynamic indication of Bit Rate Multiplier in the MAC CE.

## Indication of Desirable and Tolerable Bit Rate in Query Message

In the current framework for Recommended Bit Rate, the UE can send a Query message to indicate its desirable rate. The network may take the rate desired by the UE into account when deciding on the recommendation. Since it is crucial for XR Applications to maintain the user experience at an acceptable level, we think the UE can provide more comprehensive information in the Query message to include both “Desired Rate” and “Tolerable Rate”, where the Tolerable Rate is the minimum rate that currently the UE can accept for a particular QoS flow or logical channel to sustain the user experience. Thus, ideally the bit rate should be within the range between the indicated Desired Rate and Tolerable Rate.

Proposal 3: RAN2 can consider the direction of including both Desirable Rate and Tolerable Rate in the Query Message.

# Conclusions

In this paper, we presented our views on XR rate control signalling with the following proposals.

Proposal 1: For the indication of XR rate control, the granularity based on QoS flow is more suitable (e.g. for inter-working with L4S).

Proposal 2: RAN2 can consider the direction based on dynamic indication of Bit Rate Multiplier in the MAC CE.

Proposal 3: RAN2 can consider the direction of including both Desirable Rate and Tolerable Rate in the Query Message.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.