3GPP TSG-RAN WG2 Meeting #128	R2-2410035

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda Item:	8.6.2

Source: 	Fujitsu

Title:	Discussion on inter-CU LTM in non-DC and DC cases

Document for:	Discussion and Decision

- Introduction

This document discusses some remaining issues for inter-CU LTM in non-DC and DC case.

- Discussions

2.1	Inter-CU LTM in non-DC case

Security handling for inter-CU LTM in non-DC case

In the last SA3 meeting, SA3 sent a reply LS to RAN2 on security handling for inter-CU LTM in non-DC cases [1]. In the reply LS, SA3 kindly suggested as follows:

The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure.

Observation 1: SA3 prefers to reconfigure NCC value via RRC signalling after every inter-CU LTM cell switch execution that changes the PDCP anchor.

In the LS, SA3 also kindly provided their view as follows:

SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase.

Observation 2: If RAN2 sticks to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures, SA3 will be able to analyse other methods which may be unsecure or may give bigger impact to the core network.

RAN2 needs to discuss whether RAN2 accept the SA3 solution which is preferred by most companies in SA3 or ask SA3 to analyse further method.

If RAN2 adopts the above SA3 solution, the subsequent LTM definition in Rel-18 cannot apply to Rel-19 LTM because the term “subsequent” LTM is used for the case when cell switch between L1/L2 mobility candidates is done without RRC reconfiguration in between [2]. Also, after inter-CU LTM cell switch is executed, the network has to delay sending the next LTM CSC MAC CE until the UE completes to update the NCC value received via RRC signalling. On the other hand, if RAN2 does not accept the solution, SA3 needs to analyse other method and make the conclusion only in the next February meeting. If SA3 fails to reach the consensus by next March and delays their conclusion, RAN2 progress will be affected.

Observation 3: Based on the SA3 reply LS, the following points should be taken into consideration:
- If SA3’s preferred method is adopted, the subsequent LTM definition in Rel-18 cannot apply to Rel-19 LTM.
- If SA3’s preferred method is not accepted, SA3 may not be able to finalize their analysis for further methods by next March.

Proposal 1: RAN2 to discuss whether to accept the SA3 solution which is preferred by most companies in SA3 or ask SA3 to analyse further method.

2.2	Inter-CU LTM in DC case

Bearer type change in DC case

In Rel-18 intra-CU LTM, as security key change is not supported, PDCP re-establishment is not supported. Therefore, bearer type change between MN terminated bearer and SN terminated bearer at LTM cell switch execution is not supported implicitly as well. In Rel-19 intra-CU or inter-CU LTM, as security key change will be supported, bearer type change between MN terminated bearer and SN terminated bearer at LTM cell switch can be supported. Also, Rel-18 SCPAC already supports all cases of bearer type change at CPAC execution, and the procedure is already captured in the Rel-18 specifications. If Rel-19 intra-CU or inter-CU LTM supports all cases of bearer type change at LTM cell switch, Rel-18 SCPAC procedure will be able to be a baseline and specification impact will not be large. We propose that all cases of bearer type change, including MN-terminated and SN-terminated change, should be supported in Rel-19.

Proposal 2: All cases of bearer type change, including MN-terminated and SN-terminated change, at LTM cell switch execution should be supported in Rel-19.

In RAN2#127bis meeting, RAN2 tried to discuss whether the SCG addition is supported when an inter-CU MCG LTM cell switch is executed, but it was to be postponed.

If Proposal 2 is agreeable, for example, the following case requires SCG addition at LTM cell switch execution:
- In the source configuration, both MN-terminated and SN-terminated bearers are configured and all associated RLC bearers are mapped to MCG.
- In the target configuration, both MN-terminated and SN-terminated bearers are configured and associated RLC bearers are mapped to MCG or SCG or both.

Observation 4: If Proposal 2 is agreeable, there is a case SCG addition is required at LTM cell switch execution regardless of intra-CU or inter-CU cell switch.

Proposal 3: The SCG addition is supported when an LTM cell switch is executed.

Rel-19 Set ID and ltm-NoResetID for DC

In RAN2#127bis meeting, it was agreed [3]:

For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18

However, the agreement for the procedure highlighted in yellow was only for non-DC case. The consensus was not reached for DC case because it was unclear whether the procedure really can work with SN-terminated bearer or bearer type change cases. Consequently, the discussion was postponed.

In Rel-18 SCPAC, the radio bearer handling at CPAC execution including SN change and MN-terminated/SN-terminated bearer change has been specified in the sub clause 5.3.5.13.8 of TS 38.331. The overall UE procedure is as follows:

(1) If inter-SN CPAC is executed, i.e., servingSecurityCellSetId and securityCellSetId in the target configuration are different, perform security key update.

(2) For the DRBs associated keyToUse in the target configuration is different from associated keyToUse in the source configuration, or 
    for the radio bearers associated with secondary key in the target configuration and security key update is performed,
    configure PDCP entity of the radio bearers with the appropriate security keys and trigger PDCP and RLC re-establishment.

(3) For the radio bearers which do not meet the condition in (2) in the target configuration,
      if there is an associated SCG RLC bearer, re-establish the SCG RLC entity (regardless of inter-DU or not),
      for the AM DRBs to which associated RLC entity is re-established or removed, trigger PDCP data recovery

From the overall procedure for Rel-18 SCPAC, the following methods can be reused for Rel-19 inter-CU LTM:
- Rel-19 set ID, which is equivalent to servingSecurityCellSetID and securityCellSetID, is used to determine intra-CU LTM or inter-CU LTM.
- The keyToUse is used to determine PDCP termination point, i.e., MN-terminated bearer or SN-terminated bearer.

Observation 5: From Rel-18 SCPAC procedure, the following method can be reused for Rel-19 inter-CU LTM:
- Rel-19 set ID, which is equivalent to servingSecurityCellSetID and securityCellSetID, is used to determine intra-CU LTM and inter-CU LTM.
- The keyToUse is used to determine PDCP termination point of the DRB, i.e., MN-terminated bearer or SN-terminated bearer.

We think re-using the Rel-18 SCPAC method in Observation 5 for Rel-19 inter-CU LTM is reasonable and no need to set Rel-19 set ID per radio bearer.

Proposal 4: Rel-19 set ID is used for the UE to determine whether the executed cell switch is intra-CU or inter-CU, and the keyToUse is used for the UE to determine PDCP termination point of the each DRB, i.e., MN-terminated bearer or SN-terminated bearer.

The next discussion is whether the yellow highlighted part of the above agreement, i.e., if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18, is applicable for DC case or not. We think it will not be applicable at least for the case where CU is acting as SN and MN is unchanged.

Assuming that an MN-terminated SCG or split DRB is configured in the UE, and inter-CU SCG LTM cell switch is executed and bearer type of the DRB is unchanged, the security key of the DRB is unchanged because the DRB is associated to the master key and therefore PDCP re-establishment/RLC re-establishment will not be triggered for the DRB. However, RLC re-establishment for the DRB is necessary because DU is changed due to CU change. Therefore, even if the Rel-19 set ID is different, the L2 reset handling in Rel-18 is necessary in this case.

Also, if bearer type change is supported, PDCP re-establishment and RLC re-establishment due to PDCP termination point change at intra-CU LTM cell switch execution should also be considered.

Observation 6 For DC case, only comparing Rel-19 Set IDs and comparing ltm-NoResetIDs are not sufficient to determine appropriate L2 reset operation.

Therefore, we propose that RAN2 should discuss how to perform L2 reset for DC case with considering all bearer types.

Proposal 5: RAN2 to discuss how to perform L2 reset for DC case with considering all bearer types.

- Conclusion

We have the following observations and proposals:

<non-DC Case>

Security handling for inter-CU LTM in non-DC case

Observation 1: SA3 prefers to reconfigure NCC value via RRC signalling after every inter-CU LTM cell switch execution that changes the PDCP anchor.

Observation 2: If RAN2 sticks to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures, SA3 will be able to analyse other methods which may be unsecure or may give bigger impact to the core network.

Observation 3: Based on the SA3 reply LS, the following points should be taken into consideration:
- If SA3’s preferred method is adopted, the subsequent LTM definition in Rel-18 cannot apply to Rel-19 LTM.
- If SA3’s preferred method is not accepted, SA3 may not be able to finalize their analysis for further methods by next March.

Proposal 1: RAN2 to discuss whether to accept the SA3 solution which is preferred by most companies in SA3 or ask SA3 to analyse further method.

<DC Case>

Bearer type change in DC case

Proposal 2: All cases of bearer type change, including MN-terminated and SN-terminated change, at LTM cell switch execution should be supported in Rel-19.

Observation 4: If Proposal 2 is agreeable, there is a case SCG addition is required at LTM cell switch execution regardless of intra-CU or inter-CU cell switch.

Proposal 3: The SCG addition is supported when an LTM cell switch is executed.

Rel-19 Set ID and ltm-NoResetID for DC

Observation 5: From Rel-18 SCPAC procedure, the following method can be reused for Rel-19 inter-CU LTM:
- Rel-19 set ID, which is equivalent to servingSecurityCellSetID and securityCellSetID, is used to determine intra-CU LTM and inter-CU LTM.
- The keyToUse is used to determine PDCP termination point of the DRB, i.e., MN-terminated bearer or SN-terminated bearer.

Proposal 4: Rel-19 set ID is used for the UE to determine whether the executed cell switch is intra-CU or inter-CU, and the keyToUse is used for the UE to determine PDCP termination point of the each DRB, i.e., MN-terminated bearer or SN-terminated bearer.

Observation 6 For DC case, only comparing Rel-19 Set IDs and comparing ltm-NoResetIDs are not sufficient to determine appropriate L2 reset operation.

Proposal 5: RAN2 to discuss how to perform L2 reset for DC case with considering all bearer types.

References

[1] R2-2409534, “Reply LS on security handling for inter-CU LTM in non-DC cases”

[2] R2 119bis-e Chair Notes EOM

[3] R2-2409211, “Report from session on V2X/SL, R18/19 MOB, and R19 NES”