3GPP TSG-RAN WG2 Meeting #128                                    	R2-2410049

Orlando, USA, Nov 18th – 22th, 2024

Agenda item:	8.9.2

Source:	Qualcomm Incorporated

Title:	Discussion on S&F mode operation

Document for:	Discussion and Decision

- Introduction

In RAN2#126 meeting, following agreements were made.

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In RAN2#126 meeting, following agreements were made.

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In this document, we provide further details for solution to be addressed by RAN2.

- Discussion

Switching S&F mode

The cell operating in S&F mode should be able to switch to normal operating mode when feeder link becomes available, in which case, the connection with MME onboard will be torn down and instead the MME on the ground is used. When operating in normal mode, the satellite may include eNB only on board or may switch to transparent payload mode. In this case, it can support any NTN capable UE. Similarly, the cell can also switch back to S&F mode when feeder link is lost and use MME onboard satellite.

RAN2 assume that the cell operating in S&F mode uses MME on board and cell operating in normal mode uses MME on the ground and switching of the operation mode is supported.

Whenever the satellite establishes feeder link connection (feeder link becomes available), it is also possible that a cell may want to exit from the S&F mode operation (for example, switch the S&F mode to transparent architecture mode or only eNB onboard mode). In this case, the UEs supporting S&F mode operation may also support normal mode operation. If cellBarred-NTN-r17 is changed, the existing procedure should follow to update the SIB1.

Whenever the satellite loses feeder link connection, cell supporting S&F mode should make sure that S&F indication is included in SIB. In this case, the cell can release all UEs that do not support S&F mode operation in RRC_CONNECTED to IDLE mode. We think it is very important to avoid paging notification to eMTC and NB-IoT UEs. The existing cell stop time mechanism can be used to inform the legacy UEs that they need to find other cells. This can be useful for both cases, i.e., when feeder link will be lost or when feeder link will be available.

For new UEs, a new indication (probably in SIB19) can be introduced to indicate that the cell stop time is associated with the update of S&F mode operation, i.e., at cell stop time, the cell loses feeder link and operates in S&F mode.

The existing cell stop time is used as switching time between S&F mode and normal mode operation.

For new UEs, additional indication is provided to associate the existing cell stop time with the switching time between S&F mode and normal mode operation.

S&F indication and cell barring

SA2 have identified the following normative impact in [1].

2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

It is possible that the UE may have to be rejected in the first attach or TAU attempt by the MME on-board satellite with a cause, for example, “S&F mode operation” and ask UE to retry with wait timer. This mechanism needs new signaling and may not work for the legacy UEs as legacy UEs may end up retrying the attach or TAU procedure wasting power. Therefore, we think it is simple to bar the legacy NTN UEs from accessing the cell operating in S&F mode, i.e., operating when feeder link is not available.

The cell operating in S&F mode sets the cellBarred-NTN-r17 to “barred”.

Cell reselection

Existing frequency Priorities and cell reselection offsets can be used for a UE to perform measurements for cell reselection. However, the UE could prioritize to select a cell operating in normal mode over a cell operating in S&F mode. This is specially for NB-IoT case where no frequency priorities are considered. This can be used in case equal frequency priority or in case UE meets cell selection criteria for both S&F and normal mode cells. When waiting in S&F mode cell or when camping on S&F mode, it can be left to UE whether it periodically searches for cells operating in normal mode.

When connected to S&F mode cell, it can be left to UE in RRC_IDLE to periodically search for better and normal mode cell (i.e., cells with feeder link connection available).

When in RRC_IDLE and waiting to connect to a S&F mode satellite, it can be left to UE to prioritize the search for better and normal mode cell (i.e., cells with feeder link connection available).

- Conclusion

Following observation and proposals are made:

Proposal 1	RAN2 assume that the cell operating in S&F mode uses MME on board and cell operating in normal mode uses MME on the ground and switching of the operation mode is supported.

Proposal 2	The existing cell stop time is used as switching time between S&F mode and normal mode operation.

Proposal 3	For new UEs, additional indication is provided to associate the existing cell stop time with the switching time between S&F mode and normal mode operation.

Proposal 4	The cell operating in S&F mode sets the cellBarred-NTN-r17 to “barred”.

Proposal 5	When connected to S&F mode cell, it can be left to UE in RRC_IDLE to periodically search for better and normal mode cell (i.e., cells with feeder link connection available).

Proposal 6	When in RRC_IDLE and waiting to connect to a S&F mode satellite, it can be left to UE to prioritize the search for better and normal mode cell (i.e., cells with feeder link connection available).

- References

[1] TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3