3GPP TSG-RAN WG2 #128	R2-2410062

Orlando, USA, 18 – 22 November 2024

Agenda item:		8.6.3	L1 event triggered measurement reporting

Title:		Further details of L1 event triggered measurement reporting

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

On the L1 event triggered measurement, there were a lot of agreements in RAN2#127bis below:

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

- MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

In addition to the agreements above, there are some remaining issues. Among those issues, in this contribution we discuss the details of L1 event evaluation, and the MAC CE contents and then we provide our views.

# 2. Discussion

2.1	L1 event evaluation

As the L1 LTM measurement events, RAN2 agreed to support the following new events:

- Event LTM2: Current beam of serving cell becomes worse than absolute threshold

- Event LTM3: Any beam in candidate RS configuration of candidate cell becomes amount of offset better than current beam of serving cell

- Event LTM4: Any beam in candidate RS configuration of candidate cell becomes better than absolute threshold

- Event LTM5: Current beam of serving cell becomes worse than absolute threshold1 AND Any beam in candidate RS configuration of candidate cell becomes better than another absolute threshold2

In RAN2#127bis, there are two remaining issues: one is whether TTT operation is per beam or per cell. The other is whether TTT is reset or not when changing the current beam. Considering the previous discussions, it seems that these issues should be considered together to make it clear what the expected behaviour is.

For the Event LTM2, it is clear that single beam is used in the event evaluation. The question is whether the TTT is kept running when the current beam is changed while the TTT is already running. There were two different views in the last meeting. On one hand, some companies including us consider that the TTT should be kept running, because the current beam can be changed dynamically but new current beam should be better than the old one. As long as the best or better beam is considered in the evaluation, there would be no big issue to keep running. If the TTT is stopped and restarted upon changing the current beam, the UE may detect the RLF before TTT is expired or the network has to trigger the L3 HO instead, which means the gain of LTM cell switch is reduced. On the other hand, some others consider that if the TTT is kept running, the UE may initiate LTM cell switch prematurely. However, new current beam should be normally better than the old one and thus the better beam is still worse than the threshold. It would imply that it would be better to change a serving cell. So, for the Event LTM2, it seems reasonable to keep the TTT running.

Observation 1: For the Event LTM2, the TTT should not be reset when the current beam of serving cell is changed, as long as the new current beam satisfies the event condition.

For the Event LTM4, it is clear that the event evaluation is per beam of the candidate cell. If two beams are configured in the candidate RS configuration, the UE evaluates the event condition for each beam. For the TTT handling, it is reasonable to handle also the TTT per beam.

Observation 2: For the Event LTM4, the event evaluation is done per beam of candidate cell and the TTT should be also handled per beam of the candidate cell.

For the Event LTM3, the observations for the Event LTM2 and LTM4 can be applied in combination. From serving cell perspective, the current beam is used for the event evaluation, while from candidate cell perspective, each beam is used for event evaluation independently. In other words, the UE should consider a beam pair, i.e. a pair of current beam and candidate beam. For the TTT, the main issue is how to consider change of the current beam. From serving cell perspective, it is similar to the Event LTM2 and thus the TTT should be kept. On top of that, the TTT handling is applied per candidate beam.

Observation 3: For the Event LTM3, the TTT should not be reset when the current beam of serving cell is changed, as long as the new current beam satisfies the event condition. This TTT handling is applied per beam pair, i.e. a pair of current beam and each beam of the candidate cell.

For the Event LTM5, even though the conditions are different, the same approach as the Event LTM3 can be applied. For the serving cell evaluation, if the current beam is changed but new current beam satisfies the condition and the candidate beam(s) also still satisfies the condition, the TTT should be kept running. Otherwise, the TTT should be reset.

Observation 4: For the Event LTM5, same approach as the Event LTM3 can be applied.

Proposal 1: RAN2 to agree with the following approaches in the LTM event evaluation:

- For the Event LTM2, the TTT is not reset when the current beam of serving cell is changed, as long as the new current beam satisfies the event condition.

- For the Event LTM3/5, the TTT is not reset when the current beam of serving cell is changed, as long as the new current beam satisfies the event condition. This TTT handling is applied per beam pair, i.e. a pair of current beam and each beam of the candidate cell.

- For the Event LTM4, the event evaluation is done per beam of candidate cell and the TTT is also handled per beam of the candidate cell.

2.2	Serving cell beams in MR MAC CE

In RAN2#127bis, there were some agreements on the contents of new MAC CE for L1 event triggered measurement reports. One of them is that MR MAC CE can include up to N beams, where it is FFS on whether the N-1 beams should satisfy the event or not. Our understanding is that reporting the serving cell beams are under condition that the network includes the serving cell in the candidate cell configuration and the UE should perform the L1 measurements accordingly. There is no additional UE effort for inclusion of N beams.

The intention of including N-1 beams is to get more picture on the serving cell quality by the network side. To achieve this intention, there seems to be no strong need for other N-1 beams to satisfy the L1 measurement event.

Proposal 2: RAN2 to confirm that the UE reports (up to) N beams of serving cell in the MR MAC CE when the network configures the serving cell in the candidate cell configuration.

Proposal 3: RAN2 to agree that N-1 beams of the serving cell does not need to satisfy the event, when those are included in the MR MAC CE.

# 3. Conclusion

In this contribution we discussed the details of L1 event evaluation and the MAC CE contents. Then we made the following observations and proposals.

L1 event evaluation

Observation 1: For the Event LTM2, the TTT should not be reset when the current beam of serving cell is changed, as long as the new current beam satisfies the event condition.

Observation 2: For the Event LTM4, the event evaluation is done per beam of candidate cell and the TTT should be also handled per beam of the candidate cell.

Observation 3: For the Event LTM3, the TTT should not be reset when the current beam of serving cell is changed, as long as the new current beam satisfies the event condition. This TTT handling is applied per beam pair, i.e. a pair of current beam and each beam of the candidate cell.

Observation 4: For the Event LTM5, same approach as the Event LTM3 can be applied.

Proposal 1: RAN2 to agree with the following approaches in the LTM event evaluation:

- For the Event LTM2, the TTT is not reset when the current beam of serving cell is changed, as long as the new current beam satisfies the event condition.

- For the Event LTM3/5, the TTT is not reset when the current beam of serving cell is changed, as long as the new current beam satisfies the event condition. This TTT handling is applied per beam pair, i.e. a pair of current beam and each beam of the candidate cell.

- For the Event LTM4, the event evaluation is done per beam of candidate cell and the TTT is also handled per beam of the candidate cell.

Serving cell beams in MR MAC CE

Proposal 2: RAN2 to confirm that the UE reports (up to) N beams of serving cell in the MR MAC CE when the network configures the serving cell in the candidate cell configuration.

Proposal 3: RAN2 to agree that N-1 beams of the serving cell does not need to satisfy the event, when those are included in the MR MAC CE.

# References

[1] R2-2409211, Report from session on V2X/SL, R18/19 MOB, and R19 NES, Vice Chairman (Samsung)