3GPP TSG-RAN WG2 Meeting #128	R2-2410098

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.2.5

Source: 	China Telecom

Title:	Discussion on Topology 2 related aspects for Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, Topology 2 related aspects for Ambient IoT were discussed and the following agreements were reached.

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

In this contribution, we would like to share our opinions on the remaining issues on Topology 2 related aspects for Ambient IoT.

- Discussion

Validity of radio resource for A-IoT

In last RAN2 meeting, it was agreed that the UE reader may perform the A-IoT procedure on A-IoT interface when the resource configuration is valid in the cell under network control. However, there remains FFS on how to determine the validity of A-IoT radio resource and whether to allow resource validity across multiple cells.

To allow resource validity across multiple cells, the network need to support interaction or coordination among multiple gNBs or cells, which will bring extra signalling overhead and complexity. The serving cell or gNB has to coordinate with related cells or gNBs before providing the A-IoT radio resource to the UE reader. Moreover, the A-IoT resources shall be reserved or preconfigured in the related cells, which will reduce the frequency efficiency. Thus, we prefer not to support resource validity across multiple cells.

Proposal 1: For A-IoT Topology 2, the validity of A-IoT radio resource is within the cell providing the resource configuration. No need to support resource validity across multiple cells.

In the network side, it is no doubt that the resource validity could be under network control. Regarding how to determine the validity of A-IoT radio resource in the UE side, the most straightforward way is to follow the configuration from the network. The UE determines the resource as valid after receiving the configuration of resource from the network. If the network indicates the UE to release or reconfigure the A-IoT resource, the UE can treat the previous resource as invalid. In last RAN2 meeting, timer based solution for determining the resource validity was discussed with no consensus reached. For timer based solution, potential benefit is to save signalling overhead since the UE can determine the resource validity by maintaining a single or multiple timers. However, it may need further discussion on the start, restart and stop conditions for the timer, the UE behaviour upon expiry of the timer, the candidate values for the timer and so on. Moreover, even if the timer based solution is introduced, it is very likely the network could send explicit indication to release or reconfigure the A-IoT resource when needed. In this case, the signalling overhead could not be reduced. Thus, the actual gain of timer based solution is limited.

Proposal 2: For A-IoT Topology 2, the UE determines the validity of A-IoT radio resource according to the configuration or indication from the network. No need to introduce the timer based solution.

Proposal 3: The UE determines the resource as valid after receiving the configuration of resource from the network. If the network indicates the UE to release or reconfigure the A-IoT resource, the UE can treat the previous resource as invalid.

UE reader “temporarily” out of connection

In previous RAN2 meetings, the issue on UE reader “temporarily” out of connection was discussed. RAN2 will study the potential cases such as RLF and HO, etc.. Considering the commercial use cases and the network deployment for A-IoT, the intermediate UE might be relatively stationary or moving slowly in a small area. The possibility of HO is relatively low. To have a seamless solution for the ongoing A-IoT service during HO is too complex and will increase the cost of the UE reader. Moreover, it is quite likely that the potential HO of the UE reader is under the control of the gNB, since there is no clear need to support the coexistence of A-IoT and UE triggered HO such as CHO. Thus, a simple way might be that the UE reader suspends the A-IoT service during the HO. After HO to the target cell, the UE reader can just follow the configuration of the network to resume/re-initiate/stop the A-IoT service.

Proposal 4: For HO of the UE reader, the UE reader suspends the ongoing A-IoT service during the HO procedure.  After HO to the target cell, the UE reader follows the configuration of the network to resume/re-initiate/stop the A-IoT service.

Regarding the Uu RLF handling of UE reader, the UE reader can follow the legacy behaviour to initiate RRC re-establishment procedure and suspend the transmission of all RBs in the Uu interface. For the A-IoT air interface, the ongoing A-IoT service shall be suspended as well. After the re-establishment procedure, the UE reader can follow the configuration of the network to resume/re-initiate/stop the A-IoT service, which is similar to the HO case.

Proposal 5: For Uu RLF handling of UE reader, the UE reader follows the legacy behaviour to initiate RRC re-establishment procedure in the Uu interface.

Proposal 6: For the A-IoT air interface, the ongoing A-IoT service shall be suspended upon Uu RLF of the UE reader. After the re-establishment procedure, the UE reader follows the configuration of the network to resume/re-initiate/stop the A-IoT service.

UE reader selection

For the selection of UE reader, it could be the CN and/or gNB to determine a suitable UE reader upon an inventory and/or command service. Since this issue is under discussion in RAN3 and SA2, RAN2 can leave it for RAN3/SA2 decision.

Proposal 7: For the selection of UE reader, RAN2 leave it for RAN3/SA2 decision in the study phase.

Moreover, from RAN2’s perspective, before selecting a UE reader and sending the A-IoT radio configuration to a UE reader, the gNB shall get the UE radio capability of A-IoT and decide the configuration accordingly. Regarding the A-IoT UE radio capability as a UE reader, RAN2 can further discuss the details in the normative work phase.

Proposal 8: For A-IoT Topology 2, the gNB shall get the UE radio capability of A-IoT before sending the A-IoT related configuration. Further details could be discussed in the normative work phase.

- Conclusion

In this contribution, we discuss Topology 2 specific issues for Ambient IoT. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For A-IoT Topology 2, the validity of A-IoT radio resource is within the cell providing the resource configuration. No need to support resource validity across multiple cells.

Proposal 2: For A-IoT Topology 2, the UE determines the validity of A-IoT radio resource according to the configuration or indication from the network. No need to introduce the timer based solution.

Proposal 3: The UE determines the resource as valid after receiving the configuration of resource from the network. If the network indicates the UE to release or reconfigure the A-IoT resource, the UE can treat the previous resource as invalid.

Proposal 4: For HO of the UE reader, the UE reader suspends the ongoing A-IoT service during the HO procedure.  After HO to the target cell, the UE reader follows the configuration of the network to resume/re-initiate/stop the A-IoT service.

Proposal 5: For Uu RLF handling of UE reader, the UE reader follows the legacy behaviour to initiate RRC re-establishment procedure in the Uu interface.

Proposal 6: For the A-IoT air interface, the ongoing A-IoT service shall be suspended upon Uu RLF of the UE reader. After the re-establishment procedure, the UE reader follows the configuration of the network to resume/re-initiate/stop the A-IoT service.

Proposal 7: For the selection of UE reader, RAN2 leave it for RAN3/SA2 decision in the study phase.

Proposal 8: For A-IoT Topology 2, the gNB shall get the UE radio capability of A-IoT before sending the A-IoT related configuration. Further details could be discussed in the normative work phase.

- Reference

RP-240826	Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR 		CMCC, Huawei, T-Mobile USA

3GPP TR 38.848 v18.0.0: " Study on Ambient IoT (Internet of Things) in RAN".

R2-xxxxxx	draft Report of 3GPP TSG RAN WG2 meeting #125bis, 126,127