3GPP TSG-RAN WG2 Meeting #128	R2-2410138

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.2.4

Source:	Spreadtrum, UNISOC

Title:	Discussion on random access of Ambient IoT

Document for:	Discussion and Decision

1 Introduction

In RAN2#127bis meeting, RAN2 discussed the random access for A-IoT and achieved the following agreements [1]:

Agreements for Random Access 1 In case of D2R data transmission failure, device follows the reader instruction: - it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and - reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R). 2 Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data. 3 For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access. The re-access is always controlled by reader. 4 For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure. 5 R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message. FFS additional indications to differentiate if needed in paging message. FFS if other message can be used. 6 From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA. FFS if this is applicable to FDMA. Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

In this contribution, we will continue discuss the random access of Ambient IoT.

2 Discussion

## 2.1 Contention-based random access for A-IoT

3-step CBRA:

For initial inventory to identify multiple A-IoT devices, when a response is expected from multiple devices, an A-IoT contention-based access (CBRA) procedure initiated by the reader is used. For A-IoT contention-based access procedure, slotted-ALOHA based access is baseline. 3-step CBRA random access for A-IoT is shown in Figure 1.

Figure 1 3-step CBRA random access for A-IoT

Msg3 (re)transmission failure handling

In last meeting, it agreed that in case of D2R data (including Msg3 and following D2R data) transmission failure, device can re-access in another opportunity controlled/provided by the reader and reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response”. Whether/how to handle the Msg3 re-transmission for inventory case is still FFS.

If device use the re-access procedure to send the D2R data, while the contention resolution may be needed again in the re-access, there is a certain amount of redundant access for device and waste of resources. If reader repeats Msg2 to trigger device resend Msg3, we think device can retransmit Msg3 instead of re-access.

Proposal 1: If Msg3 transmission failures, device will retransmit Msg3 based on the received repeated msg2 from reader.

Failure/success feedback indication for following D2R data

In last meeting, it agreed that support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  But the following D2R data is still FFS.

We think it also can apply to the following D2R data after Msg3, since it is up to reader implementation to send the explicit R2D failure/success feedback indication without additional specification impact.

Proposal 2: Support optional explicit R2D failure/success feedback indication for following D2R data for re-access purpose.

2-step CBRA:

For some cases (e.g. Low collision probability of access), such as a small number of devices are triggered, Msg1 and Msg2 in 3-step CBRA may not be needed.  For 2-step CBRA, RAN2 meeting has agreed that A-IoT device can transmit device ID and any other upper layer data directly in Msg1 when receives an initial trigger message. And when Mgs2 is needed, the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.

Figure 2 2-step CBRA random access for A-IoT

In 2-step CBRA, if the device has command service after inventory, it needs a random ID to communication with the reader. For further optimization the signaling overhead, Msg1 only includes Device ID and any other upper layer data. After reader receiving Msg1, reader generates a random ID for the device based on the device ID. The random ID generation rules are also known to the device side. Then, the random ID is sent to the device in Msg2.

Proposal 3: Reader generates a random ID based on device ID and echoes Msg1 with the random ID in Msg2.

## 2.2 Contention resolution failure in CBRA

In CBRA, multiple devices may collide in the same access occasion since device is randomly selecting the access occasion. Device considers the contention resolution failure if the same ID is not received in the response from reader within a certain time, after transmitting the random access ID.

Last RAN2 meeting has agreed that device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure. But where to perform re-access is still controversial.

In random access email discussion [2], there are two options for re-access, option 1 is the access occasions following the subsequent A-IoT paging can be also used for re-access, and option 2 is reader can assign some access occasions for re-access purpose between two A-IoT paging.

We prefer option 2, same as RFID, consecutive access rounds triggered by one piece of paging message are for the same group of A-IOT devices. A-IOT devices fail to access in the previous access round could try to re-access in the next access round.

Proposal 4: Reader can assign some access occasions for re-access purpose between two A-IoT paging (e.g. in the next access round).

## 2.3 Down selection for RA types

In A-IoT SI, three random access types (i.e. 2-step CBRA, 3-step CRA, CFRA) have been studied so far. Different random access types are suitable for different scenarios. For example, 3-step CBRA is applied to the initial group inventory which the number of devices is not known and reader cannot allocate appropriate random access resources. 3-step CBRA can detect collisions much faster, it is beneficial to reader to adjust the resource allocation according to the collisions and improve the inventory efficiency.

2-step CBRA is suitable for lower latency inventory, the signalling overhead is also smaller. CFRA is suitable for dedicated inventory of one or multiple device. All RA types are baseline supported by devices, no needed for down selection.

Proposal 5: All RA types (e.g. 3-step CBRA, 2-step CBRA, CFRA) are supported by device, no need for down selection.

## 2.4 Efficiency enhancement on random access

Slotted-ALOHA is a contention resolution based on TDMA in backscatter wireless system. It is often referred as a probabilistic anti-collision algorithm, only slots that do not collide can be decoded successfully, with an efficiency of 36.8%. The efficiency here is defined as the ratio of the number of devices successfully inventoried to the number of time slots used.

And there indeed is another anti-collision algorithm candidate that is conflict resolution diversity slotted-ALOHA (CRDSA). The CRDSA key idea is to transmit two replicas of the same packet at random two slots within the same frame with a little extra signalling to point to the replica packet location. In case of successful packet reception at network this extra signalling information allows to locate the replica packet within the frame and to accurately cancel it in addition to the one successfully decoded. The effect of the other replica packet is eliminated by iterative self-interference cancellation (SIC) technology. The efficiency of CRDSA is 52%.

Figure 4 Conflict resolution diversity slotted-ALOHA (CRDSA)

Figure 5 Efficiency comparison of slotted-ALOHA and CRDSA

| Random access algorithm | Efficiency |
| --- | --- |
| Slotted-ALOHA | 36.8% |
| CRDSA | 52% |

Figure 6 The efficiency of random access algorithm

If there are chances to discuss enhancement for slotted-ALOHA, and the CRDSA access procedure should be considered.

Proposal 6: RAN2 to study CRDSA based access as a candidate for random access.

3 Conclusion

In this contribution we discuss the random access procedure of A-IoT device, with the following proposals:

Proposal 1: If Msg3 transmission failures, device will retransmit Msg3 based on the received repeated msg2 from reader.

Proposal 2: Support optional explicit R2D failure/success feedback indication for following D2R data for re-access purpose.

Proposal 3: Reader generates a random ID based on device ID and echoes Msg1 with the random ID in Msg2.

Proposal 4: Reader can assign some access occasions for re-access purpose between two A-IoT paging (e.g. in the next access round).

Proposal 5: All RA types (e.g. 3-step CBRA, 2-step CBRA, CFRA) are supported by device, no need for down selection.

Proposal 6: RAN2 to study CRDSA based access as a candidate for random access.

- References

- Draft report of 3GPP TSG RAN WG2 #127bis, RAN2 chair (interDigtal), Hefei, China, Oct 14th – 18th, 2024.

- R2-2407985, Report of [POST127][033][AIoT] Random Access, Huawei, HiSilicon, Hefei, China, 14th – 18th October, 2024.