3GPP TSG-RAN WG2 Meeting #128	R2-2410154

Orlando, US, 18th – 22nd Nov, 2024

Agenda item:	8.7.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM enhancements for XR

Document for:	Discussion and Decision

1. Introduction

During the last meeting, RAN2 has discussed about RAN2 impacts considering MG skipping. It was agreed to focus on supporting the solution chosen by RAN1 and RAN4. In this contribution, we will analyse the RAN2 impacts for the RAN1-selected DCI-based solution in detail, including the interaction with DSR and DRX. Besides, we will also discuss about some further solutions to improve the work considering the disadvantages of the DCI-based solution.

2. Discussion

2.1 Specification impacts for the DCI-based solution

From the RAN1 agreement, an explicit skip indication is carried by the physical layer DCI to indicate the MG skipping. While the indication is carried by the DCI, the exact UE procedure for UL/DL channels during the MG is performed by the MAC layer, e.g., as specified in TS 38.321 [1]. And the MG configuration and the semi-static activation/deactivation status is maintained in the RRC layer.

Considering that the skip indication in DCI will not affect the MG configuration, and the UE handling of measurement gap occasions is specified in MAC spec, it is better to let the PHY layer indicate such skipping indication to MAC layer only and the RRC layer should not be aware of such dynamic MG occasion skip. Hence, from spec modelling point of view, the lower layer should indicate the need of gap skipping to the MAC layer and the MAC layer shall consider the first MG occasion after the DCI reception with an offset to be deactivated. The following modification shows a potential way to implement the MAC spec.

## 5.14 Handling of measurement gaps During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5], for each measurement gap occasion which is not indicated to skip by the indication from the lower layer as specified in TS 38.212 [9]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH.

Proposal 1: PHY layer indicates the first MG after the DCI reception with an offset as skipped to MAC layer, and MAC layer considers the measurement gap occasion to be deactivated.

Besides, there are also some RRC impacts to support the DCI-based solution. Since the skipping indication is carried via the existing scheduling DCI, the UE will need to interpret the DCI differently when the MG skipping feature is enabled or disabled. To align the interpretation, the gNB should configure the UE to enable or disable the feature. Thus, a new parameter can be introduced to the measurement gap (re-)configuration messages, e.g., to add a new IE in the MeasGapConfig IE. Also, a new UE capability shall be introduced to indicate whether the UE supports to skip MG based on the indication in DCI.

Proposal 2: RAN2 to support the DCI-based MG skipping solution for the RRC spec:

- Introduce a new RRC parameter to configure MG skipping based on DCI;

- Introduce a new UE capability.

2.2 Interactions with RAN2 features

2.2.1 Impacts on DSR

According to RAN1 and RAN4 discussion, it is required that the time period between the MG and the DCI which indicates UE to skip the MG is larger than a minimum time offset. With this restriction, the gNB may be unable to indicate a specific MG skipping whenever it wants. For example, when DSR is configured, a DSR may be triggered before the MG but within the time offset. Upon receiving the DSR, the gNB realizes there is delay-critical data, but it has no chance to indicate the UE to skip the upcoming MG. Consequently, the transmission of the delay-critical data is blocked by the MG and may be outdated during the MG.

To address the issue, some companies proposed to introduce DSR enhancements, e.g., to trigger DSR sooner if the data is foreseen to expire during the MG, or to cancel the DSR if it is too close to the MG. However, in our view, such enhancements are an overoptimization and the issue can be addressed by the gNB implementation.

For UL XR services, the typical PSDB is 30ms according to the traffic model in TR 38.838. While the potential time offset is several milliseconds according to RAN4’s discussion, and the MG duration for RRM measurement is up to 6ms as specified in TS 38.331 in TS 383.133, the PSDB is large enough to prevent the MG from impacting the in-time scheduling.

Observation 1: PSDB for UL XR services is larger than the potential time offset and the MG duration.

As a feasible implementation based solution, the gNB can set the remaining time threshold for DSR to a value larger than the sum of the required time offset and the MG duration, so that for the data which would expire during the MG, the corresponding DSR can be triggered before the time offset. As illustrated in Fig.1, if the data is going to expire at the time point #A, DSR would be triggered at the time (#A – remaining time threshold), which is earlier than the time offset. If the data is going to expire at the time point #B, DSR would be triggered later than the time offset, which means there is no chance to skip the MG. But in this case, there is still remaining time for the data after the MG.

Fig.1 Implementation based solution for DSR interaction with MG skipping.

Observation 2: The issue of DSR interaction with MG skipping can be avoided by NW implementation, e.g., by setting a larger remaining time threshold.

Based on the above analysis, it can be seen that there is no need to enhance DSR considering interacting with MG skipping.

Proposal 3: No DSR enhancement is needed to consider for the interaction with MG skipping.

2.2.2 Impacts on DRX

There is also some concern for MG skipping when DRX is configured. One issue is that UE may go to sleep according to the DRX configuration during the skipped MG, which makes the skipping meaningless. Some companies proposed to consider the skipped MG duration as DRX active time to make sure data transmission can be performed during the skipped MG. However, we think the assumption is not reasonable. Since the gNB is aware of the DRX active time and the MG time, it should not indicate the UE to skip an MG which is out of the DRX active time.

Another issue is that when the MG overlapped with the DRX on-duration from the very beginning, there may be no opportunity for the gNB to indicate the MG skipping. This can happen when the DRX periodicity is larger than the MG periodicity since there may be no DRX active time between two MGs. While the DRX periodicity is usually configured according to the traffic pattern, the above issue is possible to appear considering the typical frame rates of XR services and the typical MG configuration.

An example can be found in Fig.2, where the DRX is configured with the periodicity 33.33ms, corresponding to 30fps, while the MG is configured with 20ms periodicity. The MG #3 conflicts with the DRX on-duration #2, while there no time for the gNB to send the DCI for MG skipping before the MG #3 during the on-duration #2. The last period of active time is the DRX on-duration #1. However, DCI sent during on-duration #1 can only indicate the skipping for MG#2. Consequently, MG#3 cannot be skipped and may influence the data transmission in the on-duration #2.

Fig.2 Issue for MG skipping with DRX configured.

Observation 3: When DRX is configured and the DRX periodicity is longer than the MG periodicity, the gNB may be unable to indicate the UE to skip the conflicting MGs considering the limited active time.

To resolve the above issue, we think some semi-static solutions can be considered. As both DRX and MG are configured semi-statically, it is unnecessary to use dynamic solutions to address the collision issue. For example, the gNB can configure a pattern for MG skipping in advance, so that the UE can skip the MG accordingly without receiving the DCI. In this way, the limited active time caused by DRX will not impact the effect of MG skipping. To make things clear, in the next part of this contribution, we will further introduce some semi-static solutions in detail.

Proposal 4: Semi-static solutions can be considered for addressing the collision between DRX and MG.

2.3 Semi-static solutions

As we analysed above, the dynamic DCI-based solution may be ineffective when DRX is configured. RRC-based semi-static solutions can be considered to overcome the issue.

Besides, semi-static solutions are more efficient in case CG or SPS is used. If the DCI-based solution is used to perform MG skipping for CG/SPS transmissions, the gNB would have to send more unnecessary scheduling DCIs, which is redundant to the CG/SPS transmission and decreases the CG/SPS benefit. Similar to DRX, CG and SPS are also configured in a semi-static way, the collision with MG can be easily derived, so that the semi-static solutions are more appropriate.

Proposal 5: Semi-static solutions can be considered for addressing the collision between CG and MG.

With the above analysis, to better support DRX and CG/SPS considering the coexistence with MG skipping, the following RRC-based solutions can be taken into account.

- Alternative 1: Skip the MGs which overlap with the DRX on duration or CG transmission occasions with a ratio larger than a threshold.

Considering the length of DRX on-duration, it may be not necessary to skip every conflicting MG. For example, if the overlapping ratio is small, the gNB can still perform the scheduling out the scope of the MG. Similar for CG, as we introduced multi-PUSCH CG in R18, if there is only a few CGOs overlapping with the MG, the UE can use the other CGOs for transmission and using UTO-UCI to indicate which CGOs are used to the gNB. In this case, the MG can be kept for RRM measurement. Thus it is proposed to determine whether to skip the MG based on its overlapping ratio with the DRX on duration or CG occasions. When the ratio of the overlapped part and the whole duration of DRX on duration or CG transmission occasions is larger than a threshold, the MG is skipped by the UE proactively to guarantee the data transmission.

To have a uniform solution for CG/SPS and DRX, a periodic pattern can be configured with the parameters such as periodicity, offset and duration via RRC, and this pattern can be related to XR traffic’s periodicity and PDB, which are also related to the CG and DRX configurations. By comparing the overlapping ratio between MG and this configured pattern, the UE can determine whether to skip the MG. It can be applied for CG/SPS and DRX simultaneously, without designing enhancements for each feature separately.

As shown by the example in Fig.3, the pattern periodicity is 16.67ms, which is the same as the XR traffic periodicity and the duration of pattern (yellow box in Fig.3) is 10ms which corresponds to PDB. Whether the MG overlapped with configured pattern is skipped or not can be derived by the overlapping ratio. The overlap between the duration of pattern and the duration of MG is marked as blue box. The higher the overlapping ratio, the greater impact on XR traffic. Then, if the overlapping ratio is larger than an RRC configured threshold, the MG can be cancelled. The gNB can configure a proper threshold based on historical XR frames delivery time.

Fig.3 Example of cancelling the measurement gaps based on overlapping ratio.

- Alternative 2: Configure the priority between the CG configuration and MG.

Different CG configurations may be used to transmit the data of different traffics. When the traffic requires stringent latency, the corresponding CG occasions are not expected to be disabled due to RRM measurements. While for the traffic without such requirements, it could be tolerable when the CG occasions colliding with MG are disabled. With the above considerations, it would be helpful to allow some of the CG configurations not to be impacted by RRM measurements

It should be noted that in the legacy positioning discussion in R17, priority between the PDSCH and PRS positioning window (PPW) was already supported. During the positioning procedure, the UE can determine whether to perform PRS reception within the PPW or PDSCH reception by the priority defined between these two DL. Similar to PPW, one feasible method is to set the priorities between the CG configuration and RRM measurement. For example, when configuring CG, it can be configured with a priority between the CG configuration and RRM measurement. In this way, the CG transmission for low-latency XR traffic can be guaranteed without having frequent dynamic indications for each CG occasion.

The above discussion is also applicable for DRX. When DRX overlaps with MG occasion, it can also be determined whether the MG should be skipped based on priority.

Proposal 6: RAN2 consider the following semi-static solutions for MG skipping:

- Alternative 1: Skip the MGs which overlap with the DRX on duration or CG transmission occasions with a ratio larger than a threshold.

- Alternative 2: Configure the priority between DRX or CG configuration and MG.

3. Conclusion

In this contribution, we analyzed the potential RAN2 impacts of the DCI-based solution for RRM measurement enhancements, and further propose to consider RRC-based solutions from RAN2 perspective. The following proposals were made:

Proposal 1: PHY layer indicates the first MG after the DCI reception with an offset as skipped to MAC layer, and MAC layer considers the measurement gap occasion to be deactivated.

Proposal 2: RAN2 to support the DCI-based MG skipping solution for the RRC spec:

- Introduce a new RRC parameter to configure MG skipping based on DCI;

- Introduce a new UE capability.

Proposal 3: No DSR enhancement is needed to consider for the interaction with MG skipping.

Proposal 4: Semi-static solutions can be considered for addressing the collision between DRX and MG.

Proposal 5: Semi-static solutions can be considered for addressing the collision between CG and MG.

Proposal 6: RAN2 consider the following semi-static solutions for MG skipping:

- Alternative 1: Skip the MGs which overlap with the DRX on duration or CG transmission occasions with a ratio larger than a threshold.

- Alternative 2: Configure the priority between DRX or CG configuration and MG.

4. Reference

[1]. 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[2]. 3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[3]. 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".