3GPP TSG-RAN WG2 Meeting #128											R2-2410164

Orlando, USA, Nov.18th–22nd, 2024

Agenda item:		8.5.3

Title: 	Remaining issues of on-demand SIB1 in idle and inactive mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In the latest RAN#105, the on demand SIB1 in idle and inactive mode has been turn from study phase into normative phase with case#2 only:

Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

In this paper, we share some consideration on the remaining issues for case#2.

- Discussion

  - UL WUS configuration

The following agreements have been reached in the past for UL WUS [1-3]:

Agreements at RAN2#125bis:

- RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least RACH configuration.

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

Agreements at RAN2#126:

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

Agreements at RAN2#127:

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Firstly we need to consider what kind of information should be included in the UL WUS configuration.

- Random access related configuration

We have clarify whether to support 2-step RACH procedure for on demand SIB1 request, which will impact the content of the RACH configuration. From our perspective, we see benefits in using the 2-step RACH procedure to reduce the latency.

Proposal 1: 2-step RACH based on demand SIB1 procedure is supported.

We have agreed that the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline, but currently we have SI-RequestConfig and SI-RequestConfigRepetition. We understand that SI-RequestConfigRepetition is used for Msg1 repetition while there is no need to support Msg1 repetition as we expect UE to stay in Cell A as long as possible and only request SIB1 from NES Cell when needed thus there is no motivation to enhance the coverage of NES cell.

Proposal 2: Msg1 repetition is not supported for on demand SIB1 request for requesting to the NES Cell.

For the following parameters in SI-RequestConfig, we understand si-RequestPeriod and si-RequestResources are not needed as they are mainly used to associate the SI message with the RACH resources while for on demand SIB1 case, UE does not need to differentiate.

SI-RequestConfig ::=                SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}     OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

Proposal 3: si-RequestPeriod and si-RequestResources are not needed for the on demand SIB1 request.

Apart from the above two parameters, we understand the following parameters in RACH-ConfigGeneric and ssb-perRACH-Occasion are needed:

- prach-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

- ra-ResponseWindow

Apart from the above parameters, we understand the following parameters in RACH-ConfigCommon are also needed:

- totalNumberOfRA-Preambles, but refers to the total number of preambles for on demand SIB1 request

- CB-PreamblesPerSSB

- ra-ContentionResolutionTimer

- rsrp-ThresholdSSB

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

Observation 1: The followings parameter are needed for Msg1 based on demand SIB1 request and should be configured per NES cell:

- ssb-perRACH-Occasion

- prach-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

- ra-ResponseWindow

- totalNumberOfRA-Preambles, refers to the total number of preambles for on demand SIB1 request

- CB-PreamblesPerSSB

- ra-ContentionResolutionTimer

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

Compared with RAN1 agreement, we understand the following parameters are not included:

- totalNumberOfRA-Preambles, refers to the total number of preambles for on demand SIB1 request

- CB-PreamblesPerSSB

- msg3-transformPrecoder

For totalNumberOfRA-Preambles, if not included, we understand that all 64 preambles would be for on demand SIB1 request. For CB-PreamblesPerSSB, if not included, we understand that all the preambles would be contention based. For msg3-transformPrecoder, it is up to RAN1 to decide whether to always disable or enable Msg3 transform pre-coder.

Proposal 4: An LS is sent to RAN1 to confirm that:

- All the 64 preambles would be for on demand SIB1 request and would all be contention based thus there is no need to include the totalNumberOfRA-Preambles and CB-PreamblesPerSSB.

- And it is up to RAN1 to decide whether to always disable or enable Msg3 transform pre-coder if we do not include msg3-transformPrecoder in WUS configuration.

If we support 2-step RACH based procedure, the following parameters from RACH-ConfigGenericTwoStepRA and RACH-ConfigCommonTwoStepRA are also needed:

- msgA-PRACH-ConfigurationIndex

- msgA-RO-FDM

- msgA-RO-FrequencyStart

- msgA-ZeroCorrelationZoneConfig

- msgA-PreamblePowerRampingStep

- msgA-PreambleReceivedTargetPower

- msgB-ResponseWindow

- preambleTransMax

- msgA-SSB-PerRACH-Occasion

- msgA-SSB-SharedRO-MaskIndex

- groupB-ConfiguredTwoStepRA

- msgA-PRACH-RootSequenceIndex

- msgA-TransMax

- msgA-RSRP-Threshold

- msgA-RSRP-ThresholdSSB

- msgA-SubcarrierSpacing

- msgA-RestrictedSetConfig

Proposal 5: If 2-step RACH based on demand SIB1 request procedure is supported, the following parameters are needed and should be configured per NES cell:

- msgA-PRACH-ConfigurationIndex

- msgA-RO-FDM

- msgA-RO-FrequencyStart

- msgA-ZeroCorrelationZoneConfig

- msgA-PreamblePowerRampingStep

- msgA-PreambleReceivedTargetPower

- msgB-ResponseWindow

- preambleTransMax

- msgA-SSB-PerRACH-Occasion

- msgA-SSB-SharedRO-MaskIndex

- groupB-ConfiguredTwoStepRA

- msgA-PRACH-RootSequenceIndex

- msgA-TransMax

- msgA-RSRP-Threshold

- msgA-RSRP-ThresholdSSB

- msgA-SubcarrierSpacing

- msgA-RestrictedSetConfig

- Triggering condition of on demand SIB1 request

The following agreements have been made before on the triggering condition of WUS transmission:

Triggering condition of WUS transmission

If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

With the above agreement, we understand the NES cells would be included as neighbour cells in the cell reselection information provided in SIB3 and SIB4 and UE would trigger on demand SIB1 procedure if the candidate target cell for cell reselection is a NES cell. With such understanding, the existing cell reselection parameters can be reused.

Proposal 6a: An NES cell should be configured as a neighbor cell in the cell reselection information provided via SIB3 or SIB4. UE trigger on demand SIB1 procedure if the candidate target cell for cell reselection is an NES cell.

Proposal 6b: The existing cell reselection parameters in SIB2/3/4 can be reused for UE to select the target NES cell to trigger on demand SIB1 request.

  - UE behavior related to OD-SIB1 request and failure

The following agreement has been made with FFS left on the UE behavior:

Agreements at RAN2#125bis

Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

Agreements at RAN2#126

RAR monitoring for MSG1 based OD-SIB1 REQ

RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

We understand for the Msg1 based on demand procedure, UE can start to acquire the requested SIB1 upon reception of RAR. For 2-step RACH based on demand procedure, UE would start to acquire the requested SIB1 upon reception of MsgB.

Proposal 7a: UE start to acquire the requested SIB1 upon reception of RAR for Msg1 based on demand procedure.

Proposal 7b: If 2-step RACH based on demand SIB1 procedure, UE start to acquire the requested SIB1 upon reception of the MsgB.

The following agreement had been made last meeting for the failure case [4]:

Agreements at RAN2#127bis

The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

We understand for the existing on demand SI request, we allow UE to re-try after RACH failure with the following note captured in 5.2.2.3.3 in TS 38.331.

NOTE:	After RACH failure for SI request it is up to UE implementation when to retry the SI request.

Thus for the FFS, we would like to have similar principle and allow UE to re-try in implementation.

Proposal 8: After RACH failure for SIB1 request, it is up to UE implementation when to re-try the SIB1 request.

  - Allow NES UE to reselect to cells that are prevented from legacy UEs

The following agreements have been at RAN2#127bis [2]:

1. Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

2. NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

For NES UE, if the WUS configuration is available for a cell with no SIB1 indication in MIB, NES UE should not treat it as barred before triggering on demand SIB1 request. NES UE would consider it as barred if UE is unable to acquire SIB1 after triggering on demand SIB1 request.

Proposal 9a: For NES UE, if the WUS configuration is available for a cell with no SIB1 indication in MIB, NES UE should not treat it as barred before triggering on demand SIB1 request.

Proposal 9b: NES UE would consider this cell as barred if UE is unable to acquire SIB1 after triggering on demand SIB1 request.

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

UL WUS configuration

Proposal 1: 2-step RACH based on demand SIB1 procedure is supported.

Proposal 2: Msg1 repetition is not supported for on demand SIB1 request for requesting to the NES Cell.

Proposal 3: si-RequestPeriod and si-RequestResources are not needed for the on demand SIB1 request.

Proposal 4: An LS is sent to RAN1 to confirm that:

- All the 64 preambles would be for on demand SIB1 request and would all be contention based thus there is no need to include the totalNumberOfRA-Preambles and CB-PreamblesPerSSB.

- And it is up to RAN1 to decide whether to always disable or enable Msg3 transform pre-coder if we do not include msg3-transformPrecoder in WUS configuration.

Proposal 5: If 2-step RACH based on demand SIB1 request procedure is supported, the following parameters are needed and should be configured per NES cell:

- msgA-PRACH-ConfigurationIndex

- msgA-RO-FDM

- msgA-RO-FrequencyStart

- msgA-ZeroCorrelationZoneConfig

- msgA-PreamblePowerRampingStep

- msgA-PreambleReceivedTargetPower

- msgB-ResponseWindow

- preambleTransMax

- msgA-SSB-PerRACH-Occasion

- msgA-SSB-SharedRO-MaskIndex

- groupB-ConfiguredTwoStepRA

- msgA-PRACH-RootSequenceIndex

- msgA-TransMax

- msgA-RSRP-Threshold

- msgA-RSRP-ThresholdSSB

- msgA-SubcarrierSpacing

- msgA-RestrictedSetConfig

Proposal 6a: An NES cell should be configured as a neighbor cell in the cell reselection information provided via SIB3 or SIB4. UE trigger on demand SIB1 procedure if the candidate target cell for cell reselection is an NES cell.

Proposal 6b: The existing cell reselection parameters in SIB2/3/4 can be reused for UE to select the target NES cell to trigger on demand SIB1 request.

UE behaviour related to OD-SIB1 request and failure

Proposal 7a: UE start to acquire the requested SIB1 upon reception of RAR for Msg1 based on demand procedure.

Proposal 7b: If 2-step RACH based on demand SIB1 procedure, UE start to acquire the requested SIB1 upon reception of the MsgB.

Proposal 8: After RACH failure for SIB1 request, it is up to UE implementation when to re-try the SIB1 request.

Allow NES UE to reselect to cells that are prevented from legacy UEs

Proposal 9a: For NES UE, if the WUS configuration is available for a cell with no SIB1 indication in MIB, NES UE should not treat it as barred before triggering on demand SIB1 request.

Proposal 9b: NES UE would consider this cell as barred if UE is unable to acquire SIB1 after triggering on demand SIB1 request.

- Reference

- R2-2404102 RAN2#125bis Meeting Report

- R2-2406202 RAN2#126 Meeting Report

- Draft_RAN2_127_Meeting_Report_v2

[4] Draft_RAN2_127bis_Meeting_Report

- Annex- RAN1 agreements for UL WUS configuration

----------------------------------------------------------------RAN1#118bis--------------------------------------------------------------

Agreement

For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration:

- rsrp-ThresholdSSB

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon

Agreement

For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration.

| Purpose | Parameters |  |
| --- | --- | --- |
| WUS transmission | frequencyInfoUL | frequencyBandList |
|  |  | absoluteFrequencyPointA |
|  |  | offsetToCarrier |
|  |  | p-Max |
|  |  | frequencyShift7p5khz (working assumption)ULSubCarrierSpacing |

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

Note:

- In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB

- Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

----------------------------------------------------------------RAN1#118bis--------------------------------------------------------------