3GPP TSG RAN WG2 #128				R2-2410178

Orlando, USA, Nov. 18th – 22nd, 2024	Revision of R2-2408356

Agenda Item:	8.2.3

Source:		ASUSTeK

Title:				Discussion on Ambient IoT paging message with multiple IDs

Document for: Discussion and Decision

# Introduction

In RAN2#126 meeting [1], the following agreements on Ambient IoT device paging were made:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In RAN2#127bis meeting [2], the following agreements on Ambient IoT device paging were made:

Agreements on AIoT paging

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

And in RAN1#117 meeting [3], the following agreements on Ambient IoT channel design were made:

Agreement

Scheduling information of PDRCH transmission is provided by a corresponding PRDCH.

Agreement

For R2D, the only physical channel is PRDCH.

- PRDCH carries any higher-layer payload

- PRDCH carries L1 R2D control information (if defined)

- FFS details of device behaviour(s) for receiving PRDCH

Agreement

For D2R

- PDRCH carries any higher-layer payload

- PDRCH carries L1 D2R control information (if defined)

- Note: PDRCH carries the response agreed at RAN1#116

In this contribution, we further discuss the schedule information in the A-IoT paging message.

# Discussion

In previous meetings, it was agreed that it is feasible to support paging multiple device IDs. The agreement raised another question that how the paged device(s) respond to the A-IoT paging message including multiple device IDs. On the other hand, it was agreed in RAN2 that if multiple device IDs in single paging is supported and if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA. It was also agreed in RAN1 that the scheduling information of PDRCH transmission is provided by a corresponding PRDCH. For the case of A-IoT paging and its corresponding response, the scheduling information in the A-IoT paging message should be associated to an ID of the paging request.

For example, if the A-IoT paging message indicates an ID of a single device, the A-IoT paging message should include the scheduling information for PDRCH transmission corresponding to the device ID. And if the A-IoT paging message indicates a group ID, the A-IoT paging message should include the scheduling information for PDRCH transmission corresponding to the group ID. Additionally, as demonstrated in other discussions [4][5], if more than one devices are supposed to perform D2R transmission without collision (e.g. for CFRA), the D2R resource (e.g. time and/or frequency resource) should be provided to each device separately. As a result, if multiple IDs are included in a A-IoT paging message, there should be a D2R resource, which is indicated by the scheduling information in the paging message, mapped to each device ID.

Proposal 1:  The A-IoT paging message includes scheduling information for corresponding response, where each scheduling information is associated to an ID of the paging request.

Moreover, it is also possible to include multiple “requests” (e.g. command, inventor, inventory + command) in the paging message for a single device, at least in some conditions. For example, the reader would read some (first) information from a device group to which a first device belongs, and write some (second) information to the first device. It would be efficient to include both requests in the same paging message, if the two requests don’t conflict with or have impact on each other. As for which conditions the multiple “requests” are allowed to be sent to the same device can be further discussed.

Proposal 2: 	A-IoT paging message could include multiple command(s), inventor(ies), and inventory(ies) plus command(s) for a single device, at least in some conditions.

# Conclusion

Proposal 1:  The A-IoT paging message includes scheduling information for corresponding response, where each scheduling information is associated to an ID of the paging request.

Proposal 2: 	A-IoT paging message could include multiple command(s), inventor(ies), and inventory(ies) plus command(s) for a single device, at least in some conditions.

# Reference

[1] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

[2] Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China

[3] RAN1 Chair’s Notes for 3GPP TSG RAN WG1 #117 (Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024)

[4] R2-2407283, “Discussion on A-IoT paging”

[5] R2-2406856, “Ambient-IoT Paging”