3GPP TSG-RAN WG2 #128	R2-2410188

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda Item:	8.3.5

Source:	OPPO

Title:	Discussion on generalization

Document for:	Discussion, Decision

Introduction

This paper discusses how to study generalization aspect of this SID.

Discussion

For RRM measurement use case, after 2nd round evaluation in Oct. meeting, it is expected that the evaluation can be likely conclude in November meeting. Considering the similarity w.r.t model between RRM measurement and indirect measurement event/RLF event prediction, RAN2 should focus on RRM measurement prediction for generalization study.

Proposal 1: As for the generalization study, RAN2 focus on RRM measurement prediction

RAN2 conclude some general observations as following:

Agreements on intra-frequency

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors

- Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy.  RAN2 will not define the actual threshold and fast fading assumption.

- Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy

- Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used.   The companies should at least provide the results of only one value it should be the last value at the end of the PW.   We will add two columns in the spreadsheet to capture the last value and the average value.

Agreements on inter-frequency

- Companies need to report whether earlier predicted results are also used as inputs for future RRM prediction.

- Companies should report with their simulation the correlation coefficient

- Higher-to-lower and lower-to-higher frequency prediction is comparable

- For co-located scenario, the UE speed in the inter-frequency case has minor impact on prediction accuracy

Table 2.1 observation of RRM measurement prediction

For temporal domain prediction, some key parameters i.e. UE speed, OW/PW and sub cases will impact the prediction accuracy significantly. Among those parameters proper OW and PW configuration will result in best prediction accuracy. And obviously it is not necessary to try all sub-cases for further study if majority company already conclude that sub-case 2 demonstrate better performance than sub-case 1 and sub-case 3. So, it sounds that UE speed is the key parameter for generalization study.

For temporal domain prediction, prediction accuracy will decrease with the increase of frequency distance because it is obivous that bigger frequency distance will have lower frequency correlation efficient. For example it makes less sense to predict a FR2 frequency based on FR1 frequency. Within same frequency range, it does. However majority companies have data set for two frequencies i.e. 2GHz and 4GHz. It means more data set is needed if generalization over frequencies is considered. But considering now SLS for HO performance is also on the table, RAN2 may spend less time on generalization study.

Proposal 2: RAN2 focus on generalization over UE speed for temporal domain prediction

There are 3 generalization cases in RAN1’s CR to 38.843 which could be a good reference for RAN2 study also. Assuming the generalization elements is UE speed and there are UE speed #A, #B and #C

Case 1: The AI/ML model is trained based on dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B

Case 2: The AI/ML model is trained based on dataset subject to a certain UE speed#B and applied for inference with a same UE speed#A

Case 3: The AI/ML model is trained based on mixed datasets subject to multiple UE speed #A,#B and#C and applied for inference with a single UE speed out of #A,#B and #C

Proposal 3: To introduce generalization use case 1,2 and 3 as following:

Case 1: The AI/ML model is trained based on dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B

Case 2: The AI/ML model is trained based on dataset subject to a certain UE speed#B and applied for inference with a same UE speed#A

Case 3: The AI/ML model is trained based on mixed datasets subject to multiple UE speed #A,#B and#C and applied for inference with a single UE speed out of #A,#B and #C

For FR1 temporal domain case B, the 3 UE speeds are 30Km/h, 60Km/h and 90Km/h

For FR2 temporal domain case A, the 3 UE speeds are 60Km/h, 90Km/h and 120Km/h

It is obvious the simulation assumption for FR1 temporal domain case B and FR2 temporal domain case A should be reused for generalization study.

Proposal 4: The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h

Proposal 5: The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h

Conclusion

Proposal 1: As for the generalization study, RAN2 can focus on RRM measurement prediction

Proposal 2: RAN2 focus on generalization over UE speed

Proposal 3: To introduce generalization use case 1,2 and 3 as following:

Case 1: The AI/ML model is trained based on dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B

Case 2: The AI/ML model is trained based on dataset subject to a certain UE speed#B and applied for inference with a same UE speed#A

Case 3: The AI/ML model is trained based on mixed datasets subject to multiple UE speed #A,#B and#C and applied for inference with a single UE speed out of #A,#B and #C

Proposal 4: The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h

Proposal 5: The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h

Reference

RP-242393 Revised Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RP‑241911  CR for TR 38.843 capturing agreements from study objectives, Qualcomm Innovation Center Inc