3GPP TSG-RAN WG2 Meeting #128	R2-2410199

# Orlando, USA, 18th – 22nd November 2024

Agenda Item:	8.7.5

Source:	Ericsson

Title:	More Discussions on RLC AM Enhancements

Document for:	Discussion, Decision

# 1	Introduction

The following were the agreements from the RLC AM enhancement discussions in RAN2#127bis:

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In this contribution, we discuss further on the details of the combined approach for avoid unnecessary retransmissions. Further, provide our views on the two approaches for RLC timely retransmissions.

This paper will also provide a discussion on the incoming SA2 LS (S2-2410999) on AL-FEC.

# 2	Discussion

## 2.1 	Combined Tx-Rx Approach

The new local timer enables the RLC layer to be aware of the SDUs relevance. When running, the RLC layer is aware that this SDU is still relevant to the PDCP layer but upon expiry, would recognize that the corresponding PDCP PDU is irrelevant and hence, the SDU can be abandoned.

- The local timer enables RLC to be aware of the SDUs relevance in the PDCP.

The local timer need not be perfectly synchronized with t-Reordering, but it should allow for a certain number of retransmissions before abandonment thereby limiting the number of ARQ retransmissions to deliver the missing RLC data. As a result, the length of the new timer can have values in the range of t-Reordering.

The length of the new timer can be of the order of t-Reordering.

The local timer is started when a higher RLC SN than RX_Next has been delivered to the upper layers (i.e., when a gap is created), then RLC knows that the PDCP will only wait a finite time for the missing PDCP SN before advancing the PDCP Rx window. The stopping condition is when the gap no longer exists i.e., the missing SNs have been received.

Local timer can be started when a higher RLC SN than RX_Next has been delivered to the upper layers.

Local timer can be stopped/reset when RX_Next_Highest is equal to RX_Next.

Upon expiry of the local timer, as agreed in the last meeting, the Rx side considers the corresponding SDU to be abandoned and informs the Tx side with an ACK for the abandoned SDU i.e., indicating to the Tx side with a fake ACK for the abandoned SDUs.

Upon expiry of the local timer, the Rx side informs the Tx side with an ACK in the RLC status report for the abandoned SDU.

For handling of the control PDUs, typically the size of control PDUs are quite small, < 2 octets except for the PDCP status report/PDCP SN gap report which can be at least 6 octets. In practice, with a 10% BLER target, such small PDUs are successfully transmitted in the initial transmission or with the least number of retransmission attempts. Thus, proper network configuration of the local timer is sufficient to allow for receiving the control PDUs without being abandoned.

Proper network configuration of the local timer is sufficient to handle the control PDUs.

For the indication from the Tx to the Rx, given the agreement that the Rx window is moved first and then the Tx window is moved to maintain synchronization, there is no need for any Tx side indication. In addition, given that RLC performs out-of-order delivery, there is reordering delay at the Rx side. As a result, we think there is no need for any indication from the Tx to Rx.

As RLC performs out-of-order delivery to upper layers, no indication is needed from the Tx to Rx.

Refer to the TP in the Annex.

## 2.2 	Timely Retransmissions

In the last meeting, RAN2 agreed to focus on the autonomous retransmissions and polling enhancements. In the following sections, we provide our analysis on these methods.

### 2.2.1	Autonomous Retransmissions

The proponents for the autonomous retransmissions claim that there is a significant delay in receiving the RLC status report from the Rx side and thus, the discard timer on the PDU can run out. As the round-trip-time (RTT) for receiving the RLC status report seems to be the bottleneck, we should first discuss the possible RTTs and thus, the applicability of the autonomous retransmissions.

- It is important to establish the applicability of autonomous retransmissions in the context of the discard timers.

In practice, for ongoing UL traffic, the RTT for receiving the RLC status report is around 8-10 ms and not more than 20 ms in a loaded gNB scenario. For discard timers longer than 20 ms, the UE has enough time to receive the RLC status report and thus, these autonomous retransmissions are not relevant. Hence, the autonomous retransmissions are only relevant for discard timers less than 20 ms i.e., short discard timers.

- Autonomous retransmissions are only relevant for short discard timers.

For short discard timers, although one could argue that the RLC status PDUs (RLC control PDUs) are prioritized, there is not enough time to trigger RLC retransmissions i.e., RLC AM is not suited for such short discard timers. However, the blind HARQ retransmissions (under gNB control) can be done and are better when compared to RLC retransmissions because of the 3 dB gain from the soft combining across different transmissions.

- For shorter discard timers, the Tx entity (under gNB control) can continue to perform blind HARQ retransmissions (rather than triggering RLC retransmissions) thereby obtaining a 3 dB gain from soft combining.

To better understand the difference between HARQ and RLC transmissions, we ran simulations with XR traffic for short and long discard timers. We quantified the result in terms of satisfied number of UEs with RLC and HARQ retransmissions shown below:

Figure 1: Satisfied Fraction of UEs with a combination of HARQ and RLC retransmissions for short discard timers of 10, 15 and 20 ms.

Figure 2: Satisfied Fraction of UEs with a combination of HARQ and RLC retransmissions for short discard of 10, 15 and 20 ms with shorter t-reassembly.

As seen in Figure 1, RLC retransmissions do not provide any gains as opposed to HARQ retransmissions. With the red curve (which runs RLC UM, without a limit for the #HARQ retransmissions) being a good indication for the efficacy of HARQ retransmissions. Additionally, we tried to increase the number of RLC retransmissions by reducing the t-reassembly timer, but this results in the same outcome.

- For short discard timers, the efficacy of HARQ retransmissions is higher than RLC retransmissions.

Figure 3: Satisfied Fraction of UEs with a combination of HARQ and RLC retransmissions for long discard timers of 100 and 200 ms.

Figure 4: Satisfied Fraction of UEs with a combination of HARQ and RLC retransmissions for long discard timers of 100 and 200 ms with shorter t-reassembly.

In comparison and as expected, for longer discard timers, RLC retransmissions are more effective, provide more gains and results in a higher percentage of satisfied users as shown in Figure 3 and 4.

- Gains with RLC retransmissions are only obtained for long discard timers.

For autonomous retransmissions, with a BLER target usually set to 10%, 90% of these transmissions are redundant and thus leads to wastage of resources. In addition, from a system perspective, UEs in bad radio channel conditions would essentially consume 2x UL resources per UE, per RLC SDU retransmission, where x depends on segmentation. This would be detrimental in an already resource constrained environment exacerbating UL congestion in the cell. Further, there is no guarantee that such autonomous retransmissions will be successful.

- Autonomous retransmissions will be redundant 90% of the time and would consume 2x UL resources per UE, per RLC SDU retransmission. For example, if segmented into 4 AMD PDUs, a UE would consume eight times the UL resources which would exacerbate UL congestion.

Do not pursue autonomous retransmissions to achieve timely retransmissions.

### 2.2.2	Polling Enhancements

One could optimize the current polling mechanism based on the argument that this mechanism might result in the RLC status reports being sent either too frequently (too early) or sparsely (too late). Such frequent status reports are not always necessary because not all the AMD PDUs are delay critical at a given time and hence, it leads to overheads in transmission. On the other hand, it cannot be too sparse (too late) because this would result in the AMD PDU to be delay critical or even discarded if received too late i.e., current polling mechanism might not always be synchronized with the delay requirements of the AMD PDU as the Rx entity is unaware of the remaining time to discard.

- The current polling mechanism cannot consider the delay criticality of the AMD PDU in the buffer. It is a trade-off between transmission overhead and timely reception of the RLC status PDU.

As a result, if any enhancements are to be pursued to enable timely retransmission for XR traffic, it should be enhancements to the polling mechanism. Such enhancements to the polling mechanism could consider the delay criticality of the AMD PDU in the buffer. The advantages are that the polling is set only when required and it helps the Rx entity to send the RLC status report in a timely manner.

Pursue enhancements to the polling mechanism where the delay criticality of the AMD PDU is considered.

With the polling enhancements and upon receiving this RLC status report, the UE should also prioritize retransmissions for packets which are delay critical among those which are pending retransmission. Thus, RAN2 should also discuss the enhancements to the retransmission procedures upon receiving the RLC status report.

Consider enhancements to the retransmission procedure up on receiving the RLC status report based on the polling enhancements.

## 2.3	Discussion on SA2 LS on AL-FEC

### 2.3.1	General on AL-FEC in RAN

The solution depicted using AL-FEC is for discarding data in RAN. Related to this topic RAN2 introduced PDU Set dropping in Rel-18 with the purpose to discard all PDCP PDUs belonging to one PDU Set when the PDU Set will not be delivered on time to the receiver. The algorithms in which the gNB decide to discard PDU Sets are not specified; however, it can be understood that the gNB will assess whether it can deliver each PDU Set within the corresponding time budget before transmission of the PDU Set starts. Either all packets are attempted to be transmitted or none of them. Evaluations have shown that discarding only a few PDCP PDUs within a PDU Set does not result in any substantial benefit for the network. When looking at the FEC discarding solution, this non-beneficial scenario is the scenario targeted and there will thus be very small gains.

Furthermore, and more importantly, when the network is experiencing congestion, one of the worst solutions is that the application transmits redundant data such as FEC. Adding FEC information will always result in an increase of the congestion level in the network unless perfect discarding is applied. Such perfect discarding is unachievable with an FEC solution. Thus, queues will increase and so will delays. More PDU Sets may be dropped, and more applications and services will have to rate adapt down their bit rates. This will, in turn, result in poorer image quality and worse perceived service to the end user.

Therefore, as a principle, transmitting even more data in congestion situations, is not really justified and not desirable regardless of the underlying RAN protocols or configured RLC mode. In general, the best the application can do is to transmit the minimal amount of data for the given desired service quality and provide the corresponding PDU Set information specified in Release 18.

- Adding redundant data has a negative impact on RAN performance and can be harmful to user experience.

### 2.3.2	Discussion on Question 1

As outlined in 2.3.1, FEC or any other mechanism adding redundancy information is not desired, especially during congestion. Thus, RAN2 should not recommend the use of FEC. In fact, if redundancy information is added, the gNB will not be able to make use of the content ratio, or any other marking, as is explained below and there will be negative performance impacts.

A baseline principle is that in congestion, the NW will not want to transmit redundant information. In the context of FEC this means that, before the network transmits the additional FEC data, it should be very clear that not enough packets have been successfully received. That implies that the gNB should have received, when in RLC AM, an RLC status report message from the UE. It is obvious that, during congestion, the gNB will not be able to request RLC status reports often and, transmission of the RLC status report in the UL may take time. In any case until this information is obtained, the NW should not transmit the FEC content. However, when a negative status report is received, the RLC entity will anyway retransmit the failed packets. The network will continue retransmitting the initial packets and, thus, the additional FEC packets are of no help.

- RLC AM will continue retransmission of the initial packets making the additional FEC packets redundant.

Additionally, as long as there is one packet in the head of the queue, other successive packets will not be transmitted. This means that buffered FEC packets of one PDU Set will negatively impact the following packets or PDU Sets and they will have less delay budget to be transmitted due to the buffered FEC packets blocking new transmissions.

- Buffered FEC packets of a PDU set will negatively impact the subsequent packets or PDU sets due to head of the line blocking.

For FEC packets to be of any use, they should be sent on time and that may imply that the NW will likely have to send them before receiving any status reports, i.e. no buffering as outlined above. This means that, anyway, the NW will transmit the complete PDU Set including FEC information and only when RLC ACKs are received, there is a possibility to move the transmitting or receiving window. However, at this point the transmissions cannot be stopped. All data which is in the MAC entity will continue being transmitted regardless of any RAN2 improvements done in Release 19.

Despite any potential RLC AM enhancements the content ratio for discarding will not improve XR performance but instead introduce additional packet delays or increased network load.

### 2.3.2	Discussion on Question 2

The same answer as above applies: the gNB will not be able to make use of the content ratio or any other marking.

The main difference is that in RLC UM, there are no status reports and, thus, the transmitter side cannot know for sure if the packets were successfully transmitted. That would result in that FEC packets need to be transmitted too. All the problems outlined in earlier sections are still applicable also to RLC UM.

Content ratio or any other FEC information is of no use for the network regardless of the RLC mode configured by the NW.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	The local timer enables RLC to be aware of the SDUs relevance in the PDCP.

Observation 2	It is important to establish the applicability of autonomous retransmissions in the context of the discard timers.

Observation 3	Autonomous retransmissions are only relevant for short discard timers.

Observation 4	For shorter discard timers, the Tx entity (under gNB control) can continue to perform blind HARQ retransmissions (rather than triggering RLC retransmissions) thereby obtaining a 3 dB gain from soft combining.

Observation 5	For short discard timers, the efficacy of HARQ retransmissions is higher than RLC retransmissions.

Observation 6	Gains with RLC retransmissions are only obtained for long discard timers.

Observation 7	Autonomous retransmissions will be redundant 90% of the time and would consume 2x UL resources per UE, per RLC SDU retransmission. For example, if segmented into 4 AMD PDUs, a UE would consume eight times the UL resources which would exacerbate UL congestion.

Observation 8	The current polling mechanism cannot consider the delay criticality of the AMD PDU in the buffer. It is a trade-off between transmission overhead and timely reception of the RLC status PDU.

Observation 9	Adding redundant data has a negative impact on RAN performance and can be harmful to user experience.

Observation 10	RLC AM will continue retransmission of the initial packets making the additional FEC packets redundant.

Observation 11	Buffered FEC packets of a PDU set will negatively impact the subsequent packets or PDU sets due to head of the line blocking.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	The length of the new timer can be of the order of t-Reordering.

Proposal 2	Local timer can be started when a higher RLC SN than RX_Next has been delivered to the upper layers.

Proposal 3	Local timer can be stopped/reset when RX_Next_Highest is equal to RX_Next.

Proposal 4	Upon expiry of the local timer, the Rx side informs the Tx side with an ACK in the RLC status report for the abandoned SDU.

Proposal 5	Proper network configuration of the local timer is sufficient to handle the control PDUs.

Proposal 6	As RLC performs out-of-order delivery to upper layers, no indication is needed from the Tx to Rx.

Proposal 7	For timely retransmissions, do not pursue autonomous retransmissions.

Proposal 8	Pursue enhancements to the polling mechanism where the delay criticality of the AMD PDU is considered.

Proposal 9	Consider enhancements to the retransmission procedure up on receiving the RLC status report based on the polling enhancements.

Proposal 10	Despite any potential RLC AM enhancements the content ratio for discarding will not improve XR performance but instead introduce additional packet delays or increased network load.

Proposal 11	Content ratio or any other FEC information is of no use for the network regardless of the RLC mode configured by the NW.

# 5	TP – 38.323

START OF CHANGES

#### 5.2.3.2	Receive operations

##### 5.2.3.2.1	General

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

-	a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

-	a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received AMD PDU or place it in the reception buffer (see clause 5.2.3.2.2);

-	if the received AMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see clause 5.2.3.2.3).

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reassembly as needed (see clause 5.2.3.2.4).

##### 5.2.3.2.2	Actions when an AMD PDU is received from lower layer

When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the RLC SDU with SN = x have been received before:

-	discard the received AMD PDU.

-	else:

-	place the received AMD PDU in the reception buffer;

-	if some byte segments of the RLC SDU contained in the AMD PDU have been received before:

-	discard the duplicate byte segments.

##### 5.2.3.2.3	Actions when an AMD PDU is placed in the reception buffer

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= RX_Next_Highest:

-	update RX_Next_Highest to x+ 1.

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Highest_Status:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

-	if x = RX_Next:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

-	if t-Reassembly is running:

-	if RX_Next_Status_Trigger = RX_Next; or

-	if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

-	if t-WinAdvance is running:

-   if RX_Next_Highest = RX_Next; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	stop and reset t-WinAdvance.

-	if t-WinAdvance is not running (includes the case t-WinAdvance is stopped due to actions above):

-   if RX_Next_Highest > RX_Next + 1 and at least one RLC SDU has been delivered to upper layer with SN > RX_NEXT; or

-	start t-WinAdvance.

##### 5.2.3.2.4	Actions when t-Reassembly expires

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received;

-	if RX_Next_Highest> RX_Highest_Status +1: or

-	if RX_Next_Highest = RX_Highest_Status + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Highest_Status before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

NEXT CHANGE

##### 5.2.3.2.4	Actions when t-WinAdvance expires

When t-WinAdvance expires, the receiving side of an AM RLC entity shall:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received:

-	start t-WinAdvance.

NEXT CHANGE

### 5.3.4	Status reporting

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them).

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an AMD PDU

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires.

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer.

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU or the skipped SN due to t-WinAdvance expiry which is not indicated as missing in the resulting STATUS PDU.

END OF CHANGES