3GPP TSG-RAN WG2 Meeting #128	R2-2410259

Orlando, USA, 18 – 22 November 2024

Agenda item:	7.0.2.10

Source:	Nokia

Title:	SSBless handling

WID/SID:	Netw_Energy_NR-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#127bis (Hefei) RAN2 agreed following regarding SSBless handling in release 18:

Agreements

1	 From RAN2 perspective, no benefit is identified for configuring servingCellMO for inter-band SSB-less SCell.

2	 No LS is sent to R4.

3	Discuss in the next meeting on whether to capture “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO, depending on R4 progress.

4	Postpone the discussion on intra-band SSB-less SCell.

In this paper we consider this from RAN4 requirements point of view and possible implications to RAN2.

# 2	Neighbor cell measurement on SSB-less SCell carrier

In RAN4#112 meeting, the neighbor cell measurement on the carrier of SSB-less SCell was discussed but could not conclude in terms of intra-frequency vs. inter-frequency and gaps vs. without gaps issues.

Issue 1-1-4: Neighbour cells on carrier of SSB-less SCell FFS: · Intra-frequency/Inter-frequency definition: o Inter-frequency based on existing definition. No impact to spec about the intra-f and inter-f definition. · No spec impact in terms of inter- or intra- frequency, and with or without gap.

Once an SCell has been configured (added), the SCell is a serving cell. The configured SCell is a serving cell no matter if the SCell is activated, dormant or deactivated. For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 of 38.133 applies.

Observation #1: For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 of 38.133 applies.

According to current definition of intra-frequency measurement, it is defined based on the center frequency of the SSBs in the serving cell and neighbor cell. As there is no SSB in the serving cell (SSB-less SCell), the current intra-frequency measurement definition does not very well cover the SSB-less SCell scenario. Hence, it was considered whether the measurement requirements applicable for an SSB-less serving should be intra-frequency or inter-frequency measurement requirements.

Observation #2: The current intra-frequency measurement definition does not account when the serving cell is an SSB-less SCell. Hence, it is not clear if it does fulfill the conditions for intra-frequency measurement as defined in TS 38.133.

However, the SSB-less SCell is a serving cell hence the carrier of SSB-less SCell is a serving CC. Although there is no SSB transmission on the SCell. It would be beneficial to consider neiggbour cell measurements in the serving cell carier to be intra-frequency ones – Anyway UE is able to measure those without interruptions.

Observation #3: It would be beneficial to consider serving cell frequency measurements as intra-frequency to allow fluent mobility for SCell

Signaling allows UE to be configured with servingCellMO on the carrier of SSB-less SCell and the neighbor cell measurement in this MO shall be considered as neighbors on a serving carrier and therefore possibility to be measured according to intra-frequency measurement requirements. In particular, in the MO indicated by servingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine the intra-frequency or inter-frequency measurement based on the ssbFrequency. As there is no actual SSB being transmitted for the SCell, the ssbFrequency is configured indicating an ARFCN value not associated with any SSB.

Observation #4: The SSB-less SCell is a serving cell and the carrier of SSB-less SCell is a serving CC.

- Proposal 1:In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

In order to capture this in RAN2 we would need to modify conditional presence of ssbFrequency:

Where:

SSBorAssociatedSSB would need to be modified that ssbFrequency field is allowed to be included for SSBless SCell (similarly as for IE referenceCell-r18). In the Annex we provide TP how to do this in very simple way:

# 3	Conclusion

Observation #1: For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 of 38.133 applies.

Observation #2: The current intra-frequency measurement definition does not account when the serving cell is an SSB-less SCell. Hence, it is not clear if it does fulfill the conditions for intra-frequency measurement as defined in TS 38.133.

Observation #3: It would be beneficial to consider serving cell frequency measurements as intra-frequency to allow fluent mobility for SCell

Observation #4: The SSB-less SCell is a serving cell and the carrier of SSB-less SCell is a serving CC.

- Proposal 1:In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

# 3	Annex: TP

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements and/or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEASOBJECTNR-START

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB2

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

excludedCellsToRemoveList           PCI-RangeIndexList                                              OPTIONAL,   -- Need N

excludedCellsToAddModList           SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

allowedCellsToRemoveList            PCI-RangeIndexList                                              OPTIONAL,   -- Need N

allowedCellsToAddModList            SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

...,

[[

freqBandIndicatorNR                 FreqBandIndicatorNR                                             OPTIONAL,   -- Need R

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

]],

[[

smtc3list-r16                       SSB-MTC3List-r16                                                OPTIONAL,   -- Need R

rmtc-Config-r16                     SetupRelease {RMTC-Config-r16}                                  OPTIONAL,   -- Need M

t312-r16                            SetupRelease { T312-r16 }                                       OPTIONAL    -- Need M

]],

[[

associatedMeasGapSSB-r17            MeasGapId-r17                                                   OPTIONAL,   -- Need R

associatedMeasGapCSIRS-r17          MeasGapId-r17                                                   OPTIONAL,   -- Need R

smtc4list-r17                       SSB-MTC4List-r17                                                OPTIONAL,   -- Need R

measCyclePSCell-r17                 ENUMERATED {ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, spare1}

OPTIONAL,   -- Cond SCG

cellsToAddModListExt-v1710          CellsToAddModListExt-v1710                                      OPTIONAL    -- Need N

]],

[[

associatedMeasGapSSB2-v1720         MeasGapId-r17                                               OPTIONAL, -- Cond AssociatedGapSSB

associatedMeasGapCSIRS2-v1720       MeasGapId-r17                                               OPTIONAL  -- Cond AssociatedGapCSIRS

]],

[[

measSequence-r18                    MeasSequence-r18                                                OPTIONAL,   -- Need R

cellsToAddModListExt-v1800          CellsToAddModListExt-v1800                                      OPTIONAL    -- Need N

]]

}

SSB-MTC3List-r16::=                 SEQUENCE (SIZE(1..4)) OF SSB-MTC3-r16

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

T312-r16 ::=                        ENUMERATED { ms0, ms50, ms100, ms200, ms300, ms400, ms500, ms1000}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                       SetupRelease { SSB-ToMeasure }                                  OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                             OPTIONAL,   -- Need M

...,

[[

ssb-PositionQCL-Common-r16              SSB-PositionQCL-Relation-r16                                OPTIONAL,   -- Cond SharedSpectrum

ssb-PositionQCL-CellsToAddModList-r16   SSB-PositionQCL-CellsToAddModList-r16                       OPTIONAL,   -- Need N

ssb-PositionQCL-CellsToRemoveList-r16   PCI-List                                                    OPTIONAL    -- Need N

]],

[[

deriveSSB-IndexFromCellInter-r17    ServCellIndex                                                   OPTIONAL,   -- Need R

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                                OPTIONAL,   -- Cond SharedSpectrum2

ssb-PositionQCL-Cells-r17           SetupRelease {SSB-PositionQCL-CellList-r17}                     OPTIONAL    -- Need M

]],

[[

cca-CellsToAddModList-r17           PCI-List                                                        OPTIONAL,   -- Need N

cca-CellsToRemoveList-r17           PCI-List                                                        OPTIONAL    -- Need N

]],

[[

ssb-ToMeasureAltitudeBasedList-r18  SetupRelease { SSB-ToMeasureAltitudeBasedList-r18 }             OPTIONAL    -- Need M

]]

}

Q-OffsetRangeList ::=               SEQUENCE {

rsrpOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrqOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

sinrOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrpOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

rsrqOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

sinrOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0

}

ThresholdNR ::=                     SEQUENCE{

thresholdRSRP                       RSRP-Range                                                      OPTIONAL,   -- Need R

thresholdRSRQ                       RSRQ-Range                                                      OPTIONAL,   -- Need R

thresholdSINR                       SINR-Range                                                      OPTIONAL    -- Need R

}

CellsToAddModList ::=               SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddModListExt-v1710 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1710

CellsToAddModListExt-v1800 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1800

CellsToAddMod ::=                   SEQUENCE {

physCellId                          PhysCellId,

cellIndividualOffset                Q-OffsetRangeList

}

CellsToAddModExt-v1710 ::=          SEQUENCE {

ntn-PolarizationDL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL,  -- Need R

ntn-PolarizationUL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL   -- Need R

}

CellsToAddModExt-v1800 ::=          SEQUENCE {

ntn-NeighbourCellInfo-r18           NTN-NeighbourCellInfo-r18                                       OPTIONAL   -- Cond NeighbourCell

}

RMTC-Config-r16 ::=                 SEQUENCE {

rmtc-Periodicity-r16                ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r16             INTEGER(0..639)                                                 OPTIONAL,   -- Need M

measDurationSymbols-r16             ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60},

rmtc-Frequency-r16                  ARFCN-ValueNR,

ref-SCS-CP-r16                      ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP},

...,

[[

rmtc-Bandwidth-r17                  ENUMERATED {mhz100, mhz400, mhz800, mhz1600, mhz2000}           OPTIONAL,   -- Need R

measDurationSymbols-v1700           ENUMERATED {sym140, sym560, sym1120}                            OPTIONAL,   -- Need R

ref-SCS-CP-v1700                    ENUMERATED {kHz120, kHz480, kHz960}                             OPTIONAL,   -- Need R

tci-StateInfo-r17               SEQUENCE {

tci-StateId-r17                  TCI-StateId,

ref-ServCellId-r17               ServCellIndex                                                  OPTIONAL   -- Need R

} OPTIONAL -- Need R

]],

[[

ref-BWPId-r17                   BWP-Id                                                              OPTIONAL   -- Need R

]]

}

SSB-PositionQCL-CellsToAddModList-r16 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-CellsToAddMod-r16

SSB-PositionQCL-CellsToAddMod-r16 ::= SEQUENCE {

physCellId-r16                        PhysCellId,

ssb-PositionQCL-r16                   SSB-PositionQCL-Relation-r16

}

SSB-PositionQCL-CellList-r17 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-Cell-r17

SSB-PositionQCL-Cell-r17         ::= SEQUENCE {

physCellId-r17                        PhysCellId,

ssb-PositionQCL-r17                   SSB-PositionQCL-Relation-r17

}

SSB-ToMeasureAltitudeBasedList-r18 ::= SEQUENCE (SIZE (1..maxNrofAltitudeRanges-r18)) OF SSB-ToMeasureAltitudeBased-r18

SSB-ToMeasureAltitudeBased-r18 ::=     SEQUENCE {

altitudeRange-r18                      SEQUENCE {

altitudeMin-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeMax-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeHyst-r18                       HysteresisAltitude-r18                                   OPTIONAL   -- Need R

},

ssb-ToMeasure-r18                      SSB-ToMeasure                                                OPTIONAL   -- Need S

}

NTN-NeighbourCellInfo-r18 ::=          SEQUENCE {

epochTime-r18                          EpochTime-r17,

ephemerisInfo-r18                      EphemerisInfo-r17,

referenceLocation-r18                  ReferenceLocation-r17                                        OPTIONAL   -- Need R

}

-- TAG-MEASOBJECTNR-STOP

-- ASN1STOP

CellsToAddMod field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific cell.

physCellId Physical cell identity of a cell in the cell list.

MeasObjectNR field descriptions

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per CSI-RS resource as described in 5.5.5.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per SS/PBCH block index as described in 5.5.5.2.

allowedCellsToAddModList List of cells to add/modify in the allow-list of cells. It applies only to SSB resources.

allowedCellsToRemoveList List of cells to remove from the allow-list of cells.

associatedMeasGapSSB Indicates the associated measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR. If this field is absent, the associated measurement gap is the gap configured via gapFR1, gapFR2, or gapUE.

associatedMeasGapSSB2 Indicates the associated additional measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object for NTN deployments. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR. If this field is absent, the associated measurement gap is the gap indicated by associatedMeasGapSSB.

associatedMeasGapCSIRS Indicates the associated measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object. If this field is absent, the associated measurement gap is the gap configured via gapFR1, gapFR2, or gapUE.

associatedMeasGapCSIRS2 Indicates the associated additional measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object for NTN deployments. If this field is absent, the associated measurement gap is the gap indicated by associatedMeasGapCSIRS. In this release of the specification, this field is not configured for NTN deployments.

cellsToAddModList List of cells to add/modify in the cell list. If the network includes cellsToAddModListExt-v1710 and/or cellsToAddModListExt-v1800, it contains the same number of entries listed in the same order as in cellsToAddModList (i.e without suffix).

cellsToRemoveList List of cells to remove from the cell list.

excludedCellsToAddModList List of cells to add/modify in the exclude-list of cells. It applies only to SSB resources.

excludedCellsToRemoveList List of cells to remove from the exclude-list of cells.

freqBandIndicatorNR The frequency band in which the SSB and/or CSI-RS indicated in this MeasObjectNR are located and according to which the UE shall perform the RRM measurements. This field is always provided when the network configures measurements with this MeasObjectNR.

measCyclePSCell The parameter is used only when the PSCell is configured on the frequency indicated by the measObjectNR and the SCG is deactivated, see TS 38.133 [14]. The field may also be configured when the PSCell is not configured on that frequency. The network always configures measCyclePSCell for the measObjectNR associated with the PSCell if bfd-and-RLM is set to true and the SCG is deactivated. Value ms160 corresponds to 160 ms, value ms256 corresponds to 256 ms and so on.

measCycleSCell The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

measSequence Indicates the recommended sequence for intra/inter-RAT intra/inter-frequency measurement. Value 1 means the corresponding frequency is measured firstly. Value 2 means the corresponding frequency is measured secondly and so on. If more than one frequency is configured with the same value, it means no recommended sequence among these frequencies. If not provided, it means there is no recommended sequence for the corresponding frequency. This field is only configured for NR standalone or if the measObject is associated to the MCG.

nrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell associated with this MeasObjectNR.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell associated with this MeasObject.

ntn-NeighbourCellInfo Includes satellite assistance information of an NTN neighbour cell.

ntn-PolarizationDL If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

ntn-PolarizationUL If present, this parameter indicates polarization information for uplink transmission on service link. If not present and ntn-PolarizationDL is present, UE assumes the same polarization for UL and DL.

offsetMO Offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR.

quantityConfigIndex Indicates the n-th element of quantityConfigNR-List provided in MeasConfig.

referenceSignalConfig RS configuration for SS/PBCH block and CSI-RS.

refFreqCSI-RS Point A which is used for mapping of CSI-RS to physical resources according to TS 38.211 [16] clause 7.4.1.5.3.

smtc1 Primary measurement timing configuration. (see clause 5.5.2.10).

smtc2 Secondary measurement timing configuration for SS corresponding to this MeasObjectNR with PCI listed in pci-List. For these SS, the periodicity is indicated by periodicity in smtc2 and the timing offset is equal to the offset indicated in periodicityAndOffset modulo periodicity. periodicity in smtc2 can only be set to a value strictly shorter than the periodicity indicated by periodicityAndOffset in smtc1 (e.g. if periodicityAndOffset indicates sf10, periodicity can only be set of sf5, if periodicityAndOffset indicates sf5, smtc2 cannot be configured).

smtc3list Measurement timing configuration list for SS corresponding to IAB-MT. This is used for the IAB-node's discovery of other IAB-nodes and the IAB-Donor-DUs.

smtc4list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10.

ssbFrequency Indicates the frequency of the SS associated to this MeasObjectNR. For operation with shared spectrum channel access, this field is a k*30 kHz shift from the sync raster where k = 0,1,2, and so on if the reportType within the corresponding ReportConfigNR is set to reportCGI (see TS 38.211 [16], clause 7.4.3.1). Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15], or TS 38.101-5 [75]).

ssb-PositionQCL-Common Indicates the QCL relationship between SS/PBCH blocks for all measured cells as specified in TS 38.213 [13], clause 4.1.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

t312 The value of timer T312. Value ms0 represents 0 ms, ms50 represents 50 ms and so on.

ReferenceSignalConfig field descriptions

csi-rs-ResourceConfigMobility CSI-RS resources to be used for CSI-RS based RRM measurements.

ssb-ConfigMobility SSB configuration for mobility (nominal SSBs, timing configuration).

RMTC-Config field descriptions

measDurationSymbols Number of consecutive symbols for which the Physical Layer reports samples of RSSI (see TS 38.215 [9], clause 5.1.21). Value sym1 corresponds to one symbol, sym14or12 corresponds to 14 symbols of the reference numerology for NCP and 12 symbols for ECP, and so on. If measDurationSymbols-v1700 is signalled, the UE ignores measDurationSymbols-r16.

ref-BWPId Indicates the reference BWP for the TCI state indicated in tci-StateInfo. Network includes this field if tci-StateInfo is present. This field is only applicable for operation with shared spectrum channel access in FR2-2 and network does not configure this if the UE does not have any serving cells in FR2-2.

ref-SCS-CP Indicates a reference subcarrier spacing and cyclic prefix to be used for RSSI measurements (see TS 38.215 [9]). Value kHz15 corresponds to 15kHz, kHz30 corresponds to 30 kHz, value kHz60-NCP corresponds to 60 kHz using normal cyclic prefix (NCP), and kHz60-ECP corresponds to 60 kHz using extended cyclic prefix (ECP). If ref-SCS-CP-v1700 is signalled, the UE ignores ref-SCS-CP-r16.

ref-ServCellId Indicates the FR2-2 reference serving cell index for the TCI state. Network includes this field if tci-StateInfo is present. This field is only applicable for operation with shared spectrum channel access in FR2-2 and network does not configure this if the UE does not have any serving cells in FR2-2.

rmtc-Bandwidth Indicates the bandwidth for the RSSI measurement (see TS 38. 215 [9], clause 5.1.21).

rmtc-Frequency Indicates the center frequency of the measured bandwidth for a frequency which operates with shared spectrum channel access (see TS 38. 215 [9], clause 5.1.21).

rmtc-Periodicity Indicates the RSSI measurement timing configuration (RMTC) periodicity (see TS 38.215 [9], clause 5.1.21).

rmtc-SubframeOffset Indicates the RSSI measurement timing configuration (RMTC) subframe offset for this frequency (see TS 38.215 [9], clause 5.1.21). For inter-frequency measurements, this field is optional present and if it is not configured, the UE chooses a random value as rmtc-SubframeOffset for measDurationSymbols which shall be selected to be between 0 and the configured rmtc-Periodicity with equal probability.

tci-StateId Indicates the TCI state to be used for RSSI measurements. This field is only applicable for shared spectrum channel access in FR2-2. Network does not configure this if the UE does not have any serving cells in FR2-2 and in such a case, it is up to UE implementation how to determine the spatial domain filter for the inter-frequency RSSI measurement in FR2-2.

SSB-ConfigMobility field descriptions

cca-CellsToAddModList, cca-CellsToRemoveList Lists of cells to be added or removed from the list of neighbor cells that apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2.

deriveSSB-IndexFromCell If this field is set to true, UE assumes SFN and frame boundary alignment across cells on the same frequency carrier as specified in TS 38.133 [14]. Hence, if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, this field indicates whether the UE can utilize the timing of this serving cell to derive the index of SS block transmitted by neighbour cell. Otherwise, this field indicates whether the UE may use the timing of any detected cell on that target frequency to derive the SSB index of all neighbour cells on that frequency.

deriveSSB-IndexFromCellInter If this field is present, UE assumes SFN and frame boundary alignment between the reference serving cell indicated by ServCellIndex and all neighbour cells in this MeasObjectNR as specified in TS 38.133 [14]. This field also indicates that the UE can utilize the timing of the reference serving cell indicated by ServCellIndex to derive the index of SS block transmitted by all inter-frequency neighbour cells on the frequency indicated by the MeasObjectNR. When this field is included, the network should set deriveSSB-IndexFromCell to true.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not to be measured while value 1 indicates that the corresponding SS/PBCH block is to be measured (see TS 38.215 [9]). When the field is not configured the UE measures on all SS blocks. Regardless of the value of this field, SS/PBCH blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

ssb-ToMeasureAltitudeBasedList List of altitude-dependent ssb-ToMeasure. When the UE is within an altitude range indicated by altitudeRange, it ignores the ssb-ToMeasure (without suffix), and applies the corresponding ssb-ToMeasure-r18 if present, otherwise (i.e., the UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies. For each altitude range, altitudeMin indicates the minimum altitude in meters relative to sea level, altitudeMax indicates the maximum altitude in meters relative to sea level, and if included, altitudeHyst indicates hysteresis in meters for determination of the altitude range. I.e., when altitudeHyst is configured for an altitude range, the UE considers itself to have entered the range if altitudeMin ≤ UE altitude ≤ altitudeMax and after entering the range considers itself to be in the range while (altitudeMin – altitudeHyst) ≤ UE altitude ≤ (altitudeMax + altitudeHyst). For each altitudeRange, if altitudeMin is absent, value minAltitude-r18 is used and if altitudeMax is absent, value maxAltitude-r18 is used.

SSB-PositionQCL-CellsToAddMod field descriptions

physCellId Physical cell identity of a cell in the cell list.

ssb-PositionQCL Indicates the QCL relation between SS/PBCH blocks for a specific cell as specified in TS 38.213 [13], clause 4.1. If provided, the cell specific value overwrites the value signalled by ssb-PositionQCL-Common.

| Conditional Presence | Explanation |
| --- | --- |
| AssociatedGapCSIRS | This field is optionally present, Need R if associatedMeasGapCSIRS is configured, otherwise, it is absent. |
| AssociatedGapSSB | This field is optionally present, Need R if associatedMeasGapSSB is configured, otherwise, it is absent. |
| CSI-RS | This field is mandatory present if csi-rs-ResourceConfigMobility is configured, otherwise, it is absent. |
| IntraFreqConnected | This field is optionally present, Need R if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, otherwise, it is absent. |
| SCG | This field is optionallly present, Need R, in the measConfig associated with the SCG. It is absent in the measConfig associated with the MCG. |
| NeighbourCell | This field is mandatory present if this MeasObject is configured by the serving cell for a neighbour cell served by a NTN Earth-moving cell and is associated with a ReportConfig which contains EventD2 or condEventD2. Otherwise, it is optional, Need R. |
| SharedSpectrum | This field is mandatory present if this MeasObject is for a frequency which operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this MeasObject is for a frequency which operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |
| SSBorAssociatedSSB | This field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. Otherwise, it is absent, Need R. |
| SSBorAssociatedSSB2 | This field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. And optionally present in case of measurement object is associated to SSBless SCell. Otherwise, it is absent, Need R. |