3GPP TSG-RAN WG2 #128	R2-2410263

Orlando, US, 18th – 22nd November 2024

Agenda Item:	8.3.5 Other

Source:	Ericsson

Title:	Discussion on generalization aspects

Document for:	Discussion

# 1	Introduction

In RAN2#125bis and RAN2#127bis meetings, the following agreements have been taken on generalization:

- AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found

- Discussion on what (type of information)/how generalization study can take place in Nov. meeting

In this contribution, we will discuss some aspects related to the generalization of the AI/ML model for the RRM measurement prediction.

# 2	Discussion

An AI/ML model may provide good performance in scenarios it is trained for, but the same level of performance may not be observed when the same model is used for inferencing in scenarios where the input data set is different compared to what was trained for. The differences in the data sets may be related to different aspects, such as speed, frequency, number of cells, area etc. A question that arises is in which cases generalizations can be made, i.e. when it can be assumed that a model is still valid, even though the inference scenario is not the same as the scenario the model was trained for. The generalization cases for evaluation include that the inference dataset is the same as the training dataset (the baseline case) and that the inference dataset is different than the training dataset.

Generalization cases for evaluation include:

- Baseline case: Model is trained based on a training dataset, and the inference is based on a dataset with the same assumptions.

    - Generalization case: Model is trained based on a training dataset, and the inference is based on a dataset with different assumptions (e.g. different UE speeds or frequencies).

For verification purpose, the performance of the generalization case is evaluated by comparing it with the performance of the baseline case.

Performance of the generalization case is compared with the baseline, for evaluation purposes.

One aspect that may be considered for generalization is the UE speed. AI/ML models are likely not trained for a large number of various speeds and it would be useful to be able to have UE speed as one parameter that could be considered for generalization. It is proposed to study the suitability of UE speed as a parameter for generalization.

Another aspect that could be considered for generalization is the frequency, where the model is trained for a certain frequency, but inference is done using a different frequency. Due to issues with inter-frequency measurements, such as long delays and impact on performance, it would be a very useful case if generalization could be made related to frequencies and this scenario should be further studied.

Another scenario to investigate for generalization is whether a model that was trained for UEs deployed in small cells could work sufficiently well for UEs deployed in larger cells, or vice versa.

For generalization purposes, the following parameters can be prioritized in the study:

    - UE speed: the AI/ML model is trained using a certain UE speed and the AI/ML model performs inference for a UE with a different speed, including stationary UEs.

    - Frequency: The AI/ML model is trained using a certain frequency, but the inference is done using another frequency.

    - Cell size: the AI/ML model is trained for small cells, but the inference is done for large cells.

For generalization evaluation purposes, many values can be used for the training and the inference dataset for the parameters listed above. In this respect, the detailed parameter values need to be agreed upon, in order to limit the number of simulations. In the following, detailed parameter values are proposed.

For the UE speed, the AI/ML model is trained using a certain UE speed (e.g. 30 km/h) and then the AI/ML model performs inference for a UE with different speed (e.g. 120 km/h), or vice versa. Additionally, the input for training can include some speeds (e.g. 60 km/h and 120 km/h) and the inference can be performed to another speed (e.g. 90 km/h). For generalization related to speed, it would be also useful to consider stationary UEs for training and inference. Therefore, it is proposed that RAN2 considers one additional value for speed, i.e. 0 km/h.

Consider the additional speed, 0 km/h, to include stationary UEs for generalization purposes.

For generalization evaluation over UE speeds:

    - Training dataset is 30 km/h and inference dataset is 90 km/h

    - Training dataset is 90 km/h and inference dataset is 30 km/h

    - Training dataset is 0 km/h and inference dataset is 60 km/h

    - Training dataset is 60 km/h and inference dataset is 0 km/h

    - Training dataset is (30 km/h, 90 km/h) and inference dataset is 60 km/h

    - Training dataset is (60 km/h, 120 km/h) and inference dataset is 90 km/h

For inter-frequency measurement predictions, RAN2 has previously agreed on two frequencies: 2 GHz and 4 GHz. For generalization related to frequency predictions, it would be useful to have the option of training with two frequencies and predict a third frequency, e.g. a frequency in-between the frequencies used for the training. Therefore, it is proposed that RAN2 considers one additional frequency for training, e.g. 0.8 GHz.

Consider the frequency 0.8 GHz to be used for inter-frequency predictions training.

For generalization evaluation over carrier frequencies:

    - Training dataset is 0,8 GHz and 4 GHz, and inference dataset is 2 GHz.

    - Training dataset is 2 GHz and inference dataset is 4 GHz.

    - Training dataset is 4 GHz and inference dataset is 2 GHz.

Similarly, for generalization related to the size of the cell, the training can be done using small cells and the inference using large cells and vice versa. This is to cover the case that the training was done in an area with a certain type of cells, but the inference is done in an area with a different type of cells.

For generalization evaluation over cell size:

    - Training dataset is small cells, and inference dataset is large cells.

    - Training dataset is large cells and inference dataset is small cells.

The performance of the generalization of the AI/ML model can be evaluated by doing simulations using parameter values (e.g. speed, frequency, etc.) which the model was not trained for. Since this study is focusing on generalization of the AI/ML model for RRM measurement predictions, the simulation assumptions agreed for the RRM measurement predictions can be reused for the generalization purpose.

The companies can report the configuration and the values of the parameters used in the simulations for the generalization verification.

The current simulation assumptions for the RRM measurement predictions can be reused when studying the AI/ML model generalization.

Companies can report the selected configurations and values used in generalization verification.

# Conclusion

In the previous sections we made the following proposals:

Proposal 1	Generalization cases for evaluation include:

	Baseline case: Model is trained based on a training dataset, and the inference is based on a dataset with the same assumptions.

	Generalization case: Model is trained based on a training dataset, and the inference is based on a dataset with different assumptions (e.g. different UE speeds or frequencies).

Proposal 2	Performance of the generalization case is compared with the baseline, for evaluation purposes.

Proposal 3	For generalization purposes, the following parameters can be prioritized in the study:

	UE speed: the AI/ML model is trained using a certain UE speed and the AI/ML model performs inference for a UE with a different speed, including stationary UEs.

	Frequency: The AI/ML model is trained using a certain frequency, but the inference is done using another frequency.

	Cell size: the AI/ML model is trained for small cells, but the inference is done for large cells.

Proposal 4	Consider the additional speed, 0 km/h, to include stationary UEs for generalization purposes.

Proposal 5	For generalization evaluation over UE speeds:

	Training dataset is 30 km/h and inference dataset is 90 km/h

	Training dataset is 90 km/h and inference dataset is 30 km/h

	Training dataset is 0 km/h and inference dataset is 60 km/h

	Training dataset is 60 km/h and inference dataset is 0 km/h

	Training dataset is (30 km/h, 90 km/h) and inference dataset is 60 km/h

	Training dataset is (60 km/h, 120 km/h) and inference dataset is 90 km/h

Proposal 6	Consider the frequency 0.8 GHz to be used for inter-frequency predictions training.

Proposal 7	For generalization evaluation over carrier frequencies:

	Training dataset is 0,8 GHz and 4 GHz, and inference dataset is 2 GHz.

	Training dataset is 2 GHz and inference dataset is 4 GHz.

	Training dataset is 4 GHz and inference dataset is 2 GHz.

Proposal 8	For generalization evaluation over cell size:

	Training dataset is small cells, and inference dataset is large cells.

	Training dataset is large cells and inference dataset is small cells.

Proposal 9	The current simulation assumptions for the RRM measurement predictions can be reused when studying the AI/ML model generalization.

Proposal 10	Companies can report the selected configurations and values used in generalization verification.

# References

RP-242393,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR