3GPP TSG-RAN WG2 Meeting #128 	R2-2410295

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.2.3

Source:	Panasonic

Title:	Discussion on A-IoT paging

Document for:	Discussion

# Introduction

In last meeting, RAN2 achieved the following agreements on A-IoT paging.

Agreements on AIoT paging

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.  RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.

In this paper, we further discuss remaining issues related to skip response to paging due to energy constraint and multi-reader scenario.

# Discussion

## Skip response to paging due to energy constraint

In previous meeting, energy status reporting has been discussed, and the conclusion is to capture one-bit indication in the TR. If the device does not have sufficient energy to perform the remaining or follow-up procedure, the device can send this one-bit energy status indication to reader in a D2R message (e.g. MSG1, MSG3 and command response message).

However, if device can identify lack of energy for the follow-up communication, another option that needs to be considered is that the device can simply skip responding to the paging in the first place, since sending a D2R message itself also consumes non-negligible amount of energy and negatively impacts the energy harvesting at the device. Therefore, a more important aspect is that the A-IoT device should be allowed to skip responding if the device does not have sufficient energy to complete the remaining access procedure triggered by the paging. Whether the energy is sufficient can be judged by device implementation. This procedure can be illustrated using Fig.1. below.

Fig.1. Device skips responding based on judgement of its energy

Proposal 1. Device is allowed to skip responding if device judges its remaining energy cannot sustain to complete whole access procedure triggered by this A-IoT paging.

## Multi-reader area

In previous meeting, it has been agreed that the A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. Based on this information the device determines whether to skip sending the response to paging.

On the “short and simple” information, the following options have been mentioned in the discussion:

- (Partial) session ID or service ID from CN

- Transaction ID generated by reader

- NDI-like indication generated by reader

It has been decided to wait for further information and requirements from other working groups and make this decision in normative phase.

On the other hand, we would like to point out that information discussed above is not sufficient for a multi-reader scenario. For example, a device can be located in a multi-reader overlapping area. See Fig.2. as an example.

Fig.2. Multi-reader overlapping area

In this case, device may receive paging messages from different readers with the same or different the above-mentioned ID information, e.g. a transaction ID. Assuming no coordination among readers, it may happen that the paging transactions from different readers are interleaved, as illustrated in the following Fig.3.

Fig.3. Paging from multiple readers

In the scenario shown in Fig.3, even with transaction ID, the device may still need to respond to repeated paging messages because the device sees every paging as a new transaction compared to what has been kept in the device’s memory (assuming device only stores one transaction ID due to power/complexity constraint). Since A-IoT does not have a serving cell concept, it seems that some rule needs to be define for device to know the device-reader association. With such association, device will respond only to the corresponding reader until such association is changed. For example, the association should be regulated by a timer to avoid ping-pong. Before timer expires, the device only responds to the paging from the same associated reader, and ignore the paging from other readers regardless of the same or different session/service/transaction ID.

Observation 1. In a multi-reader overlapping area, using session/service/transaction ID alone may not avoid duplicated responses from the device.

Proposal 2. RAN2 to further discuss the need to introduce device-reader association, e.g. to introduce reader ID in the paging.

Proposal 3. RAN2 to further discuss whether/how device-reader association is kept for a certain time.

# Conclusion

In this paper, we have discussed some remaining issues on paging. We have the following observations and proposals.

Skip response to paging due to energy constraint

Proposal 1. Device is allowed to skip responding if device judges its remaining energy cannot sustain to complete whole access procedure triggered by this A-IoT paging.

Multi-reader scenarios

Observation 1. In a multi-reader overlapping area, using session/service/transaction ID alone may not avoid duplicated responses from the device.

Proposal 2. RAN2 to further discuss the need to introduce device-reader association, e.g. to introduce reader ID in the paging.

Proposal 3. RAN2 to further discuss whether/how device-reader association is kept for a certain time.