3GPP TSG-RAN WG2 Meeting #128	R2-2410298

Orlando, US, 18-22 August, 2024

Agenda Item	: 8.13.2

Source	: LG Electronics Inc.

Title	: Discussion on Relay discovery and selection for multi-hop U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we propose the condition to be an intermediate (/first) L2/L3 U2N Relay UE. We investigate under which condition the Discovery message is forwarded to support multi-hop relay.

2.	Discussion

2.1 Uu-RSRP threshold

In the Rel-17 U2N Relay, the Remote UE has an Uu-RSRP threshold restriction. If Remote UE has a Uu-RSRP value below the threshold configured via SIB, pre-configuration, or dedicated RRC, the Remote UE can attempt to connect to the U2N Relay. The Remote UE with week Uu-RSRP needs to be supported by U2N Relay UE to have connection with gNB.

For the multi-hop relay, the intermediate U2N Relay UE and U2N Remote UE can be configured with the same Uu-RSRP threshold restriction as the legacy Rel-17 U2N Remote UE. The U2N Remote UE reaches to the gNB via the intermediate U2N Relay UE and the last U2N Relay UE. The intermediate U2N Relay UE doesn’t need to have a better Uu-RSRP value than the last U2N Relay UE. Also, the intermediate U2N Relay UE doesn’t need to have a strong Uu-RSRP value because it is connected to the gNB via the last U2N Relay UE.  The reason why Rel-17 U2N Relay UE has an upper bound on Uu-RSRP is to prevent signalling interference due to relay operation. The same reason can be applied to the intermediate Relay UE.

Proposal 1: The same threshold used for Rel-17/18 U2N Remote UE can be configured for the intermediate U2N Relay UE and U2N Remote UE in the multi-hop. The Uu threshold for the intermediate U2N Relay UE and U2N Remote UE can be below a threshHighRemote.

Each intermediate U2N Relay UE and U2N Remote UE can exist in the same/different cell as the serving/camp-on cell of the last U2N Relay UE. The intermediate U2N Relay UE, and U2N Remote UE can be out-of-coverage. However, after multi-hop U2N connection via the last U2N Relay UE, the intermediate U2N Relay UE and the U2N Remote UE consider the serving/camp-on cell of the last U2N Relay UE as their own cell. This is the same principle as the legacy Rel-17 U2N Relay operation.

Proposal 2: The last U2N Relay UE, intermediate U2N Relay UE, and the U2N Remote UE can be in the same or different cells before the U2N connection.

Proposal 3: The intermediate U2N Relay UE and the U2N Remote UE can be in OoC.

If the intermediate U2N Relay UE is in a different cell as the cell of the last U2N Relay UE’s cell, the intermediate U2N Relay UE should follow the configuration of its cell before establishing a U2N connection via the last U2N Relay UE. If the intermediate U2N Relay UE’s cell does not support multi-hop relay operation, the intermediate U2N Relay UE cannot act as an intermediate U2N Relay UE. If the cell of the intermediate U2N Relay UE supports multi-path relay operation but does not configure the Uu-RSRP threshold, the intermediate U2N Relay UE can act as an intermediate U2N Relay UE without checking its own the Uu-RSRP. However, if the intermediate U2N Rely UE’cell supports multi-hop relay operation and configures the Uu-RSRP threshold, the intermediate U2N Relay UE can act as an intermediate U2N Relay UE only if the Uu-RSRP threshold of its cell is met.

Proposal 4: If the intermediate U2N Relay UE is in a different cell from the last U2N Relay UE, the intermediate U2N Relay UE can act as an intermediate U2N Relay UE as long as the cell supports multi-hop relay operation or the Uu-RSRP meets the threshold configured by the cell of the intermediate U2N Relay UE.

Proposal 5: If the intermediate U2N Relay UE is in a different cell from the last U2N Relay UE, the intermediate U2N Relay UE cannot act as an intermediate U2N Relay UE as long as intermediate Relay UE’s cell does not support multi-hop relay operation.

2.2 PC5-RSRP threshold

In the long email discussion “[Post127][401][Relay] Multi-hop relay discovery and (re)selection”, RAN2 was discussed whether PC5-RSRP criteria are required at the last Relay UE. If we follow Rel-18 U2U relay operation, the source Remote UE checks the PC5-RSRP threshold before sending the response discovery message. However, if we follow Rel-17 U2N relay operation, the U2N Relay UE does not check the PC5-RSRP threshold before sending the response/announcement discovery message.

In the case of multi-hop U2N Relay operation, as the number of hops increases, the link reliability may become vulnerable. The link quality can be changed over time. Even if the SL quality is quite good when sending solicitation message, the SL quality may become bad when receiving the response message due to the time difference between sending the solicitation message and receiving the response message as the hop count is increased. Therefore, we think that double checking in both sides of the link is not a bad choice.

In discovery model A, the announcement message can be checked at the intermediate Relay UE whether the signalling strength is above the configured threshold before forwarding the received announcement message. Also, in discovery model B, it may be better for the last Relay UE to check the link quality before sending the response discovery message. Even we think it could be good to check the link quality before forwarding the response discovery message at the intermediate Relay UE.

Proposal 6: The PC5 RSRP threshold can be checked at the intermediate Relay UE before forwarding the announcement message.

Proposal 7: The PC5 RSRP threshold can be checked at the last Relay/intermediate Relay UE before forwarding the response discovery message.

2.3 Discovery message transmission

If the intermediate U2N Relay UE already has a multi-hop U2N relay connection via the last U2N Relay UE, the intermediate U2N Relay UE should deliver the solicitation message to its connected last U2N Relay UE. An intermediate U2N Relay UE has only one indirect U2N connection. Thus, the intermediate U2N Relay UE, which already has a U2N connection, delivers the solicitation message by unicast, even though the solicitation message is received by broadcast.

Proposal 8: In Discovery model B, if the intermediate U2N Relay UE already has an indirect U2N connection with the gNB via the last U2N Relay UE, the intermediate U2N Relay UE forwards the solicitation message received from the U2N Remote UE only to the connected last U2N Relay UE. This may need to check to the SA2.

3.	Conclusion

Proposal 1: The same threshold used for Rel-17/18 U2N Remote UE can be configured for the intermediate U2N Relay UE and U2N Remote UE in the multi-hop. The Uu threshold for the intermediate U2N Relay UE and U2N Remote UE can be below a threshHighRemote.

Proposal 2: The last U2N Relay UE, intermediate U2N Relay UE, and the U2N Remote UE can be in the same or different cells before the U2N connection.

Proposal 3: The intermediate U2N Relay UE and the U2N Remote UE can be in OoC.

Proposal 4: If the intermediate U2N Relay UE is in a different cell from the last U2N Relay UE, the intermediate U2N Relay UE can act as an intermediate U2N Relay UE as long as the cell supports multi-hop relay operation or the Uu-RSRP meets the threshold configured by the cell of the intermediate U2N Relay UE.

Proposal 5: If the intermediate U2N Relay UE is in a different cell from the last U2N Relay UE, the intermediate U2N Relay UE cannot act as an intermediate U2N Relay UE as long as intermediate Relay UE’s cell does not support multi-hop relay operation.

Proposal 6: The PC5 RSRP threshold can be checked at the intermediate Relay UE before forwarding the announcement message.

Proposal 7: The PC5 RSRP threshold can be checked at the last Relay/intermediate Relay UE before forwarding the response discovery message.

Proposal 8: In Discovery model B, if the intermediate U2N Relay UE already has an indirect U2N connection with the gNB via the last U2N Relay UE, the intermediate U2N Relay UE forwards the solicitation message received from the U2N Remote UE only to the connected last U2N Relay UE. This may need to check to the SA2.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.