3GPP TSG-RAN2 Meeting #128	R2-2410303

Orlando, US, 18th Nov 2024 - 22nd Nov 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SIB1 operation for NES

Agenda Item:	8.5.3

Document for:	Discussion and decision

# Introduction

In the RAN#105 meeting, the revised Work Item for network energy saving enhancement has been approved in RP-242354 to specify the Case 2 based procedure and signaling for on-demand SIB1 operation [1]:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In the last meeting, RAN2 continued the discussion on on-demand SIB1 operation and several agreements have been achieved [2]:

Agreements on OD-SIB1

1.	We will inherit all agreements made during SI phase to WI phase.

2.	A NES cell can include neighbouring NES cell’s WUS configuration.

3.	NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

4.	In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

5.	The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

6.	The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

7.	A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

8.	If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

9.	Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

10.	NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

11.	RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

In this contribution, we will discuss the remaining issues on on-demand SIB1 operation.

# Discussion

## 2.1 How to identify the NES Cell

In RAN1’s discussion, the identification of NES Cell was discussed with the following agreement [3]:

Agreement@RAN1#118bis For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

The WUS configuration from Cell A can be used by the UE to determine a cell supporting on-demand SIB1 operation. When the UE is camped or connected to Cell A, it can obtain the WUS configuration from Cell A. After that, the NES Cell, which supports on-demand SIB1 operation, can be identified by the parameter Physical Cell ID and ARFCN in the WUS configuration. RAN1 also discussed the SSB transmission pattern of the NES Cell. When the SSB is transmitted on the sync raster, the value of K_SSB is restricted to a range value larger than 23 for FR1 and larger than 11 for FR2. As captured in the TS 38.213, such SSB is the so called non-cell defining SSB without Type0-PDCCH CSS set. R19 NES UEs could detect such SSB and determine that the SSB is NCD-SSB. In TS 38.300, the PCell is always associated to a CD-SSB located on the sync raster. Since NES Cell could be a PCell for R19 idle/inactive UEs, how to determine the cell is NES Cell by PBCH is still under RAN1’s discussion. If the solution from RAN1 is feasible, we can follow the solution and change the corresponding UE behaviour in RAN2.

Cited from TS 38.213 V18.4.0 Upon detection of a SS/PBCH block, the UE determines from MIB that a CORESET for Type0-PDCCH CSS set, as described in clause 13, is present if kSSB<24 [4, TS 38.211] for FR1 or if kSSB<12 for FR2. The UE determines from MIB that a CORESET for Type0-PDCCH CSS set is not present if kSSB>23 for FR1 or if kSSB>11 for FR2; the CORESET for Type0-PDCCH CSS set may be provided by PDCCH-ConfigCommon.

Observation 1: NES cell’s state can be derived from the WUS configuration from Cell A.

Proposal 1: Wait for RAN1’s further process on the SSB transmission in NES Cell whether NES cell state can be derived from PBCH.

## 2.2 The transmission behavior of OD-SIB1

After the UE has sent the WUS to trigger the OD-SIB1 and has received the RAR from the NES Cell to indicate that the NES Cell will broadcast OD-SIB1, the next issue is how to broadcast the OD-SIB1. To save the NES Cell’s energy, the periodical broadcast behaviour of OD-SIB1 needs to be avoided. The triggered OD-SIB1 broadcast should last for a specific time. In legacy OD-SI procedure, the si-WindowLength is provided in SIB1 to indicate the transmission of OD-SI. Following such mechanism, the si-Window based design can be reused for OD-SIB1. The SI window for legacy is broadcasted in SIB1. Since we don’t have SIB1 we would need to broadcast it e.g. in msg2 in the NES cell.

Proposal 2: Reuse the SI-window mechanism for the OD-SIB1 procedure.

Agreement@RAN1#118bis RAN1 to discuss the contents of RAR in response to UL WUS using the legacy RAR for OSI as a starting point

RAN1 will discuss the content of the RAR to the WUS signal by considering the legacy RAR for OSI as a starting point. To make sure the SI-window is always up to date for the requesting UE it should be conveyed in RAR. If the NW does not intend to send OD-SIB1 in a specific timeframe, it should be also possible to indicate that SIB1 will not be broadcasted, so that the UE will not repeat the OD-SIB1 request. For that purpose, the RAR should also include status indication of SIB1 transmission. If the NES cell receives the UE request and decides to response negatively, i.e., not ready to send OD-SIB1 at least not in the near future shown in Figure 1, then the UE will benefit from this response to stop its WUS request procedure (power ramp up with retransmissions) creating less interference in the uplink and saving the UE power. If the cell response is positive, i.e., the OD-SIB1 provision will start as shown in Figure ‎2, then the UE could start monitoring the type 0 PDCCH monitoring occasions during the window indicated in the WUS response.

Figure 1: Illustration of a WUS response with sib1-BroadcastStatus = broadcast and a time window for type 0 PDCCH monitoring occasions to receive OD-SIB1.

Figure 2: Illustration of a WUS response with sib1-BroadcastStatus = not broadcast and a time window for no type 0 PDCCH monitoring occasions and no OD-SIB1 reception nor WUS request (re)transmission.

Proposal 3: The RAR responding to OD-SIB1 request should contain at least a time window and OD-SIB1 broadcast status.

In legacy OD-SIBx request procedure, the preamble transmission counter will be increased when the ra-ResponseWindow expires and the RAR has not been received. In last meeting, we have also agreed that the preamble can be transmitted several times. Following the legacy spec, we can reuse to condition to trigger WUS signal retransmission. That means that when the ra-ResponseWindow expires and the RAR has not been received, UE will increase the preamble transmission count until it reached the preamble transmission maximum number.

Cited from TS 38.321 V18.3.0 1> if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received: 2> consider the Random Access Response reception not successful; 2> increment PREAMBLE_TRANSMISSION_COUNTER by 1; 2> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 3> if the Random Access Preamble is transmitted on the SpCell: 4> indicate a Random Access problem to upper layers; 4> if this Random Access procedure was triggered for SI request: 5> consider the Random Access procedure unsuccessfully completed.

Proposal 4:  Reuse the legacy on-demand SIBx triggering condition to trigger WUS retransmission.

## 2.4 Details of the UL WUS configuration

RAN1 has discussed UL WUS configuration contents during RAN1#118bis and reached the following agreement.

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| WUS transmission | frequencyBandList |  |  |
|  | absoluteFrequencyPointA |  |  |
|  | offsetToCarrier |  |  |
|  | p-Max |  |  |
|  | frequencyShift7p5khz (working assumption)ULSubCarrierSpacing |  |  |
| Purpose | Parameters |  |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

RAN1 has agreed to include the above parameters in the WUS configuration. The CORESET#0 and the SS#0 can be included in the WUS configuration when SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2. RAN2 can consider this as the baseline and wait for more progress.

Proposal 5: Take RAN1’s agreement on UL WUS configuration contents as baseline and wait for more progress.

In the last meeting, some companies proposed to discuss whether to support 2-step based on-demand SIB1 request. In our view, there is no need to implement 2-step random access, since the PUSCH in Msg A is useless. Otherwise, NW will waste the resources to allocate some PUSCH for such purpose. In legacy on-demand SIBx, the 2-step based mechanism is also not introduced. We do not see the benefit to support 2-step based mechanism.

Proposal 6: 2-step RACH based on-demand SIB1 request is not supported.

## 2.5 Legacy UE impact

For legacy UE access restrictions RAN2 achieved the following agreements:

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

Since legacy UE barring is left to RAN1, RAN2 can address the issue of reselection. There are two options left on the table to be discussed. Legacy UEs are prevented from reselecting to the NES Cell by excluded cell list or reselection priorities.

Option 1: Use excluded cell list for the legacy UEs not to consider the NES Cell as candidate. At the same time a new excluded cell list without the NES Cells included in it should be provided to the NES UEs to allow the NES UEs to consider camping on the NES Cells. For this purpose, a new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs in SIB3 can be added, in addition to the legacy IntraFreqExcludedCellList / InterFreqExcludedCellList, to give the NW a possibility to configure NES cell camping restrictions separately for legacy and NES capable UEs.

Option 2: Set lower priority level to the NES Cell frequency for legacy UEs. The NES UEs would need a new priority to be allowed to camp on the NES Cell. For this option we would like to note that the OD-SIB1 feature is controlled per cell (not per carrier) therefore any modifications to cell reselection need to be per cell. Since the priority is control by per carrier, the priority based solution is not suitable. Besides, lower priority reduces the possibility that a legacy UE could reselect to the concerned carrier but does not exclude it. Therefore, even if the cell reselection priority is lower, the legacy UE still has a chance to reselect to the NES Cell which is not desired.

Observation 2: Priority based cell reselection is operated on a per carrier basis and cannot prevent legacy UEs from camping on the NES Cells on a per cell basis.

Proposal 7: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

## 2.7 RRC_CONNECTED UE impact

In the legacy SIB1 acquiring procedure, UE will consider the stored SIB1 as valid for 3 hours. After that time expires, the UE will delete any stored version of SIB. Moreover, a UE in RRC_CONNECTED state has the requirement to acquire SIB1 while T311 is running, i.e. while initiating the RRC connection re-establishment procedure.

1> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 1> if the UE is in RRC_CONNECTED while T311 is running: 2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE: 3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13]; 3> if the UE is unable to acquire the SIB1: 4> perform the actions as specified in clause 5.2.2.5; 3> else: 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2. 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> perform the actions as specified in clause 5.2.2.5. NOTE 1: The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

How to obtain the SIB1 for RRC_CONNECTED UE when SIB1 is broadcast in on-demand manner needs to be discussed. In the legacy spec, the UEs in RRC_CONNECTED can initiate dedicated SIB request with DedicatedSIBRequest message when UE not configured with common search space with the field searchSpaceOtherSystemInformation and the UE has not stored a valid version of a SIB.

##### 5.2.2.3.5 Acquisition of SIB(s) or posSIB(s) in RRC_CONNECTED The UE shall: 1> if the UE is in RRC_CONNECTED with an active BWP not configured with common search space with the field searchSpaceOtherSystemInformation and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, or 1> if the UE is in RRC_CONNECTED and acting as a L2 U2N Remote UE and the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1: 2> for the SI message(s) that, according to the si-SchedulingInfo or posSI-SchedulingInfo in the stored SIB1, contain at least one required SIB or requested posSIB: 3> if onDemandSIB-Request is configured and timer T350 is not running: 4> initiate transmission of the DedicatedSIBRequest message in accordance with 5.2.2.3.6; 4> start timer T350 with the timer value set to the onDemandSIB-RequestProhibitTimer; 1> else if the UE is in RRC_CONNECTED with an active BWP configured with common search space with the field searchSpaceOtherSystemInformation and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1: 2> for the SI message(s) that, according to the si-SchedulingInfo in the stored SIB1, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting: 3> acquire the SI message(s) as defined in clause 5.2.2.3.2; 2> for the SI message(s) that, according to the si-SchedulingInfo in the stored SIB1, contain at least one required SIB and for which si-BroadcastStatus is set to notBroadcasting: 3> if onDemandSIB-Request is configured and timer T350 is not running: 4> initiate transmission of the DedicatedSIBRequest message in accordance with 5.2.2.3.6; 4> start timer T350 with the timer value set to the onDemandSIB-RequestProhibitTimer; 4> acquire the requested SI message(s) corresponding to the requested SIB(s) as defined in clause 5.2.2.3.2.

Following the legacy mechanism, the OD-SIB1 should also be supported to be obtained by RRC_CONNECTED UE since SIB1 is essential for UEs to camp on a cell. Since the UE is in RRC_CONNECTED, there is no need to use WUS to trigger on-demand SIB1 transmission, the legacy DedicatedSIBRequest message can be enhanced to trigger on-demand SIB1 acquisition.

Proposal 8: Support RRC_CONNECTED UEs to trigger on-demand SIB1 acquisition by enhancing the legacy DedicatedSIBRequest or a new RRC message.

# Conclusion

In this contribution, we discussed the issues on on-demand SIB1 operation, and made the following observations and proposals:

Observation 1: NES cell’s state can be derived from the WUS configuration from Cell A.

Proposal 1: Wait for RAN1’s further process on the SSB transmission in NES Cell whether NES cell state can be derived from PBCH.

Proposal 2: Reuse the SI-window mechanism for the OD-SIB1 procedure.

Proposal 3: The RAR responding to OD-SIB1 request should contain at least a time window and OD-SIB1 broadcast status.

Proposal 4:  Reuse the legacy on-demand SIBx triggering condition to trigger WUS retransmission.

Proposal 5: Take RAN1’s agreement on UL WUS configuration contents as baseline and wait for more progress.

Proposal 6: 2-step RACH based on-demand SIB1 request is not supported.

Observation 2: Priority based cell reselection is operated on a per carrier basis and cannot prevent legacy UEs from camping on the NES Cells on a per cell basis.

Proposal 7: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

Proposal 8: Support RRC_CONNECTED UEs to trigger on-demand SIB1 acquisition by enhancing the legacy DedicatedSIBRequest or a new RRC message.

# References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- R2-2407571 Report from session on V2X/SL, R18/19 MOB, and R19 NES

- Chair notes RAN1#118bis eom0.doc