3GPP TSG- Meeting #128			R2-2410318

Orlando, USA, Nov. 18th-22nd, 2024

Agenda item:	8.7.6

Title:	Further consideration on XR rate control

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting, RAN2 discusses the rate control and has the following agreements [1],

Agreements on XR rate control - FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. - RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier - Send LS to SA4 asking about range/granularity which is required

This contribution discuses some details on rate control.

Discussion

  - Granularity and impact on legacy QoS

There was considerable discussion around rate control granularity at the last meeting, focusing primarily on whether control should be implemented per DRB/LCH or per QoS flow. While per-QoS flow control might offer finer granularity, the MAC layer typically does not manage QoS flow details. For instance, LCP, BSR, and DSR operations are all conducted per LCH, which serves as the smallest scheduling unit within the MAC layer.

If rate control MAC CEs were to include QoS flow identifiers, such as QFI, the MAC entity would be compelled to handle QoS-related information. This approach risks violating the established layered architecture principles by introducing QoS details into the MAC layer, whose primary role is to manage LCH-based data transfer and radio resource allocation. Moreover, the MAC layer currently lacks both QoS-specific information and QoS profile, meaning that even if QFI were included in a new rate control MAC CE, the MAC entity would have limited capacity to implement effective rate control, as it would not be able to identify the LCH corresponding to the specified QoS flow.

Observation 1a: MAC entity lacks QoS related information, e.g., QoS profile and mapping rules, therefore MAC entity may not understand QFI in MAC CE.

Therefore, if RAN2 intends to reuse or extend the Recommended Bit Rate MAC CE, it would be preferable to use per DRB/LCH indication, allowing the MAC entity to adjust PBR/BSD/Bj parameters for rate control without impacting other layers. Alternatively, if this approach is not adopted, using an RRC message or an AT command for rate control purposes would be recommended.

Moreover, if there is a 1:1 mapping between QoS flows and DRB/LCH, the granularity of per DRB/LCH and per QoS flow would be identical. If not, CN can help to make sure only similar QoS flows are typically mapped to the same DRB/LCH—for instance, pose and movement data would generally not be mapped to the same DRB/LCH as video data. Consequently, any impact from using a per DRB/LCH granularity approach, if it exists, is likely to be not very significant.

Observation 1b: QoS flow can be 1:1 mapped to DRB/LCH, and CN can help to make sure only similar QoS flows are mapped into the same DBR/LCH.

Regarding L4S, whose congestion control is performed at the TCP layer, it has a minimal impact on the MAC layer. Therefore, from our perspective, the key issue is determining which layer is responsible for rate control. If rate control is handled by the MAC layer, it should operate on a per DRB/LCH basis. However, if a higher layer, such as the PDCP layer, is responsible, rate control could be implemented on a per QoS flow basis. But it is against the scope formulated in RAN#105.

- Specify uplink congestion signaling [RAN2]: - Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion

Observation 1c: Using per QoS flow granularity or QFI may need RRC or other upper layer, which is out of scope.

Therefore, we think it would be better to use per DRB/LCH indication.

Proposal 1: If RAN2 uses MAC CE for rate control, per DRB/LCH indication should be considered.

  - Rate Control identifier in new MAC CE

Regarding the intention to facilitate rapid rate adaptation for congestion control, the gNB can send multiple new rate control MAC CEs to the device within a short period of time. This approach allows for quick adjustments to congestion, and once congestion is relieved, the gNB can leverage the new MAC CE for fast rate recovery, enabling the UE to get back to its normal data rate quicker.

In cases where gNB sends multiple rate control MAC CEs in quick succession for fast adaption, it is essential that both the UE and gNB have a mechanism to distinguish each rate control MAC CE, ensuring that the UE executes rate control actions based solely on the most recent MAC CE. To confirm that each rate control command is both received and implemented by the UE, a short identifier—2 or 3 bits, for example—can be included in the MAC CE. This identifier would provide reliable tracking while adding minimal overhead to the MAC CE. Also, re-transmission may also cause these kinds of “out of order delivery” issue.

Observation 2: For fast rate adaption, gNB may send multiple rate control messages, an identifier in rate control MAC CE is beneficial for UE to distinguish them and only execute the latest command.

Proposal 2: Introduce a short identifier (e.g., 2-3 bits) in Rate Control MAC CE to identify different rate control messages.

  - UE confirmation to rate control MAC CE

Another issue that RAN2 may need to address is the behaviour of the gNB and UE after the UE adjusts its rate based on the rate control MAC CE. Currently, the gNB can only infer that the UE has applied the rate adjustment by monitoring the data rate over the air interface, which is inefficient and introduces a delay. This method is especially problematic when an unsuitable rate is configured in MAC CE, as it can take a significant amount of time before the gNB detects that the rate is inappropriate. Introducing a rate control confirmation MAC CE would therefore be advantageous, as it would allow the gNB to promptly verify that the UE has implemented the intended rate adjustment.

For example, there is a Configured Grant Confirmation MAC CE, for a similar purpose.

Observation 3: To enable fast rate control and avoid over-adjustment, it would be beneficial for UE to notify the gNB once it completes the rate adaption.

Proposal 3: Introduce a Rate Control Confirmation MAC CE to inform gNB that UE has completed rate control based on former gNB indication.

Conclusion

Based on our observation and analysis, we propose that,

Rate control granularity:

Observation 1a: MAC entity lacks QoS related information, e.g., QoS profile and mapping rules, therefore MAC entity may not understand QFI in MAC CE.

Observation 1b: QoS flow can be 1:1 mapped to DRB/LCH, and CN can help to make sure only similar QoS flows are mapped into the same DBR/LCH.

Observation 1c: Using per QoS flow granularity or QFI may need RRC or other upper layer, which is out of scope.

Proposal 1: If RAN2 uses MAC CE for rate control, per DRB/LCH indication should be considered.

New identifier:

Observation 2: For fast rate adaption, gNB may send multiple rate control messages, an identifier in rate control MAC CE is beneficial for UE to distinguish them and only execute the latest command.

Proposal 2: Introduce a short identifier (e.g., 2-3 bits) in Rate Control MAC CE to identify different rate control messages.

Feedback to rate control:

Observation 3: To enable fast rate control and avoid over-adjustment, it would be beneficial for UE to notify the gNB once it completes the rate adaption.

Proposal 3: Introduce a Rate Control Confirmation MAC CE to inform gNB that UE has adjusted its rate based on former gNB indication.

References

- R2_127bis R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM