3GPP TSG RAN WG2 Meeting #128	R2-2410333

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.9.4

Source:	CMCC

Title:	Support of PWS messages for NB-IoT

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of PWS for NB-IoT and some agreements were achieved as follows [1]:

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

In this contribution, we would like to provide some further considerations on the PWS for NB-IoT.

Discussion

Whether added PWS indication in Paging-NB

We have agreed to add etws-Indication and cmas-Indication in direct indication information for NB-IoT in last meeting and whether add both the indications in Paging-NB is left to this meeting. As description in section 5.2.1.4 and 5.2.1.5 of TS 36.331, the Paging message is used to inform ETWS/CMAS capable UEs in RRC_IDLE and UEs not in CE in RRC_CONNECTED about presence of an ETWS primary notification and/ or ETWS secondary notification and one or more CMAS notifications. In order to support PWS for NB-IoT better, we could also consider to add PWS indication (i.e. etws-Indication and cmas-Indication) in Paging-NB for PWS reception in RRC_IDLE.

Proposal 1: It is proposed to add PWS indication (i.e. etws-Indication and cmas-Indication) in Paging-NB for PWS reception in RRC_IDLE.

Whether support PWS notification in connected mode

Per the existing TS 36.300, network uses the paging messages to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. And the NB-IoT UE is not required to detect SIB changes when in connected mode. Then if NB-IoT UE needs to acquire SI change, network may release it to RRC_IDLE.

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. For NB-IoT UEs, BL UEs, and UEs in CE, the UE is not required to detect SIB changes when in RRC_CONNECTED, and the network may release the NB-IoT UE, BL UE or UE in CE to RRC_IDLE if it wants the NB-IoT UE, BL UE or UE in CE to acquire changed SIB(s).

And in TS 36.331, it also indicates that NB-IoT UE would not monitor paging in RRC_CONNECTED for SI change and PWS including both ETWS and CMAS notification.

- 5.3.2 Paging - 5.3.2.1 General Figure 5.3.2.1-1: Paging The purpose of this procedure is: - to transmit CN initiated paging information to a UE in RRC_IDLE or RRC_INACTIVE and/ or; - to transmit RAN initiated paging information to a UE in RRC_INACTIVE and/or; - to inform UEs in RRC_IDLE, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about a system information change and/ or; - to inform UEs in RRC_IDLE other than NB-IoT UEs, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about an ETWS primary notification and/ or ETWS secondary notification and/ or; - to inform UEs in RRC_IDLE other than NB-IoT UEs, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about a CMAS notification and/ or; - to inform UEs other than NB-IoT UEs in RRC_IDLE, and other than UEs connected to 5GC about an EAB parameters modification and/ or; - to inform UEs other than NB-IoT UEs in RRC_IDLE, and UEs in RRC_INACTIVE to perform E-UTRAN inter-frequency redistribution procedure.

Then if monitor direct indication information in RRC_CONNECTED similar to eMTC would lead to energy consumption which is not expected for NB-IoT UEs. And if support PWS reception in RRC_CONNECTED for NB-IoT would also introduce large impact on current specification. Further, PWS does not occur frequently in actual, there is no significant gain to support PWS reception in RRC_CONNECTED. And even if the PWS is triggered in the RRC_CONNECTED, it could up to network implementation to release UE to the RRC_IDLE for receiving the PWS message. Therefore, introduce PWS notification in connected mode is not supported based on above description.

Observation 1: There is no significant gain to support PWS reception in RRC_CONNECTED due to that PWS is not frequent. And even if the PWS is triggered in the RRC_CONNECTED, it could up to network implementation to release UE to the RRC_IDLE for receiving the PWS message

Proposal 2: Not support PWS notification in connected mode.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on Discussion on broadcast of PWS messages for NB-IoT and have the following observation and proposals:

Proposal 1: It is proposed to add PWS indication (i.e. etws-Indication and cmas-Indication) in Paging-NB for PWS reception in RRC_IDLE.

Observation 1: There is no significant gain to support PWS reception in RRC_CONNECTED due to that PWS is not frequent. And even if the PWS is triggered in the RRC_CONNECTED, it could up to network implementation to release UE to the RRC_IDLE for receiving the PWS message

Proposal 2: Not support PWS notification in connected mode.

Reference

- R2-127bis NR-NTN-IoT-NTN (Sergio)_ EOM

- RP-242397 WID Rev R19 IOT-NTN cl

- 3GPP TS 36.300 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18)

- 3GPP TS 36.331 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)