3GPP TSG RAN WG2 Meeting #128	R2-2410335

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.2.5

Source:	CMCC

Title:	Discussions on topology 2 for A-IoT

Document for:	Discussion

Introduction

In RAN2#127bis meeting, the following agreements are made on Topology 2 for A-IoT:

- Agreements - 1. Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release. - 2. The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells. - 3. The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

Then in this contribution, we would like to provide our views on the remaining issues for topology 2.

Discussion

### 2.1 General aspects

In RAN2#127 meeting, its has been agreed that RAN2 is responsible for the interface between intermediate node and RAN. Further with the consideration that only UE as the intermediate node is in scope of the revised SID [1], NR Uu naturally becomes one of the options. In order to minimize the workload of RAN’s study, we suggest that NR Uu should be reused as much as possible between A-IoT enabled UE and A-IoT enabled gNB with necessary enhancements, such as supporting resource allocation for A-IoT air interface, etc.

Proposal 1: NR Uu should be fully reused as the interface between A-IoT enabled UE and A-IoT enabled gNB with necessary enhancements.

### 2.2 RRC state of intermediate UE

In terms of the RRC state of intermediate UE, it has been discussed but do not make agreements in RAN2#127bis meeting. From our point of view, it should be analyzed according to the following two situations:

- Situation 1: the RRC state of the A-IoT enabled UE when initiate

- For situation 1, the RRC state of A-IoT enbaled UE when initiate, that is, before it is selected as an intermediate, depends on the RRC state of A-IoT enbaled UE can be reached by CN or A-IoT enbaled gNB. To be specific, for the selection of intermediate UE, it may be conducted by A-IoT enabled gNB or CN. To be specific, for the intermediate UE selection dominated by the CN, the A-IoT capable UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be reached. While, for the intermediate UE selection dominated by gNB, the A-IoT capable UE in RRC_INACTIVE and RRC_CONNECTED state can be reached directly. Note that gNB-dominated selection and CN-dominated selection are not contradictory, that is, they may exist simultaneously. While, note that upon one A-IoT enabled UE is selected as an intermediate UE, some parameters which are related to A-IoT air interface resources or A-IoT service should be configured to it. Therefore, an direct and efficient solution is to transmit those parameters via dedicated RRC message after intermediate UE transferred to RRC_CONNECTED stste. For A-IoT capable UE in RRC_CONNECTED state, dedicated RRC message can be used to carry such parameter. For A-IoT capable UE in RRC_INACTIVE state, it can be resumed to RRC_CONNECTED by a RAN-initiated paging at first. And for A-IoT capable UE in RRC_IDLE state, it should be transfer to RRC_CONNECTED state by CN-initiated paging at first. For intermediate UEs in RRC_INACTIVE or RRC_IDLE state, dedicated RRC message which contains parameters related to A-IoT air interface resources or subsequent A-IoT procedure can be transmitted to them after they are transferred to RRC_CONNECTED. Based on above analysis, the A-IoT capable UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be selected as intermediate UEs. While, for A-IoT capable UEs in RRC_IDLE or RRC_INACTIVE, they should transfer to RRC_CONNECTED under network control before initiating A-IoT procedure.

Observation 1: CN or A-IoT enabled gNB initiated intermediate UE selection are both feasible.

Observation 2: Upon one A-IoT enabled UE is selected as an intermediate UE, some parameters which are related to A-IoT air interface resources or subsequent A-IoT procedure should be transmitted to it. An direct and efficient solution is to transmit those parameters with dedicated RRC message after intermediate UE transferred to RRC_CONNECTED state.

Proposal 2: Upon one A-IoT enabled UE is selected as an intermediate UE, some parameters which are related to A-IoT air interface resources or subsequent A-IoT procedure should be transmitted to it.

Proposal 3: The A-IoT capable UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be selected as intermediate UEs.

Proposal 4: If A-IoT capable UEs in RRC_IDLE or RRC_INACTIVE state is selected, it should be transferred to RRC_CONNECTED state before initiating A-IoT procedure.

- Situation 2: the RRC state of the intermediate UE when conducting A-IoT procedure

Situation 2 was the main topic of discussion at the last meeting but no agreement has been made. After an intermediate UE is selected, it conducts the inventory-only or command-only or inventory and command procedure according to the received parameters which indicates the resource allocation of A-IoT air interface and/or the subsequent A-IoT procedure. For situation 2, both how to keep the validity of resources in A-IoT air interface and how an intermediate transmit the feedback to gNB via Uu interface should be taken into consideration. For both factors mentioned above, keeping the intermediate UE in RRC_CONNECTED state is a good solution and minimizes impacts on standard. However for intermediate UE in RRC_IDLE or RRC_INACTIVE state, how to keep the validity of resources in A-IoT air interface and how an intermediate transmit the feedback to gNB via Uu interface needs further discussion and will bring larger impacts on legacy standard. To this end we suggest that RAN2 assumes the intermediate UE in RRC_CONNECTED state when conducting A-IoT procedure in R19.

Observation 3: In terms of the RRC state of the intermediate UE when conducting A-IoT procedure, both how to keep the validity of resources in A-IoT air interface and how an intermediate transmit the feedback to gNB via Uu interface should be taken into consideration.

Observation 4: Keeping the intermediate UE in RRC_CONNECTED state when conducting A-IoT procedure is a good solution and minimizes impacts on standard.

Observation 5: For intermediate UE in RRC_IDLE or RRC_INACTIVE state, how to keep the validity of resources in A-IoT air interface and how an intermediate transmit the feedback to gNB via Uu interface needs further discussion and will bring larger impacts on legacy standard.

Proposal 5: RAN2 assumes the intermediate UE in RRC_CONNECTED state when performing A-IoT procedure in R19.

### 2.3 Resource allocation for intermediate UE

In RAN2#127bis meeting, it has been agree that:

- 1. Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release. - 2. The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells. - 3. The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

- It can be found that dedicated resource configuration is supported and mechanisms for shared resource pool amongst multiple intermediate UEs is not considered in R19. It is benefitial for an A-IoT enabled gNB allocating resources of A-IoT air interface efficiently if some information, e.g. the (rough) number of target device(s), etc., can be provided/visible to it. To be specific, if such information is visible to A-IoT enabled gNB, dedicated resource configuration can be transmitted to the intermediate UE via dedicated signalling directly. On the other hand, if such information is transmitted to intermediate UE directly and invisible to A-IoT enabled gNB, the intermediate UE may beed to request the resources to be used by A-IoT air interface from A-IoT enabled gNB. Note that whether some information which is benefitial for A-IoT enabled gNB allocating resources can be provided/visible to it is closely related to the architectures of topology 2. Therefore, RAN2 can support 1) A-IoT enabled gNB allocating resources directly and 2) intermediate UE requesting resources from A-IoT enabled gNB and capture both of them to TR.

- Observation 6: For an A-IoT enabled gNB allocating resources of A-IoT air interface efficiently, some information should be provided or visible to it.

- Observation 7: The methods for an A-IoT enabled gNB allocating A-IoT air interface resources to intermediate are closely related to whether some information from CN is provided or visible to A-IoT enabled gNB.

Proposal 6: RAN2 can support 1) A-IoT enabled gNB allocating resources directly and 2) intermediate UE requesting resources from A-IoT enabled gNB and capture both of them to TR.

Apart from this, the resources which is related to A-IoT service to be used in Uu interface should be configured by gNB as well. Note that the configuration of A-IoT related resources in Uu interface is closely related to how an intermediate UE reports the A-IoT procedure feedback to network, e.g. one device ID each time or multiple device IDs a time. From our perspective both above options can be supported. With the consideration that there are many legacy NR solutions of resource allocation can be considered to reused, RAN2 can design the methods of allocating Uu resources for an intermediate UE reporting the results related to A-IoT procedure in work item.

- Observation 8: The configuration of A-IoT related resources in Uu interface is closely related to how an intermediate UE reports the A-IoT procedure feedback to network, e.g. one device ID each time or multiple device IDs each time.

Proposal 7: RAN2 should support following options for an intermediate UE reports its results related to A-IoT procedure: 1) one device ID each time, 2) multiple device IDs a time.

Proposal 8: RAN2 can design the methods of allocating Uu resources for an intermediate UE reporting the results related to A-IoT procedure in work item.

### 2.4 Issues for “temporary” out of connection cases

In RAN2#127 meeting, it has been agreed that:

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

Further in RAN2#127bis meeting, the following agreement has been made:

- 2. The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells.

- The behavior of intermediate UE when it is temporary out of connection depends on whether there is valid A-IoT air interface resources. If it has valid A-IoT air interface resources, it can continue the A-IoT procedure, otherwise, it should suspend or terminate A-IoT procedure. For the validity of A-IoT air interface resources, the following issues should be discussed:

- Issue 1: whether there are valid resources across multiple cells

- From our perspective, to achieve the validity of resources across multiple cells, a common resources should be reserved by all these cells. Therefore, the pros and cons for supporting valid resources across multiple cells are analyzed as following:

- Pros: The intermediate UE that uses valid A-IoT air interface resources across multiple cells can continue to use above resources to conduct A-IoT procedure while moving across these cells, thereby better ensuring the continuity of the A-IoT procedure.

- Cons: The reserved resources may affect the spectrum utilization of the system, as even if the intermediate UE does not move to the coverage of other cells, the reserved resources of other cells still need to be there. Apart from this, the process of determining reserved resources among multiple cells may require information exchange on the F1 or Xn interfaces, which may bring additional standardization impacts to RAN3.

- Based on above analysis, supporting resource validity across multiple cells brings much more cons than pros which is the optimization instead of basic functions. Therefore, we suggest no to support resource validity across multiple cells in R19.

- Observation 9: Supporting resource validity across multiple cells brings much more cons than pros which is the optimization instead of basic functions.

Proposal 9: From RAN2’s perspective, resource validity across multiple cells is not supported in R19.

- Issue 2: how for an intermediate UE determine whether A-IoT air interface resources are valid or not for “temporary” out of connection cases

- In RAN2#127bis meeting, some solutions such as timer-based validity mechanism have been mentioned for an intermediate UE determine whether A-IoT air interface resources are valid or not. Note that one of the important reasons for gNB controlling the A-IoT air interface resources is to better avoid the interference between multiple intermediate UEs or between intermediate UE and legacy UEs. However for timer-based validity mechanism, if the timer is not expired, the intermediate UE may continue its A-IoT procedure even if it has declared RLF, thus may cause interference to other UE(s). Apart from this, the length of timer is not easy to set. If the timer is too short, then the A-IoT procedure it is performing will fail as well. If the timer is too long, then the interference will aslo be caused. To this end, timer-based validity mechanism is not preferred. Whether the definition of “validity of A-IoT air interface resources” is needed, we suggest decide it in the work item.

Proposal 10: Timer-based validity mechanism is not supported, otherwise it may cause interference to other intermediate UEs and it is difficult to deciding the length of the timer.

Proposal 11: Whether the definition of “validity of A-IoT air interface resources” is needed can be decided in work item.

### 2.5 Architecture and signalling/data transmission procedure

In previous RAN2 meeting, three solutions (e.g. RRC-based solution, NAS-based solution and UP-based solution) are discussed. The detailed architecture and signalling/data transmission procedure are analyzed as following:

#### 2.5.1 RRC-based solution

For RRC-based solution, the signalling or data interacted between A-IoT enabled gNB and intermediate UE is RRC message. The A-IoT enabled gNB generates RRC messages based on the parameters provided by the CN when transmitting downlink signalling/data to intermediate UE. To this end, it is more efficient for A-IoT enabled gNB allocation A-IoT air interface resources since some parameters from CN can be visible to A-IoT enabled gNB directly. The detailed protocol stack and corresponding procedure can be given in Figure 1 and Figure 2, respectively.

Figure 1: Protocol stack of Topology 2 with RRC-based solution

Figure 2: Procedure of RRC-based solution

1. The AIoT CN sends an Inventory request message to the AIoT-enabled gNB via NG/XX-AP.

- The AIoT-enabled gNB sends an Inventory response message to the AIoT CN via NG/XX-AP.

- The A-IoT enabled gNB generates and transmits a RRC message which contains inventory or command to intermediate UE.

- The intermediate UE transmits an A-IoT paging to A-IoT device(s) and conduct the corresponding procedure indicated in the received RRC message.

- A-IoT device transmits its response to intermediate UE.

- The intermediate UE transmits a RRC message with inventory report or command report to A-IoT enabled gNB.

- The A-IoT enabled gNB transmits a message with inventory report or command report to A-IoT CN in NG/XX-AP interface.

Note 1: The details of step 1, 2 and 7 should be discussed and determined by RAN3.

Note2: The configuration of intermediate UE for A-IoT air interface resources can be transmitted to intermediate without request before step 4, e.g. in step 3.

Observation 10: For RRC-based solution, the gNB or intermediate UE generates and transmits a RRC message which contains command or response.

Observation 11: For RRC-based solution, it is more efficient for A-IoT enabled gNB allocation A-IoT air interface resources since some parameters from A-IoT CN can be visible to A-IoT enabled gNB directly.

Proposal 12: For Topology 2 protocol stack, RAN2 can capture Figure 1 as the baseline of the RRC-based solution.

Proposal 13: RAN2 can capture the procedure of RRC-based solution shown in Figure 2 as a baseline.

#### 2.5.2 NAS-based solution

In this case, A-IoT CN interacts with intermediate UE using NAS message directly. Considering that the NAS message is transparent for A-IoT enabled gNB, the intermediate UE need to request the resources used in A-IoT air interface from A-IoT enabled gNB, thus increases the signalling overhead compared to RRC-based solution. The protocol stack and corresponding procedure can be given in Figure 3 and Figure 4, respectively.

Figure 3: Protocol stack of Topology 2 with NAS-based solution

Figure 4: Procedure of NAS-based solution

- 1. The AMF sends a NAS message which contains inventory request or command request to intermediate UE.

- 2. The intermediate UE sends an NAS message contains inventory response or command response message to the AMF.

- 3. The intermediate UE sends a message to request the A-IoT air interface resources from A-IoT enabled gNB.

- 4. The A-IoT CN sends the configuration message to intermediate UE.

- 5. The intermediate UE sends the A-IoT paging to A-IoT device(s) and conducts the corresponding procedure indicated in the received inventory request or command request.

- 6. A-IoT device sends its response to intermediate UE.

- 7. The intermediate UE sends a NAS message which contains the inventory report or command report to AMF.

Note 1: The details of step 1, 2 and 7 may also be discussed and determined by RAN3/SA2.

Observation 12: For NAS-based solution, the CN or intermediate UE transmits a NAS message which contains command or response.

Observation 13: For NAS-based solution, the intermediate UE may need to request the A-IoT radio resources from gNB when no available A-IoT air interface resources allocated, thus increases the signalling overhead compared to RRC-based solution.

Proposal 14: For Topology 2 protocol stack, RAN2 can capture Figure 3 as the baseline of the NAS-based solution.

Proposal 15: RAN2 can capture the procedure of NAS-based solution shown in Figure 4 as a baseline.

#### 2.5.2 UP-based solution

In this case, CN interacts with intermediate UE using user plane PDU transmitted in PDU session. Note that for UP-based solution, signalling transmission via control plane may also be needed because A-IoT air interface resources should be configured by A-IoT enabled gNB. The protocol stack and corresponding procedure can be given in Figure 5 and Figure 6, respectively.

Figure 5: Protocol stack of Topology 2 with UP-based solution

Figure 6: Procedure of UP-based solution

- 0.The PDU session is successfully established between Intermediate UE and UPF.

1. The UPF sends a inventory request or command request which a PDU session to intermediate UE.

2. The intermediate UE sends a inventory response or command response in a PDU session to UPF.

- 3. The intermediate UE sends a message to request the A-IoT air interface resources from A-IoT enabled gNB.

4. The A-IoT CN sends the configuration message to intermediate UE.

5. The intermediate UE sends the A-IoT paging to A-IoT device(s) and conducts the corresponding procedure indicated in the received inventory request or command request.

3. A-IoT device(s) sends its response to the intermediate UE.

4. The intermediate UE sends the inventory report or command report in a PDU session to UPF.

Note 1: The details of step 0, 1, 2 and 7 may also be discussed and determined by RAN3/SA2.

Note 2: signalling transmission via control plane may also be needed because A-IoT enabled gNB controls the resources to be used in A-IoT air interface which is not depicted in Figure 6.

Observation 14: For UP-based solution, CN interacts with intermediate UE using user plane PDU transmitted in PDU session.

Observation 15: For UP-based solution, signalling transmission via control plane may also be needed when A-IoT enabled gNB controls resource allocation.

Proposal 16: For Topology 2 protocol stack, RAN2 can capture Figure 5 as the baseline of the UP-based solution.

Proposal 17: RAN2 can capture the procedure of UP-based solution shown in Figure 6 as a baseline.

#### 2.5.4 Recommendation for normative work

In TR 23.700-13, SA2 has already concluded that RRC-based solution and UP-based solution will be specified, while NAS-based solution is still FFS. Based on the remaining issues, we think we can conclude for the study item in this meeting. With the consideration that the time budget of work item is also limited, we suggest to narrow down the scope of work item by down selecting between three architectures. For RRC-based solution, it is efficient for A-IoT enabled gNB allocating A-IoT air interface resources to intermediate. What’s more, RRC-based solution brings benefits for A-IoT enabled gNB to select more suitable intermediate UEs to mitigate inter-UE interference since there is more real-time information in the gNB, e.g. the beam where intermediate UE in RRC_CONNECTED state is camping, etc., which can be obtained by A-IoT enabed gNB. And from network deployment point of view, for RRC based solution, the same network framework is applied for both topology 1 and topology 2, which is more deployment friendly. For UP-based solution, it is simple from UE point of view, an obvious advantage is that end-to-end IP connectivities are between intermediate UE and UPF and between UPF and AIoTF node. The drawback is that, there is big difference between UP-based solution and topology 1 framework, which requires network to deploy a new framework in addition to topology 1. While for NAS-based solution, it is still being discussed by SA2 whether it is needed and we do not find many obvious advantages compared with the other two solutions. Therefore, we recommend RRC-based solution and UP-based solution to work item.

Observation 16: For RRC-based solution, it is efficient for A-IoT enabled gNB allocating A-IoT air interface resources to intermediate and it brings benefits for A-IoT enabled gNB to select more suitable intermediate UEs since more real-time information, e.g. the beam where intermediate UE camped, etc., can be obtained by A-IoT enabed gNB.

Observation 17: For UP-based solution, an obvious advantage is that end-to-end connectivities are achieved between intermediate UE and UPF and between UPF and AIoTF node.

Observation 18: For NAS-based solution, it is still being discussed by SA2 whether NAS-based solution is needed and we do not see obvious advantage compared with the other two solutions.

Proposal 18: RRC-based solution and UP-based solution are recommended for normative work.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some considerations for topology 2 as following:

Observation 1: CN or A-IoT enabled gNB initiated intermediate UE selection are both feasible.

Observation 1: CN or A-IoT enabled gNB initiated intermediate UE selection are both feasible.

Observation 2: Upon one A-IoT enabled UE is selected as an intermediate UE, some parameters which are related to A-IoT air interface resources or subsequent A-IoT procedure should be transmitted to it. An direct and efficient solution is to transmit those parameters with dedicated RRC message after intermediate UE transferred to RRC_CONNECTED state.

Observation 3: In terms of the RRC state of the intermediate UE when conducting A-IoT procedure, both how to keep the validity of resources in A-IoT air interface and how an intermediate transmit the feedback to gNB via Uu interface should be taken into consideration.

Observation 4: Keeping the intermediate UE in RRC_CONNECTED state when conducting A-IoT procedure is a good solution and minimizes impacts on standard.

Observation 5: For intermediate UE in RRC_IDLE or RRC_INACTIVE state, how to keep the validity of resources in A-IoT air interface and how an intermediate transmit the feedback to gNB via Uu interface needs further discussion and will bring larger impacts on legacy standard.

- Observation 6: For an A-IoT enabled gNB allocating resources of A-IoT air interface efficiently, some information should be provided or visible to it.

- Observation 7: The methods for an A-IoT enabled gNB allocating A-IoT air interface resources to intermediate are closely related to whether some information from CN is provided or visible to A-IoT enabled gNB.

- Observation 8: The configuration of A-IoT related resources in Uu interface is closely related to how an intermediate UE reports the A-IoT procedure feedback to network, e.g. one device ID each time or multiple device IDs each time.

- Observation 9: Supporting resource validity across multiple cells brings much more cons than pros which is the optimization instead of basic functions.

Observation 10: For RRC-based solution, the gNB or intermediate UE generates and transmits a RRC message which contains command or response.

Observation 11: For RRC-based solution, it is more efficient for A-IoT enabled gNB allocation A-IoT air interface resources since some parameters from A-IoT CN can be visible to A-IoT enabled gNB directly.

Observation 12: For NAS-based solution, the CN or intermediate UE transmits a NAS message which contains command or response.

Observation 13: For NAS-based solution, the intermediate UE may need to request the A-IoT radio resources from gNB when no available A-IoT air interface resources allocated, thus increases the signalling overhead compared to RRC-based solution.

Observation 14: For UP-based solution, CN interacts with intermediate UE using user plane PDU transmitted in PDU session.

Observation 15: For UP-based solution, signalling transmission via control plane may also be needed when A-IoT enabled gNB controls resource allocation.

Observation 16: For RRC-based solution, it is efficient for A-IoT enabled gNB allocating A-IoT air interface resources to intermediate and it brings benefits for A-IoT enabled gNB to select more suitable intermediate UEs since more real-time information, e.g. the beam where intermediate UE camped, etc., can be obtained by A-IoT enabed gNB.

Observation 17: For UP-based solution, an obvious advantage is that end-to-end connectivities are achieved between intermediate UE and UPF and between UPF and AIoTF node.

Observation 18: For NAS-based solution, it is still being discussed by SA2 whether NAS-based solution is needed and we do not see obvious advantage compared with the other two solutions.

Proposal 1: NR Uu should be fully reused as the interface between A-IoT enabled UE and A-IoT enabled gNB with necessary enhancements.

Proposal 2: Upon one A-IoT enabled UE is selected as an intermediate UE, some parameters which are related to A-IoT air interface resources or subsequent A-IoT procedure should be transmitted to it.

Proposal 3: The A-IoT capable UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be selected as intermediate UEs.

Proposal 4: If A-IoT capable UEs in RRC_IDLE or RRC_INACTIVE state is selected, it should be transferred to RRC_CONNECTED state before initiating A-IoT procedure.

Proposal 5: RAN2 assumes the intermediate UE in RRC_CONNECTED state when performing A-IoT procedure in R19.

Proposal 6: RAN2 can support 1) A-IoT enabled gNB allocating resources directly and 2) intermediate UE requesting resources from A-IoT enabled gNB and capture both of them to TR.

Proposal 7: RAN2 should support following options for an intermediate UE reports its results related to A-IoT procedure: 1) one device ID each time, 2) multiple device IDs a time.

Proposal 8: RAN2 can design the methods of allocating Uu resources for an intermediate UE reporting the results related to A-IoT procedure in work item.

Proposal 9: From RAN2’s perspective, resource validity across multiple cells is not supported in R19.

Proposal 10: Timer-based validity mechanism is not supported, otherwise it may cause interference to other intermediate UEs and it is difficult to deciding the length of the timer.

Proposal 11: Whether the definition of “validity of A-IoT air interface resources” is needed can be decided in work item.

Proposal 12: For Topology 2 protocol stack, RAN2 can capture Figure 1 as the baseline of the RRC-based solution.

Proposal 13: RAN2 can capture the procedure of RRC-based solution shown in Figure 2 as a baseline.

Proposal 14: For Topology 2 protocol stack, RAN2 can capture Figure 3 as the baseline of the NAS-based solution.

Proposal 15: RAN2 can capture the procedure of NAS-based solution shown in Figure 4 as a baseline.

Proposal 16: For Topology 2 protocol stack, RAN2 can capture Figure 5 as the baseline of the UP-based solution.

Proposal 17: RAN2 can capture the procedure of UP-based solution shown in Figure 6 as a baseline.

Proposal 18: RRC-based solution and UP-based solution are recommended for normative work.

Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, Maastricht, Netherlands, CMCC, Huawei, T-Mobile USA.