3GPP TSG-RAN WG2 Meeting #128                            	                                 R2-2410337

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda Item:	8.7.5

Source: 	CMCC

Title:  	Discussion on the RLC enhancements for XR and RAN2 impacts on the SA2 LS

WID/SID:	         NR_XR_Ph3-Core

Document for:	Discussion and Decision

Introduction

In the last RAN2 #127bis meeting, the RLC enhancements has been discussed and the following related agreements have been approved.

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In this contribution, we provide our views on the RLC enhancements and RAN2 impacts on the SA2 LS.

Discussion

SA2 LS on Application-Layer FEC Awareness

In AL-FEC mechanism mentioned in [2][3], source data packets are used to generate repair packets. The repair packets are transmitted with the source data packets, for the purpose of detection and correction of errors in the data stream. If the receiver successfully receives all the source packets, then the repair packets are not needed by the receiver. Therefore the core network indicates RAN with the AL-FEC related information Content ratio, which is the ratio of PDUs of a PDU Set that are needed at the UE to be able to reconstruct the original content. The RAN may use these information, e.g. to make packet discarding decisions in case of QoS flow congestion.

The following principles of AL-FEC awareness for normative work are described below in [2]:

Normative work for AL-FEC awareness at NG-RAN will be based on the following principles: - When requesting or updating QoS for a DL flow, an AF may provide to PCF/NEF - together with PDU Set QoS parameters - the content ratio for the flow. An AF may provide either PSIHI or content ratio for a flow. - If a PCF receives the content ratio from an AF, then the PCF may include the content ratio in PCC rules that it provides to the SMF. - The SMF provides content ratio to NG-RAN when establishing/modifying a QoS flow. - If NG-RAN supports PDU Set content ratio awareness and has received content ratio information for a QoS flow, then NG-RAN may consider content ratio information as follows: - If NG-RAN needs to discard PDUs of the QoS flow during congestion, then NG-RAN may use the content ratio information for discarding.

In [2], SA2 would like to ask the following questions:

- Question 1: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode based on the above principles. Please also provide feedback on applicability of RLC AM for XR services, also considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget as per RP-240791.

In our understanding, when the gNB obtains the total number of PDUs in a PDU set (N), which is indicated in the first PDU of a PDU set, and content ration of a PDU set (K/N), the gNB could derive K PDUs in a PDU set which is sufficient for gNB to reconstruct the original content (e.g. K=content ration*the total number of PDUs in a PDU set). Even the gNB discards all K PDUs in a PDU set in case of congestion and it does not destroy the original content. Indeed, when the gNB receives STATUS Report from UE in RLC AM mode, it counts the number of successful received PDUs in a PDU set and increase the counter accordingly. In the congestion case, the gNB could discard the remaining PDUs in a PDU set once the counter reaches K.  Therefore it is useful for gNB to be provided the content ratio information for discarding of DL PDUs during congestion for RLC AM mode.

Furthermore, the content ration could also be used in the solutions for Avoidance of unnecessary re-transmission. If the Tx side could stop to retransmit the remaining PDUs in a PDU set once the counter reaches K.

Observation 1: When the gNB receives STATUS Report from UE in RLC AM mode, it counts the number of successful received PDUs in a PDU set and increase the counter accordingly. In the congestion case, the gNB could discard the remaining PDUs in a PDU set once K out of N packets was successfully received.

Observation 2: The content ration could also be used in the solutions for Avoidance of unnecessary re-transmission. If the Tx side could stop to retransmit the remaining PDUs in a PDU set once K out of N packets was successfully received.

Proposal 1: It is useful for gNB to be provided the content ratio information for discarding of DL PDUs during congestion for RLC AM mode.

- Question 2: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC UM based on the above principles.

For RLC UM PDU, currently, gNB has no idea of whether it had been successfully delivered due to the lack of RLC STATUS Report.  And RAN2 considers that it is not reliable for the gNB to derive an RLC UM PDU transmission status from related MAC HARQ feedback since the MAC HARQ is not 100% reliable.  Then it is hard for gNB to derive K PDUs in a PDU set which is successfully received to reconstruct the original content. It may not bring benefit for gNB to be provided with the content ratio information for discarding of DL PDUs during congestion for RLC UM mode.

Observation 3: For RLC UM PDU, currently, gNB has no idea of whether it had been successfully delivered due to the lack of RLC STATUS Report.  And RAN2 considers that it is not reliable for the gNB to derive an RLC UM PDU transmission status from related MAC HARQ feedback since the MAC HARQ is not 100% reliable.  It is hard for gNB to derive K PDUs in a PDU set which is successfully received to reconstruct the original content.

Proposal 2: It may not bring benefit for gNB to be provided with the content ratio information for discarding of DL PDUs during congestion for RLC UM mode.

Avoidance of unnecessary re-transmission

In the last RAN2 meeting,  the combined approach has been selected to enhance the RLC AM mechanism for avoidance of the unnecessary retransmissions as below:

- TX side stops transmissions of an outdated SDU

- RX side abandons the SDU based on a local timer

- Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified

The details of combined approach have not reach a consensus in last meeting and still need to be discussed.

The UE as transmitting side could acquire the remaining time of an SDU. Since the PDCP layer is the upper layer and could derive the remaining time of an SDU, it is more natural and accurate that PDCP indicates the RLC to trigger the discard procedure based on the remaining time.  RAN2 should confirm that the TX side should stop the transmissions of an outdated RLC SDU based on the indication from upper layer.

Proposal 3: In combined approach, TX side should stop the transmissions of an outdated RLC SDU based on the indication from upper layer.

In the combined approach, it is agreed that RX side abandons the SDU based on a local timer.  The Rx side could not obtain the exact remaining time of a PDU directly. It was proposed in previous meeting that the local timer in RX side could be introduced based on the t-Reordering. The t-Reordering is used to detect the loss of in the PDCP PDU [1] and it starts when the PDCP SN gap was detected. In this way, RLC receiver needs to monitor the status of PDCP timer t-Reordering and start a new timer accordingly. This method seems complexity for RLC to maintain the interaction between two layers (PDCP and RLC) concurrently. And t-Reordering in PDCP shall not be started additionally, i.e. only one t-Reordering per receiving PDCP entity is running at a given time [1]. If more than one RLC SN gap was found, the t-Reordering-like RLC local timer should restart for each SN gap, which could bring more delay before moving the receiving window.

Observation 4:  To introduce the t-Reordering-like timer in RLC Rx side, RLC receiving side needs to monitor the status of PDCP timer t-Reordering and start a new timer in RLC accordingly. It seems to be complexity for RLC to maintain the interaction between two layers (PDCP and RLC) concurrently.

To avoid the interaction between RLC and PDCP, Rx side may consider to set the local timer per RLC PDU. When the SN gap was found in the Rx side, the local timer was triggered for each missing RLC PDU. For example, the local timer could be set based on the remaining time reported from DSR. It seems to be more accurate to sync the Tx and Rx side and avoid the window stalling.

Proposal 4: The local timer in Rx side may consider to be set per RLC PDU. When the SN gap was found in the Rx side, the local timer was triggered for each missing RLC PDU.

In the combined approach, Rx informs Tx side about the abandoned SDUs with the fake ACK and the Tx side move its transmission window as in legacy. The remaining time of a PDU is various but the local timer in the Rx side is a configured constant value,  therefore it seems to be difficult to sync the RX and TX side. When the transmission of a PDU in Tx side already be stopped by the expired of remaining time and receive a NACK for this PDU in the status report, which means that the local timer in Rx is still running and the Rx fails to receive this PDU successfully. As result, the Tx side has to ignore the NACK in the status report and the Tx window may be stalled till the Rx local timer expired and another status report with the fake ACK will be sent from Rx to Tx. In this case, the Tx could send the SN gap report to inform the receiving RLC side for the obsolete SDUs.  When the receiving side receive the SN gap report, it will ignore the local timer and send the fake ACk back to the transmtting side immediately. The RLC SN gap report may include the indications for the sets of SN or possibly SN range associated with the abandoned retransmission of RLC SDUs.

Proposal 5: When the transmission of a PDU in Tx side already be stopped by the expired of remaining time and receive a NACK for this PDU in the status report, the SN gap report from transmitting side could be used to notify the receiving RLC side for the obsolete SDUs. When the receiving side receive the SN gap report, it will ignore the local timer and send the fake ACk back to the transmtting side immediately.

Proposal 6: The RLC SN gap report may include the indications for the sets of SN or possibly SN range associated with the abandoned retransmission of RLC SDUs.

The indication from RLC to upper layer indicates the RLC failure when max retransmission has been reached, which may lead Radio Link Failure (RLF) and bring the interruption [4]. If the RLC AM entity discards an obsolete RLC SDU and the maximum number of retransmissions is reached, the indication from RLC to upper layer, which indicates max retransmission has been reached as RLC failure, shall not be triggered due to the retransmissions discarding.

Proposal 7: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Timely RLC retransmission

In the last meeting,  the following solutions were proposed in RAN2 to achieve the timely retransmissions:

- Option 1: Retransmission based on enhanced polling

- Option 2: Autonomous retransmission

Solutions for timely RLC retransmission

- Option 1: Retransmission based on enhanced polling

The polling delay is caused by the accumulated waiting time on that the number of transmitted PDU without polling increases beyond pollPDU or the total data volume of transmitted PDU without polling increases beyond pollByte, or the timer t-PollRetransmit expires.

Firstly , reducing the transmission delay for polling caused by timer t-PollRetransmit could be considered:

The timer t-PollRetransmit is used by the transmitting side of an AM RLC entity in order to retransmit a poll. The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling. The additional values could be introduced for short PBD of XR traffic, including [1,2,3,4]in milliseconds value, which is simple way to implement the timely retransmission.

T-PollRetransmit ::= ENUMERATED { ms1,ms2,ms3,ms4,ms5, ms10, ms15, ms20, ms25, ms30, ms35, ms40, ms45, ms50, ms55, ms60, ms65, ms70, ms75, ms80, ms85, ms90, ms95, ms100, ms105, ms110, ms115, ms120, ms125, ms130, ms135, ms140, ms145, ms150, ms155, ms160, ms165, ms170, ms175, ms180, ms185, ms190, ms195, ms200, ms205, ms210, ms215, ms220, ms225, ms230, ms235, ms240, ms245, ms250, ms300, ms350, ms400, ms450, ms500, ms800, ms1000, ms2000, ms4000, ms1-v1610, ms2-v1610, ms3-v1610, ms4-v1610, spare1}

Observation 5: The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling.

Proposal 8: The additional values of the timer t-PollRetransmit could be introduced for short PBD of XR traffic, including [1,2,3,4] in milliseconds value.

Secondly, reducing the delay for polling caused by the trigger mechanism could also be considered:

In legacy, when the total number of PDUs or the total data volume of PDUs is accumulated larger than the configured value pollPDU or  pollByte, the polling could be included in AMD PDU to request the status report from receiving side. In order to reduce the polling delay for the RLC SDUs with small remaining time value, the remaining time could be considered in the polling trigger mechanism. For example, when the remaining time is below a threshold for a PDU, which means this PDU turns to be a delay-critical data, a new value pollPDU or pollByte could be applied in the RLC entity to fasten the polling trigger procedure.

Proposal 9: The remaining time could be considered to enhance the polling trigger mechanism.

Option 2: Autonomous retransmission

In the autonomous retransmission mechanism, when the new trigger is received, the RLC PDUs or the segments of the RLC PDU could be early retransmitted to reduce the delay of retransmissions of RLC SDUs and RLC SDU segments. For RLC transmission entity,  the following mechanism could be considered to trigger the early retransmission.

- Option 1: A new indication from PDCP could be used to indicate the remaining time is below a threshold.

- Option 2: A new timer could be introduced associated with a particular RLC PDU to trigger the retransmission action base on the remaining time below a threshold.

- Proposal 10: The following mechanism could be considered to trigger the early retransmission:

- Option 1: A new indication from PDCP could be used to indicate that the remaining time of RLC SDU is below a threshold to trigger the early retransmission action.

- Option 2: A new timer base on the remaining time could be introduced associated with a particular RLC SDU to trigger the early retransmission action.

If autonomous retransmission is supported, multiple retransmissions could also be considered. The retransmissions could be transmitted in the same carrier or different carriers with more than one MAC entities, e.g. CA configuration. However if the retransmissions are transmitted 	in different carriers, the RLC entity could be restricted to serve the smallest resource of transmission opportunity indicated by lower layer from different MAC entities, which leads the resource waste. So it is suggested that the retransmissions could be transmitted in the same carrier. And the multiple retransmisson could be periodically transmitted till the maximum number of retransmissions is reached or the ACK of a PDU is received.

Observation 6: The retransmissions could be transmitted in the same carrier or different carriers with more than one MAC entities,e.g. CA configuration. However if the retransmissions are transmitted 	in different carriers, the RLC entity could be restricted to serve the smallest resource of transmission opportunity indicated by lower layer from different MAC entities, which leads the resource waste.

Proposal 11: If autonomous retransmission is supported, multiple retransmission transmitted in the same carrier could also be considered.

Conclusion

In this contribution, we discussed the RLC enhancements related issues. We propose:

Observation 1: When the gNB receives STATUS Report from UE in RLC AM mode, it counts the number of successful received PDUs in a PDU set and increase the counter accordingly. In the congestion case, the gNB could discard the remaining PDUs in a PDU set once K out of N packets was successfully received.

Observation 2: The content ration could also be used in the solutions for Avoidance of unnecessary re-transmission. If the Tx side could stop to retransmit the remaining PDUs in a PDU set once K out of N packets was successfully received.

Proposal 1: It is useful for gNB to be provided the content ratio information for discarding of DL PDUs during congestion for RLC AM mode.

Observation 3: For RLC UM PDU, currently, gNB has no idea of whether it had been successfully delivered due to the lack of RLC STATUS Report.  And RAN2 considers that it is not reliable for the gNB to derive an RLC UM PDU transmission status from related MAC HARQ feedback since the MAC HARQ is not 100% reliable.  It is hard for gNB to derive K PDUs in a PDU set which is successfully received to reconstruct the original content.

Proposal 2: It may not bring benefit for gNB to be provided with the content ratio information for discarding of DL PDUs during congestion for RLC UM mode.

Proposal 3: In combined approach, TX side should stop the transmissions of an outdated RLC SDU based on the indication from upper layer.

Observation 4:  To introduce the t-Reordering-like timer in RLC Rx side, RLC receiving side needs to monitor the status of PDCP timer t-Reordering and start a new timer in RLC accordingly. It seems to be complexity for RLC to maintain the interaction between two layers (PDCP and RLC) concurrently.

Proposal 4: The local timer in Rx side may consider to be set per RLC PDU. When the SN gap was found in the Rx side, the local timer was triggered for each missing RLC PDU.

Proposal 5: When the transmission of a PDU in Tx side already be stopped by the expired of remaining time and receive a NACK for this PDU in the status report, the SN gap report from transmitting side could be used to notify the receiving RLC side for the obsolete SDUs. When the receiving side receive the SN gap report, it will ignore the local timer and send the fake ACk back to the transmtting side immediately.

Proposal 6: The RLC SN gap report may include the indications for the sets of SN or possibly SN range associated with the abandoned retransmission of RLC SDUs.

Proposal 7: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Observation 5: The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling.

Proposal 8: The additional values of the timer t-PollRetransmit could be introduced for short PBD of XR traffic, including [1,2,3,4] in milliseconds value.

Proposal 9:The remaining time could be considered to enhance the polling trigger mechanism.

- Proposal 10: The following mechanism could be considered to trigger the early retransmission:

- Option 1: A new indication from PDCP could be used to indicate that the remaining time of RLC SDU is below a threshold to trigger the early retransmission action.

- Option 2: A new timer base on the remaining time could be introduced associated with a particular RLC SDU to trigger the early retransmission action.

Observation 6: The retransmissions could be transmitted in the same carrier or different carriers with more than one MAC entities,e.g. CA configuration. However if the retransmissions are transmitted 	in different carriers, the RLC entity could be restricted to serve the smallest resource of transmission opportunity indicated by lower layer from different MAC entities, which leads the resource waste.

Proposal 11: If autonomous retransmission is supported, multiple retransmission transmitted in the same carrier could also be considered.

Reference

- 3GPP TS 38.323 V18.3.0 (2024-09), Packet Data Convergence Protocol (PDCP) specification(Release 18)

- S2-2410999 Reply LS on Application-Layer FEC Awareness at RAN, qualcomm

- 3GPP TR 23.700-70 V19.0.0 (2024-09), Study on architecture enhancement for Extended Reality and Media service (XRM);Phase 2(Release 19)

- 3GPP TS 38.322 V18.0.0 (2023-12), Radio Link Control (RLC) protocol specification (Release 18).