3GPP TSG-RAN WG2 Meeting #128  	R2-2410350

# Orlando, USA, Nov 18th – 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0931 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for MUSIM in 38.300 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DualTxRx_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For MUSIM UAI for temporary capability restriction, both of the indication of the restriciton and the removal of the restriction are described in 20.4. However, for MUSIM early indication, only the indication is mentioned but the function of removal of this indication is missing. - The following yellow-highlighted sentence restricts that only when the MUSIM device in NW B is in RRC_CONNECTED state, the MUSIM device in NW A can send the MUSIM early indication for temporary capability restriction. However, there shouldn't be such restriction considering: a) when RAN2 reached the agreement to support this early indication, it didn’t mention such restriction; b) there is also no such restriction in stage 3 specification; c) Even for the MUSIM UAI for temporary capability restriction, it only mentions that “when the MUSIM device needs transmission or reception in Network B”, instead of restricting the NW B in RRC connected state. When it is allowed by Network A in SIB1, a MUSIM device can indicate to the Network A that its capabilities are temporarily restricted in RRCSetupComplete/RRCResumeComplete/RRCReestablishmentComplete message while the MUSIM device is already in RRC_CONNECTED state in Network B. For MUSIM operation, a MUSIM device in RRC_CONNECTED state in Network A may indicate its preference on temporary UE capability restriction or removal of restriction with Network A when the MUSIM device needs transmission or reception in Network B (e.g., including start/stop connection to Network B). - “forbidden” cannot be used and should be changed to “avoided” to align with TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add sentence to describe the case that UE can indicate to the Network A the removal of the temporary capability restriction reported with early indication via UE assistance information. - Change the sentence “while the MUSIM device is already in RRC_CONNECTED state in Network B” to “while the MUSIM device needs transmission or receptionis in Network B (e.g., including start/stop connection to Network B)”. - Change “forbidden” to “avoided”. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: MUSIM Inter-operability: No inter-operability is forseen as no functional change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of removal of MUSIM early indication for temporary capability restriction is missing in stage 2 specification. The case that MUSIM device in NW A can send the MUSIM early indication for temporary capability restriction is too restricted and not aligning with the stage 3 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 20.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

20.4	Temporary UE capability restriction and removal of restriction

For MUSIM operation, a MUSIM device in RRC_CONNECTED state in Network A may indicate its preference on temporary UE capability restriction or removal of restriction with Network A when the MUSIM device needs transmission or reception in Network B (e.g., including start/stop connection to Network B). Network A is NR and Network B can either be E-UTRA or NR. The MUSIM device may request a temporary capability restriction only after the Network signals via RRC that this is allowed.

When configured to do so, a MUSIM device can indicate one or more of the following temporary capability restriction or removal of restriction to the Network A:

-	A MUSIM device can explicitly request SCell(s) or SCG to be released;

-	A MUSIM device can indicate its preference on temporary maximum MIMO layers and/or supported channel bandwidth for specific serving cells for both UL/DL;

-	A MUSIM device can indicate its preference on the temporary maximum number of CCs per UL/DL;

-	A MUSIM device can indicate its preference on the concerned band(s) or band combination(s) (e.g. avoided and/or affected band(s) or band combination(s)) based on a band-filter list configured by network. For affected band(s) and band combination(s), this preference can include temporary maximum MIMO layers and/or supported channel bandwidth for both UL/DL;

-	A MUSIM device can indicate the measurement gap requirement changes.

When it is allowed by Network A in SIB1, a MUSIM device can indicate to the Network A that its capabilities are temporarily restricted in RRCSetupComplete/RRCResumeComplete/RRCReestablishmentComplete message while the MUSIM device needs transmission or reception in Network B (e.g., including start/stop connection to Network B). When configured to do so, a MUSIM device can also indicate to the Network A there is no temporary capability restriction via UE assistance information.

When a MUSIM device is in RRC_CONNECTED state in both Network A and Network B, if Network B is NR, it is up to UE implementation to select which network to request temporary UE capability restriction; if Network B is E-UTRA, the request for temporary UE capability restriction can only be performed on Network A.

End of Change