3GPP TSG-RAN WG2 Meeting #128	R2-2410351

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	8.2.4

Source:	Huawei, HiSilicon

Title:	A-IoT random access procedure

Document for: Discussion and Decision

1	Introduction

In this contribution, the details of A-IoT random access procedure are further discussed.

2	Discussion

2.1	Re-transmission for device ID

In RAN2#127bis meeting, we had the following agreements on re-transmission of D2R “response” for R2D “upper layer command” [1]. However, whether the “device ID” can be re-transmitted and how the “device ID” is re-transmitted are unclear.

Agreements for Random Access

- In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

To trigger the D2R re-transmission, the reader anyway needs to re-schedule the D2R resource to the same device (e.g. via AS ID). We understand the re-scheduled D2R resource itself can be regarded as a trigger for re-transmission. For 3-step CBRA, if the device transmitted the “device ID” in Msg3 and then receives a re-schedule D2R resource, the device can by default re-transmit the “device ID”. Thus, additional explicit NACK indication is unnecessary and just a redundant indication.

Observation 1:	For 3-step CBRA, re-scheduling D2R resource can be regarded as a trigger for Msg3 (device ID) data re-transmission, which means an explicit NACK indication is redundant/unnecessary.

However, for 2-step CBRA, random ID and device ID are sent together in Msg1, if the Msg1 is not received successfully by the reader, the reader even does not know whether there is an access attempt from a device. Even if the reader is able to identify that there is an access attempt, it cannot schedule a retransmission for an unknown device because of the failure reception of its AS scheduling ID or random ID in Msg1. Thus, it is impossible to support device ID re-transmission, the device will rely on the re-access since it can determine there is a contention resolution failure.

Observation 2a:	For 2-step CBRA, it is impossible to support Msg1 (device ID) data re-transmission, since the reader may not identify whether there is an access attempt from a specific device in case of Msg1 failure.

Observation 2b:	In case of Msg1 failure of 2-step CBRA, there is no much difference between re-access and immediate re-transmission of Msg1 (since the contention resolution has not succeeded yet).

Regarding CFRA, the “device ID” is transmitted in Msg1, if the reader sends A-IoT paging message for inventory case, if the device transmitted Msg1 and then receives a re-schedule D2R resource, the device can know that it is required to re-transmit the “device ID”, since for command case, it was agreed that the D2R re-transmission is triggered by repeated command. However, there is no much difference to perform the re-access if Msg1 fails, as the device needs to re-transmit from the very beginning Msg1.

Observation 3:	For CFRA, re-scheduling D2R resource can be also regarded as a trigger for Msg1 (device ID) data re-transmission, but there is no difference between re-access and immediate re-transmission of Msg1.

Proposal 1:	Do not consider Msg1 (device ID) data re-transmission in 2-step CBRA and CFRA cases, i.e., re-access is used to re-send the Msg1 data.

Moreover, for 3-step CBRA, there is an uncertainty on whether the “not receiving device ID” at reader side is caused by Msg2 reception failure or Msg3 transmission failure. Normally, the device will perform re-access (not sending the Msg3 at all) if there is no correct Msg2 received. However, the reader may not be able to differentiate this kind of failure and the actual Msg3 transmission failure. If the reader detects some D2R transmission but fails to decode the Msg3, re-scheduling the D2R resource should be sufficient. If the reader detects nothing, there could be either the Msg2 failure (in which case the device will perform re-access later), or the Msg3 transmission failure, in this case, it is better for reader to also repeat the same Msg2 content if the reader is not sure whether the “device ID” reception failure cause is due to the previous Msg2 failure or Msg3 failure itself.

Proposal 2:	As to Msg3 (device ID) data re-transmission in 3-step CBRA, RAN2 to discuss the following two alternatives:

Alt.1: AS layer D2R data re-transmission is not supported;

Alt.2: It can be triggered by the reader to re-schedule D2R resource to the same device without any upper layer R2D data/command (i.e., NACK triggered Msg3 (device ID) data re-transmission is not supported). The reader implementation can also include the previous Msg2 content, for triggering the Msg3 re-transmission.

2.2	R2D failure/success indication for re-access determination

In RAN2#127bis meeting, we agreed that explicit R2D failure/success feedback indication is supported for at least Msg3 for re-access, but following D2R data is FFS [1].

Agreements for Random Access

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

According to the same principle as for Msg3, reader can detect/determine the following D2R data transmission failure, based on the RAN1 defined timing relationship value TR2D_max. For the device side, to avoid the ambiguity whether the D2R data transmission fails or there is no following R2D data, an explicit R2D failure/success feedback indication is still necessary for the D2R data transmission, so that the device can determine whether to re-access later. Some companies may argue that device can just wait for the new trigger from the CN, but it requires additional handling from the CN as CN needs to identify the failure for a specific device and re-trigger the service. Using R2D failure/success indication does not introduce any further specification impacts, and reader applies the unified handling of sending R2D failure/success indication after it determines if the procedure is successful or failed. Thus, explicit R2D failure/success feedback indication can also apply to following D2R data after the RA procedure.

Proposal 3:	The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose.

To further discuss the use of R2D failure/success indication, based on the following agreement, we understand if the device already successfully finishes the AS procedure, e.g. including RA procedure and upper layer data transmission, the device can skip the response for the same service to avoid duplicated response.

Agreements on AIoT paging

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

Considering the RA procedures:

- For 3-step CBRA, it was agreed that R2D failure/success indication is supported to confirm the success of Msg3;

- For 2-step CBRA, “device ID” is transmitted together with “Msg1 random ID”, if the device receives Msg2 including the ID matched to random ID in Msg1, the devices can know that “device ID” is also successfully transmitted. Thus, a R2D success indication is not needed in Msg2;

- For CFRA, the very initial D2R message is “device ID”, which is equal to “Mgs3” in 3-step CBRA, and device also needs to know whether the D2R data transmission fails or there is no following R2D data. Thus, similar to R2D failure/success indication for Msg3 in 3-step CBRA, such R2D failure/success indication can also apply to “Mgs1” in CFRA. If the “Mgs1” in CFRA is not successfully transmitted, the UE will perform re-access.

Observation 4:	For 2-step CBRA, the explicit R2D feedback indication is not needed in Msg2, since the random ID matching can indicate the success of previous “device ID” transmission.

Observation 5:	For CFRA, the explicit R2D feedback indication also applies in Msg2 to confirm the success of previous “device ID” transmission in Mgs1.

If there are still following R2D message/command to send, the reader just continues sending it to device until the service is over, or the A-IoT AS procedure needs to be stopped due to the failure transmission/reception; in this case, the reader sends R2D failure/success indication to inform whether the A-IoT AS procedure is successful or not, so that device can know whether to skip the duplicated response for repeated paging. Thus, the R2D failure/success indication is used to confirm the successful ending of the A-IoT AS procedure for this service.

Proposal 4:	RAN2 clarifies that the "R2D failure/success feedback indication" is used to confirm the success for one device in the end of its A-IoT AS procedure for a service.

Proposal 5:	RAN2 confirms that the re-access for the contention-free initial access is supported, i.e., device may respond in re-access opportunities (e.g., in subsequent paging). It is up to the reader on whether to use contention-based or contention-free for the re-access.

In summary, from procedure perspective, there are two phases in one access round: 1) random access for contention resolution and 2) data transmission including re-transmission. After the first access round, reader has the chance to use the re-access round to handle the failed device(s) in the previous access round, including the failure caused by contention resolution failure and/or data transmission failure.

Note that the re-access opportunity may be needed anyway due to the contention resolution failure case. Reusing those re-access opportunities to also handle the data (re)transmission failure basically comes free and is up to reader implementation.

Figure 2.2-1 "Double insurance": Relationship between re-transmission and re-access

2.3	R2D messages for random access and re-access

To trigger devices to respond to the A-IoT service, A-IoT paging message is introduced to indicate the paging identifier information to select the device(s) that need to respond, and may also indicate the access occasion(s) for random access. Furthermore, RAN2 agreed that there will be a R2D message to provide access occasion(s) which can be used for re-access purpose [1].

Agreements for Random Access

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

Although A-IoT paging message can provide the access occasion(s) for random access, the reader may not have an accurate estimation on how many devices will access, so the number of access occasion(s) assigned in A-IoT paging message may not be optimal. To improve the efficiency of access, reader should be able to send a R2D message to adjust the value Q immediately based on observed collision situation. Reusing A-IoT paging message with paging identifier information for value Q adjustment wastes A-IoT resources since in this case paging identifier information is unnecessary, especially considering that there would be multiple R2D messages for finding a suitable value Q. In addition, the reader should be able to use one or more R2D message(s) to provide access occasion(s) for re-access, the paging identifier information is not needed either since device(s) already know whether it needs to respond this R2D message to perform re-access, e.g. based on contention resolution failure detection or received R2D failure/success indication.

Observation 6:	For the R2D message which is used to adjust the access occasions (value Q) for access or provide access occasions (value Q) for re-access, paging identifier information should not be included to avoid the waste of A-IoT resources and improve the efficiency of (re)access. Hence, a separate R2D message is needed (see type 2 in proposal 6).

From RAN2 perspective, as to the slotted-ALOHA procedure, reader first selects many devices and then distribute those devices into many “occasions”. Based on the TR 38.848 target device density, there could be up to thousands of devices to respond to the paging trying to perform the random access.

Observation 7:	From RAN2 perspective, slotted-ALOHA needs to support the distribution of many devices, selected by the one A-IoT paging message, into similar number of access occasions (value Q).

Observation 8:	One A-IoT paging message may select up to several hundred of devices (or possibly even more).

As to the RAN1 discussion on the access occasions in response to/assigned by one “R2D transmission triggering”, it can be X*Y access occasions, i.e. time-frequency resource block(s) (X time domain resource(s) and Y frequency domain resource(s)), which is a limited number (e.g., 2*4 in some cases). This is because the large SFO of A-IoT device limits the value of X, and the frequency-shift capability of A-IoT device limits the value of Y.

For example, if the total number of selected devices is similar to the total number of allocated access occasion Q =600, when the (X*Y) =10, it requires R=60 access occasion sets.

RAN1 Agreement RAN1 studies following: - A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). - The study includes - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency. - Size(s) for resource allocation in the time domain - Determination of the X time domain resource(s) by the device - Addressing timing errors for adjacent time domain resources due to residual SFO of the device Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following - How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1 - How a device determines the frequency domain resource for the D2R transmissions for Msg.1 Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1

Observation 9:	When reader intends to allocate many access occasions (e.g. Q=several hundred), it needs to allocate multiple (value R) blocks of X*Y access occasions, due to the limited number of X*Y (e.g. less than or about 10).

Therefore, it is necessary to support multiple “R2D transmission triggering” after one R2D message that indicates the value Q to allocate the number of access occasions for random access and re-access. Hence, a separate R2D message is needed (see type 3 in proposal 6).

Proposal 6:	Following 3 R2D message types are supported (FFS on the combination among those):

R2D trigger message type 1: i.e., the original purpose of A-IoT paging message, at least including the paging identifier information to select the device to respond;

R2D trigger message type 2: i.e., the “Query-like” message, at least including the assigned/adjusted number of following access occasions;

R2D trigger message type 3: i.e., “QueryRep-like” message and “R2D transmission triggering random access” in RAN1 agreement, which schedules a set of time/frequency Msg1 resources.

The following Figure 2.3-1 gives an example of how those 3 types of R2D message work.

Figure 2.3-1 Example of slotted ALOHA procedure

2.4	RA type down-selection

Compared to 3-step RA, 2-step RA requires a multiplexing processing in Msg1, i.e., multiplexing the random ID (an AS payload) and the data (upper-layer payload, e.g., NAS-like or application-layer payload). It needs a cross-layer processing when generating Msg1, causing additional processing complexity.

Observation 10:	The D2R multiplexing in Msg1 for 2-step RA (i.e. including both device ID and random ID) will cause some processing complexity.

Though 2-step CBRA can save one step compared to 3-step RA (Msg4 does not need to be always sent in 4-step RA), the efficiency will not be better than 3-step RA. By re-access procedure, once Msg1 collides, Msg1 needs to be retransmitted in a later access occasion. In 2-step CBRA, Device ID with random ID, e.g., 96+16 bits, needs to be retransmitted if Msg1 collides. In 3-step RA, just a random ID, e.g., 16 bits, needs to be retransmitted if Msg1 collides. By comparison, in 2-step CBRA, retransmitting a larger Msg1 in case of Msg1 collision will result in a larger latency.

On the other hand, allocating more access occasions can reduce Msg1 collision probability. But this causes more idle/empty occasions, which leads to wasted time and increases the total inventory latency.

Furthermore, to compare the performance of 2-step CBRA and 3-step RA procedures, we calculate and compare the total latency of identifying the same amounts of devices by these two methods, and get the following conclusions. The evaluation results are shown in Appendix.

Observation 11:	The 2-step CBRA can save one step of data transmission (to reduce the latency) in case of low probability of Msg1 collision. But, once we reduce the collision probability by using a large number of access occasions, the unused access occasions and failed Msg1 transmission will increase the latency.

Observation 12:	The 2-step CBRA has low efficiency in case of relatively high probability of Msg1 collision.

Observation 13:	Based on the simulation/calculation in the Appendix, the total latency of 2-step CBRA is larger than 3-step RA.

Observation 14:	The 3-step RA has universal applicability, while 2-step RA is only beneficial in very few scenarios.

Based on the above analysis, the WFs are:

- WF1: Only support 3-step CBRA.

To further consider the need of 2-step RA, there are two other WFs:

- WF2: Agree on the unified CBRA solution, i.e. Msg1 optionally with data.

Support both RA types but with a unified solution:

- A-IoT Msg1: random ID is mandatory, and whether to also include the optional upper layer data is controlled by reader;

- A-IoT Msg2 echoes the random ID for contention resolution.

The RA type to be used can be indicated by reader explicitly or implicitly. How to select RA type is up to reader implementation.

Proposal 7:	RAN2 to discuss the following down-selection:

WF1: Only support 3-step CBRA.

WF2: Agree on the unified CBRA solution, i.e., Msg1 optionally includes data.

  - A-IoT Msg1: random ID is mandatory, and whether to also include the optional upper layer data is controlled by reader;

  - A-IoT Msg2 echoes the random ID for contention resolution.

# 3	Conclusion

This contribution makes the following observations and proposals:

Observation 1:	For 3-step CBRA, re-scheduling D2R resource can be regarded as a trigger for Msg3 (device ID) data re-transmission, which means an explicit NACK indication is redundant/unnecessary.

Observation 2a:	For 2-step CBRA, it is impossible to support Msg1 (device ID) data re-transmission, since the reader may not identify whether there is an access attempt from a specific device in case of Msg1 failure.

Observation 2b:	In case of Msg1 failure of 2-step CBRA, there is no much difference between re-access and immediate re-transmission of Msg1 (since the contention resolution has not succeeded yet).

Observation 3:	For CFRA, re-scheduling D2R resource can be also regarded as a trigger for Msg1 (device ID) data re-transmission, but there is no difference between re-access and immediate re-transmission of Msg1.

Observation 4:	For 2-step CBRA, the explicit R2D feedback indication is not needed in Msg2, since the random ID matching can indicate the success of previous “device ID” transmission.

Observation 5:	For CFRA, the explicit R2D feedback indication also applies in Msg2 to confirm the success of previous “device ID” transmission in Mgs1.

Observation 6:	For the R2D message which is used to adjust the access occasions (value Q) for access or provide access occasions (value Q) for re-access, paging identifier information should not be included to avoid the waste of A-IoT resources and improve the efficiency of (re)access. Hence, a separate R2D message is needed (see type 2 in proposal 6).

Observation 7:	From RAN2 perspective, slotted-ALOHA needs to support the distribution of many devices, selected by the one A-IoT paging message, into similar number of access occasions (value Q).

Observation 8:	One A-IoT paging message may select up to several hundred of devices (or possibly even more).

Observation 9:	When reader intends to allocate many access occasions (e.g. Q=several hundred), it needs to allocate multiple (value R) blocks of X*Y access occasions, due to the limited number of X*Y (e.g. less than or about 10).

Observation 10:	The D2R multiplexing in Msg1 for 2-step RA (i.e. including both device ID and random ID) will cause some processing complexity.

Observation 11:	The 2-step CBRA can save one step of data transmission (to reduce the latency) in case of low probability of Msg1 collision. But, once we reduce the collision probability by using a large number of access occasions, the unused access occasions and failed Msg1 transmission will increase the latency.

Observation 12:	The 2-step CBRA has low efficiency in case of relatively high probability of Msg1 collision.

Observation 13:	Based on the simulation/calculation in the Appendix, the total latency of 2-step CBRA is larger than 3-step RA.

Observation 14:	The 3-step RA has universal applicability, while 2-step RA is only beneficial in very few scenarios.

Re-transmission for device ID

Proposal 1:	Do not consider Msg1 (device ID) data re-transmission in 2-step CBRA and CFRA cases, i.e., re-access is used to re-send the Msg1 data.

Proposal 2:	As to Msg3 (device ID) data re-transmission in 3-step CBRA, RAN2 to discuss the following two alternatives:

Alt.1: AS layer D2R data re-transmission is not supported;

Alt.2: It can be triggered by the reader to re-schedule D2R resource to the same device without any upper layer R2D data/command (i.e., NACK triggered Msg3 (device ID) data re-transmission is not supported). The reader implementation can also include the previous Msg2 content, for triggering the Msg3 re-transmission.

R2D failure/success indication for re-access determination

Proposal 3:	The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose.

Proposal 4:	RAN2 clarifies that the "R2D failure/success feedback indication" is used to confirm the success for one device in the end of its A-IoT AS procedure for a service.

Proposal 5:	RAN2 confirms that the re-access for the contention-free initial access is supported, i.e., device may respond in re-access opportunities (e.g., in subsequent paging). It is up to the reader on whether to use contention-based or contention-free for the re-access.

R2D messages for random access and re-access

Proposal 6:	Following 3 R2D message types are supported (FFS on the combination among those):

R2D trigger message type 1: i.e., the original purpose of A-IoT paging message, at least including the paging identifier information to select the device to respond;

R2D trigger message type 2: i.e., the “Query-like” message, at least including the assigned/adjusted number of following access occasions;

R2D trigger message type 3: i.e., “QueryRep-like” message and “R2D transmission triggering random access” in RAN1 agreement, which schedules a set of time/frequency Msg1 resources.

RA type down-selection

Proposal 7:	RAN2 to discuss the following down-selection:

WF1: Only support 3-step CBRA.

WF2: Agree on the unified CBRA solution, i.e., Msg1 optionally includes data.

  - A-IoT Msg1: random ID is mandatory, and whether to also include the optional upper layer data is controlled by reader;

  - A-IoT Msg2 echoes the random ID for contention resolution.

# 4	References

- Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China.

- Y. Maguire and R. Pappu, “An Optimal Q-Algorithm for the ISO 18000-6C RFID Protocol,” in IEEE Transactions on Automation Science and Engineering, 2009.

5	Appendix

Considering most transmission parameters and message sizes for A-IoT are still under study, we take the related parameters captured in TR 38.848, TR 22.840 and RFID spec. According to TR 38.848, we set:

- data rate as 500 bps and,

- the size of device ID as 96 bits (in SA1 TR 22.840).

According to the agreement on random ID size, we set:

- the size of random ID as 16 bits.

We assume each packet has an 8-bit header (including both AS and upper-layer headers). According to the feature of Slotted ALOHA, we assume each device randomly select an occasion to access in each round, the collided devices can re-access in the next round.

There are three possibilities for each occasion as below, and for each we give the estimated latency by considering the assumed data rate.

- Successful occasion: one access occasion is selected by one device and the access succeeds

  - 3-step RA (Figure 5-1(a)):

    - including one access occasion trigger message (8 bits), one Msg1 (8-bit header + 16-bit random ID = 24 bits), one Msg2 (8-bit header + 16-bit random ID = 24 bits), and Msg3 (8-bit header + 96-bit device ID = 108 bits)

  - 2-step RA (Figure 5-1(b)):

    - including one access occasion trigger message (8 bits), one Msg1 with data (8-bit header + 96-bit device ID + 16-bit random ID = 120 bits) and one Msg2 (8-bit header + 16-bit random ID = 24 bits);

- Collided occasion (Figure 5-1(c)): one access occasion is selected by multiple devices and the access fails

  - including one access occasion trigger message (8 bits) and one Msg1 (24 bits)/Msg1 with data (120 bits);

- Idle occasion (Figure 5-1(b)): one access occasion is not selected by any device

  - including one access occasion trigger message (8 bits) and a minimum waiting time to at least check detect a 8-bit header;

Figure 5-1(a):	Successful occasion of using Msg1 without data and with data

Figure 5-1(b):	Collided and Idle occasion of using without data and with data

Many existing papers illustrated that the theoretical optimal efficiency of Slotted ALOHA is about 36.8% [2], and the optimal efficiency can be achieved when the number of slots equals the number of devices. Hence, we assume the total number of slots to equal the number of unidentified devices in every round.

Table 5-1 and Figure 5-2 illustrate the performance results on latency for identifying different number of devices. It is observed that the 3-step RA can achieve a lower latency, and achieves more significant improvement of efficiency as the number of devices increases.

Table 5-1	Performance results on latency of Msg1 with/without data

| The number of devices | 200 | 180 | 160 | 140 | 120 | 100 | 80 | 60 | 40 | 20 | 15 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The overall latency for Msg1 with data (seconds) | 96.9 | 87.1 | 76.5 | 67.0 | 58.4 | 48.8 | 38.6 | 28.5 | 18.6 | 9.4 | 7.3 | 4.7 |
| The overall latency for Msg1 without data (seconds) | 69.1 | 62.7 | 55.5 | 48.6 | 41.9 | 34.9 | 27.8 | 20.7 | 13.7 | 6.9 | 5.2 | 3.4 |
| Average probability of collision = 26.4%, Average probability of success = 36.8%, Average probability of idle occasions = 36.8% |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 5-2	Curves of latency changes as the number of devices change

Furthermore, we give the results of allocating more occasions, in which a lower probability of collision (about 9.2%) can be achieved, i.e., in each round, the number of slots is set to twice the number of devices. As shown in Table 5-2 and Figure 5-3, 3-step RA performs better, and achieves more significant improvement of efficiency as the number of devices increases. Though 2-step RA works better with more allocated slots, but still cannot catch up with 3-step RA. The reason is that allocating more slots can achieve a lower collision probability, but the drawback is that there are more idle slots, which leads to wasted time.

According to the above results, though 2-step RA can save one step, the efficiency cannot be greater than 3-step RA because once Msg1 (including Device ID) collide, a longer packet will be wasted and be retransmitted in the next access occasion. On the other hand, allocating much more slots can reduce Msg1 collision. However, this causes more idle/empty slots at the same time, which increases the total latency.

Table 5-2	Performance results on latency of Msg1 with/without data

NOTE: the number of occasions is set to twice the number of devices in each round (i.e. more allocated occasions)

| The number of devices | 200 | 180 | 160 | 140 | 120 | 100 | 80 | 60 | 40 | 20 | 15 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The overall latency of Msg1 with data (seconds) | 83.18 | 74.80 | 66.44 | 58.13 | 49.58 | 41.59 | 33.05 | 24.66 | 16.59 | 8.25 | 6.29 | 4.11 |
| The overall latency of Msg1 without data (seconds) | 72.79 | 65.50 | 58.23 | 50.93 | 43.56 | 36.41 | 29.03 | 21.71 | 14.54 | 7.26 | 5.48 | 3.61 |
| Average probability of collision = 9.2%, Average probability of success = 29.9%, Average probability of idle occasions = 60.9% |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 5-3	Curves of latency changes as the number of devices change (more allocated occasion)