3GPP TSG-RAN WG2 Meeting #128									   R2-2410354

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.13.3

Source: 	NEC

Title:	Considerations on Service Continuity of Multi-hop Relay

Document for:	Discussion/Decision

- Introduction

The Rel-19 SID for Ambient IoT (also referred to as A-IoT) includes following objectives on service continuity [1].

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

This contribution discusses the scenarios (i.e., Scenario A and Scenario B) of the first priority.

- Discussion

  - Scenarios

The multi-hop indirect path comprises one additional hop relay on top of the Rel-17 U2N relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB). Considering Scenario B, the target single-hop indirect path may comprise the same last relay UE or a different last relay UE. Considering the random movement of Remote UE, both cases are possible.

The target single-hop indirect path may comprise the same last relay UE or a different last relay UE.

RAN2 to confirm that both the same last relay UE and a different last relay UE for the target single-hop indirect path will be studied.

  - Procedures

Owing to the disparity between the source and destination paths, the two scenarios involve distinct procedures.

    - Procedures for Scenario A

Given the source path is an indirect path and the target path is a direct path, reconfigurationWithSync without sl-PathSwitchConfig and the related processes in TS38.331 [2] can be used for Scenario A. The main difference from the processes of Rel-17 I2D procedure is the additional relay UE (i.e., the first relay UE). Then, an added step is introduced to release the first relay UE on the source indirect path.

Using reconfigurationWithSync without sl-PathSwitchConfig for path switching of Scenario A and an added step to release the first relay UE of the source indirect path.

    - Procedures for Scenario B

Given the source path is a multi-hop indirect path and the target path is a single-hop indirect path, reconfigurationWithSync including sl-PathSwitchConfig and the related processes in TS38.331 [2] can be used for scenario B. Similar to Scenario A, an added step to release the first relay UE on the source indirect path is needed.

Using reconfigurationWithSync including sl-PathSwitchConfig for path switching of Scenario B and an added step to release the first relay UE of the source indirect path.

- Conclusion

Based on the discussion in this document, we have the following observations and proposals:

Observation 1:	The target single-hop indirect path may comprise the same last relay UE or a different last relay UE.

Proposal 1:	RAN2 to confirm that both the same last relay UE and a different last relay UE for the target single-hop indirect path will be studied.

Proposal 2:	Using reconfigurationWithSync without sl-PathSwitchConfig for path switching of Scenario A and an added step to release the first relay UE of the source indirect path.

Proposal 3:	Using reconfigurationWithSync including sl-PathSwitchConfig for path switching of Scenario B and an added step to release the first relay UE of the source indirect path.

- Reference

- RP-241609, Rel-19 WID of NR Sidelink Relay.

- TS38.331-i30.