3GPP TSG-RAN WG2 Meeting #128  	                            R2-2410391

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.5.3

Source: 	Sony

Title:	Cell barring in on-demand SIB1 cells

Document for:	Discussion

# Introduction

In RAN2#127bis meeting, the agreements on OD-SIB1 were agreed as follows:

Agreements on OD-SIB1

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

In this contribution, we discuss leftover issue in cell barring for on-demand SIB1 cells.

# Discussion

In RAN2#127bis meeting, some details on cell barring for OD-SIB1 cells have been discussed. We had one agreement on the baseline of cell barring as follows in the study phase as follows:

If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

It is the understanding that this agreement applies to the scenario that UE has acquired the valid UL WUS configuration e.g., UE camps on Cell A and has acquired the valid UL WUS configuration from Cell A. There is a question on the scenario that UE camps on a NES cell initially but couldn’t get any valid UL WUS configuration from that camped NES cell, and if this happens, do we need to specify any additional/different behaviour from UE side e.g., the UE bars the cell as in legacy, but once reselecting to the Cell A and receiving the WUS, it would declare the SIB1-less cell as not barred anymore.

In TS38.331, the specify the following behaviour when legacy UE is not able to acquire the SIB1:

#### 5.2.2.5 Essential system information missing The UE shall: 2> else if the UE is unable to acquire the SIB1: 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> if the UE is a RedCap UE: 4> perform barring as if intraFreqReselectionRedCap is set to allowed; 3> else if the UE is an eRedCap UE: 4> perform barring as if intraFreqReselection-eRedCap is set to allowed; 3> else if the UE is a 2Rx XR UE: 4> perform barring as if intraFreqReselection2RxXR is set to allowed; 3> else: 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20].

And in TS38.304, the related cell barring UE behaviour is specified in the following:

If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1: - the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds. - the UE may select another cell on the same frequency if the selection criteria are fulfilled.

The justification to define a different/additional UE behaviour e.g. UE bars the cell without SIB1 in additional to our previous agreement is that if not barring the NES cell, the UE may be stuck in the NES cell. But with existing specified behaviour (UE bars the NES cell), the problem is still not solved in guarantee as it is a “may” behaviour.

Therefore, RAN2 should stick to our previous agreement i.e. NES UE will not bar the cell if it is unable to acquire the SIB1 of that initially selected NES Cell before it initiates OD-SIB1 procedure. After NES UE re-selects to the Cell A or any other NES cell to get the corresponding specific UL WUS configuration of that initially selected NES cell, UE may re-select to the NES cell and initiate the SIB1 request procedure. NES UE only bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Proposal 1: RAN2 stick to previous agreement i.e. if the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment.

# Conclusion

We propose RAN2 to discuss following proposal:

Proposal 1: RAN2 stick to previous agreement i.e. if the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment.