3GPP TSG-RAN WG2 Meeting #128	R2-2410416

Orlando, USA 18th Nov-22nd Nov 2024

Source: 	Vodafone

Title: 		Lean Inventory procedure/counting of devices

Document for: Decision

Agenda Item: X.X.X

## Introduction

RAN WG2 127 bis meeting the following conclusion was reached:

- For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

- For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

From these agreements we understand that MSG2 is optional message and would like to confirm this understanding.

Proposal 1: It is proposed to confirm that MSG2 is optional message which does not need to be sent by the reader/ received by the device all the time

RAN WG118, the agreement was reached to support Proximity determination:

Conclusion: Proximity determination is concluded to be feasible with either of the two solutions below. Potential specification impact or not will not be determined in the Rel-19 study item.

Solution 1

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission: then the device is determined as near to the reader based on measurements at the reader side

Solution 2

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near to the reader

At the same time Vodafone proposed in R2-2408453 to cover the scenarios where customers are inventorying groups of devices or all devices and would only like to know the number of the devices which have arrived or have left, are present or are missing under a particular location/locations.

## Discussion

As it can be seen from the decision made by RAN WG1 both proposed solutions enable the Lean Inventory procedure/counting procedure proposed in R2-2408453, as in each the reader determines that a particular device is “near” just from the reception of a D2R transmissions.

The following figure summarizes the communications flows required to realize  Lean Inventory procedure/counting procedure

Figure 1: Lean inventory/counting procedure

In order to minimise specification and device impact, it is proposed to re-use the current design assumptions as much as possible. In order to perform Lean Inventory procedure/counting and/or proximity determination the following steps are needed:

- trigger paging message to one, multiple or group of devices

- the UE answers with MSG 1 including (at least Randon ID of 16bits)

- If no contention resolution is needed, MSG 2 does not need to be sent.

Based on the successful reception of the D2R transmission (MSG1 the device is deemed near according to the proximity procedure(with or without the need for measurements of this message) and can be counted.

As it can be seen from above, the impact to the standard is limited to

- Potential addition of a “service type/cause value” to the paging

- Skipping of MSG2 in case of no contention

- No D2R transmission after MSG2

Observation 1: In order to support Lean Inventory procedure/counting procedure, the impact to the specification is very limited.

During online and offline discussions concerns were raised that inventory/counting cannot be based on the fact that MSG1 is received as there might be a high risk of contentions and the device does not know if it was counted or not.

In our view, this depends on the service/business requirements. If the exact counting is required (e.g. the system would like to count that e.g. 100% of the devices expected to be in the coverage of the reader are there), the system can always execute a normal inventory procedure where also MSG3 is delivered.

But often, it is not needed to count all the devices and often it is also not needed to know their exact IDs, as all what would be needed is to understand that a similar number compared to the last Lean Inventory procedure/counting is present under a particular coverage.

Observation 2: 100% reliable counting is not needed all the time and often it is sufficient to rely on statistically deviations between counting rounds to satisfy business requirements.

Proposal 2: It is proposed to call the procedure for counting (and proximity) reasons a lean inventory/counting procedure and it is proposed to support A-IOT lean inventory procedure within the A-IOT study item and include it into  TP 38.769.

The other question raised was about if the reader needs to indicate to the device (e.g. within paging message) that it is paged due to the counting procedure. Some companies argued that it is not needed as the device is always listening to the D2R channel and there is no difference from the device perspective if the device would receive MSG2 or not.

In our opinion, there is a difference between the case MSG2 is not sent by intention or not.

During paging procedure the device will receive a pool of resources where it should randomly select one to send MSG1. In case the device successfully selects the resource and sent the MSG1, it will consider the successful reception of this message. If the reader has not received the MSG1 (obviously the probability of not receiving MSG1 might depend on the message size, e.g. bigger in case of 2 Step CBRA), the device would not know it. As a consequence it might not respond to the paging repetitions as the device assumes that MSG1 was received by the reader. On the other hand, if it would know, that MSG2 has to be transmitted by the reader, it could react to the paging repetitions which would increase the reliability of a normal inventory procedure.

Observation 3: Depending on the design and reliability requirements, it might be of the advantage to inform the device if MSG2 is going to be sent or not.

The other reason, why it would be better to inform the device if A-IOT lean inventory procedure is expected is the construction of MSG3;

Depending on device capabilities, the device might also start constructing MSG3 just after reception of paging (this is needed for 2 step CBRA anyway). To do so, it would e.g. retrieve the device ID, generate the CRC, retrieve the keys, etc and buffer this information till MSG2 is received. These tasks would not be needed if the aim for paging is Lean Inventory procedure/counting procedure without MSG2. This would again save the power on the device.

Also from operational reasons, it is of the advantage to understand which amount of which services were triggered by the reader(s).

Also it is more future proof to have a service type/cause values within paging, especially, if more cases will be discovered in the future.

Observation 4:  It is proposed that the reader can indicate within the paging if the device should expect MSG2 after sending MSG1 or not.

Proposal 3: It is proposed to introduce a discrimination (e.g. service type or cause values) within A-IoT paging to let the device know what kind of inventory procedure is expected to be performed.

Here is one example of how it could be done:

Service Type: “Normal Inventory”:

- MSG1 is expected to be confirmed by MSG2 in all cases

After reception of MSG2, the UE will send MSG3 (in case of 3 step CBRA)

Service Type: “Lean Inventory procedure/counting”:

- MSG2 once received, might not provide any resources for MSG3 (in case of 2Step CBRA)

- The device is not expected to generate MSG3 (In case of 3 Step CBRA)

## Conclusions

In order to have a structured discussion, we would suggest to treat the proposals of this document in the following order:

Observation 1: In order to support Lean Inventory procedure/counting procedure, the impact to the specification is very limited.

Observation 2: 100% reliable counting is not needed all the time and often it is sufficient to rely on statistically deviations between counting rounds.

Observation 3: Depending on the design and reliability requirements, it might be of the advantage to inform the device if MSG2 is going to be sent or not.

Proposal 2: It is proposed to call the procedure for counting (and proximity) reasons a lean inventory/counting procedure and it is proposed to support A-IOT lean inventory procedure within the A-IOT study item and include it into  TP 38.769.

Proposal 1: It is proposed to confirm that MSG2 is optional message which does not need to be sent by the reader/ received by the device all the time.

Proposal 3: It is proposed to introduce a discrimination (e.g. service type or cause values) within A-IoT paging to let the device know what kind of inventory procedure is expected to be performed.