3GPP TSG RAN WG2 Meeting #128                          	   R2-2410427

Orlando, US, Nov. 18th – 22nd, 2024

Agenda item:	8.5.3

Source:	Qualcomm Incorporated

Title:	Discussion on On-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

At RAN #105, it was agreed that on-demand SIB1 is specified for Case #2. The updated objects are outlined with the follows [1].

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective. - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1. - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE. - RAN1 specification impact to support this feature should be minimized

In this contribution, we further discuss design considerations for on-demand SIB1.

# Discussion

## Indication of broadcasting on-demand SIB1

At RAN2 #127bis, broadcasting on-demand SIB1 was agreed with the following agreements [6].

Agreements 7. A cell for which SIB1 request configuration is available, can periodically broadcast SIB1. 8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Based on the above agreements, a NES cell on-demand SIB1 transmission has two modes (similar to the on-demand SI currently supported):

- Broadcasting: on-demand SIB1 is periodically broadcasted after triggered by a UL WUS.

- Not-broadcasting: on-demand SIB1 is only transmitted via UL WUS triggering.

Additionally, a UE is required to check the on-demand SIB1 transmission mode (i.e., broadcasting or not-broadcasting) after acquires an UL WUS configuration for an on-demand SIB1 transmission. Thus, an indication is needed for a UE to check the on-demand SIB1 transmission mode.

There are multiple options to indicate if the on-demand SIB1 is broadcasted or not.

Option 1: Indication in UL WUS configuration

For example, following the on-demand SI currently specified, the indicated may be included in the IE SchedulingInfo in current SIB1 as shown below.

If this is adopted for on-demand SIB1, an indication (e.g., sib1-BroadcastStatus) may be included in UL WUS configuration (e.g., like SchedulingInfo with si-BroadcastStatus for on-demand SI configuration in current SIB1).

A UE may check the indication included in an UL WUS configuration and then decide to transmit the UL WUS or not for on-demand SIB1 transmission.

When a NES cell decides to switch between “broadcasting” and “not-broadcasting”, it needs to updated this indication in its UL WUS configuration and also needs to transmit the updated UL WUS configuration to Cell-As or other NES cells supporting this NES cell’s UL WUS configuration. If such switching is frequent, it may cause undesired traffic among CellAs and NES cells.

Observation 1. With the indication indicated in UL WUS configuration, a UE may get the indication in the UL WUS acquired by the UE. No extra UE monitoring is needed.

Observation 2. With the indication included in UL WUS configuration, frequently switching between broadcasting and not-broadcasting causes more traffic with updated UL WUS configurations among Cell-As and NES cells which should be avoid.

Option 2: Indication in MIB

For example, an indication may be supported in MIB such as using the K_SSB.

A UE may check the K_SSB value in MIB and then decide to transmit the UL WUS or not for on-demand SIB1 transmission.

When a NES cell decides to switch between “broadcasting” and “not-broadcasting”, it needs to update this indication in its MIB and does not need to update UL WUS configuration. Thus, this option avoids the traffic among CellAs and NES cells with such switching.

Observation 3. With the indication included in MIB, there is no need to update the UL WUS configuration and thus no traffic with updated UL WUS configurations among Cell-As and NES cells.

Observation 4. With the indication indicated in MIB, a UE needs to decode the MIB before decides to transmit the UL WUS or not for on-demand SIB1.

With the observation of option 1 and option 2, we make the following proposal.

Proposal 1. RAN2 waits for RAN1’s decision on whether to support the switching between broadcasting and not-broadcasting before further discussion on the indication.

## Uplink WUS configuration

The following agreements were made at RAN2 #125bis for the contents of UL WUS configuration [2].

Agreements 1. RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1. 2. UL WUS configuration includes at least below information: • RACH configuration

More agreements were made at RAN2 #126 for UL WUS configuration [3].

Agreements 3. Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell. 4. For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Additionally, the following agreement was made at RAN2 #127 for UL WUS update indication from a NES cell [4]. This implies that a NES cell may contain its UL WUS configuration in an SI.

Agreement 4. Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

Furthermore, at RAN2 #127bis, more agreements were made for NES cell’s UL WUS configuration [6].

Agreements 2. A NES cell can include neighbouring NES cell’s WUS configuration. 3. NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration)

At RAN1 #118bis, agreements were made on UL WUS configuration parameters as the follows [5].

Agreement For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon

Agreement For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz (working assumption)ULSubCarrierSpacing.

Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. SSB-positionInBurst. tdd-UL-DL-ConfigurationCommon. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResources; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. Note: - In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB - Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A. Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2. Agreement For The repetition periodicity of SIB1 within the time window of on-demand SIB1,: - Up to NW implementation (no change to existing specification) Agreement RAN1 to discuss the contents of RAR in response to UL WUS using the legacy RAR for OSI as a starting point Agreement Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell. Agreement It is up to gNB to configure whether RACH occasions for UL WUS are shared or separated from the RACH occasions for other usages.

Based on the above agreements made by RAN1, RAN2 further discuss parameters included in UL WUS configuration, e.g., sib1-RequestConfig. An LS may be sent to RAN1 based on the discussion.

Proposal 2. RAN2 reviews and discusses the UL WUS configuration parameters agreed by RAN1 and considers an LS to RAN1 based on the discussion.

# Conclusion

In this contribution, we discussed scenarios or options for on-demand SIB1 operation and concluded with the following observations and proposals.

Indication of broadcasting on-demand SIB1

Option 1: Indication in UL WUS configuration

Observation 1. With the indication indicated in UL WUS configuration, a UE may get the indication in the UL WUS acquired by the UE. No extra UE monitoring is needed.

Observation 2. With the indication included in UL WUS configuration, frequently switching between broadcasting and not-broadcasting causes more traffic with updated UL WUS configurations among Cell-As and NES cells which should be avoid.

Option 2: Indication in MIB

Observation 3. With the indication included in MIB, there is no need to update the UL WUS configuration and thus no traffic with updated UL WUS configurations among Cell-As and NES cells.

Observation 4. With the indication indicated in MIB, a UE needs to decode the MIB before decides to transmit the UL WUS or not for on-demand SIB1.

Proposal 1. RAN2 waits for RAN1’s decision on whether to support the switching between broadcasting and not-broadcasting before further discussion on the indication.

## Uplink WUS configuration

Proposal 2. RAN2 reviews and discusses the UL WUS configuration parameters agreed by RAN1 and considers an LS to RAN1 based on the discussion.

# References

[1] RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

[2] “R2_125bis_SL_R19 NES_MOB_Session Notes (Kyeongin Jeong) _EoM”, RAN2 #125bis, Changsha, Hunan Province, China, April 15th – 19th, 2024.

[3] “R2_126_R18 SL_R19 NES_MOB_Session Notes (Kyeongin Jeong) _EoM”, RAN2 #126, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

[4] R2-2407571 “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127, Maastricht, NL, August 19th – 23rd, 2024.

[5] “Chair notes RAN1#118bis eom0” RAN1 #118bis, Hefei, China, October 14th – 18th, 2024

[6] “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127bis, Hefei, China, October 14th – 18th, 2024.