3GPP TSG-RAN WG2 Meeting #128	R2-2410841

Orlando, Florida, USA, 18th – 22nd November, 2024

Agenda item:	7.3.2

Source:	Samsung

Title:	Various corrections to IoT NTN Rel-18

WID/SID:	IoT_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

This contribution focuses on various corrections to IoT NTN Rel-18.

# Capabilities

## On acquiring SIB33 in connected mode

In RAN2#127 the following was agreed:

RAN2 understands that in any case there are no mandatory requirements for a UE to keep an updated version of SIB33 in Connected mode. Come back in the next meeting to check whether we can add another capability for a UE capable of keeping an updated version of SIB33 in Connected mode

One important consideration during the discussion is whether there would be a need to indicate that a UE is capable of acquiring SIB33 in RRC_CONNECTED, i.e during the T318 timer. As has been agreed in the past, whether to acquire SIB33 in connected mode is up to UE implementation, but if the UE decides to acquire the SIB33, then certain rules has to be followed. For instance, the UE will not stop T318 after having acquired SIB31, T318 is stopped after successfully acquiring both SIB31 and SIB33 and RLF is not triggered upon T318 expiry if SIB31 has been acquired. While having capabilities for a feature which has aspects that are up to UE implementation make it difficult to test, there could be value for the network to know. This may for instance indicate to the network that T318 is expected to run for a longer duration compared to when UE does not indicate the capability.

Proposal 1: Introduce capability for eMTC to indicate that the UE is capable of acquiring and maintaining SIB33 in connected mode.

# RRC corrections

## SIB33 correction

In eMTC, it is configurable which narrowband that system information may be broadcasted on, according to si-Narrowband in the field schedulingInfoList-BR in SIB1. The UE may be then be configured to operate on any narrowband. In NB-IoT, the system information may only be configured on the anchor carrier, but the UE may configured to operate on a non-anchor carrier.

In release 17, the following note was introduced in Section 5.3.18 of 36.331:

NOTE 1:	SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) may be broadcast on a different narrowband or different NB-IoT carrier than the one configured to the UE.

The note indicates that SIB31 may be broadcasted on another narrowband or NB-IoT carrier than where the UE is currently configured to be on, this implies (but does not specify) that the UE may need to move to another narrowband or NB-IoT carrier to acquire the SIB31 during T318.

SIB33 was introduced in Release 18 and similar to any other system information in eMTC, it may be configurable on any narrowband. Any type of restriction where the SIB33 can only be broadcast on the same narrowband or carrier would equal in the inability to deploy more than one narrowband or more than one NB-IoT carrier. Thus we think that there should not be any issue for adding SIB33 to the NOTE 1 of 5.3.18.

Observation 1: SIB33 can be broadcast on a different narrowband or different NB-IoT carrier than the one configured to the UE according to current specifications.

Proposal 2: Add SystemInformationBlockType33(-NB) to NOTE1 of 5.3.18.

# Conclusion

In this contribution we have the following observations and proposals:

Observation 1: SIB33 can be broadcast on a different narrowband or different NB-IoT carrier than the one configured to the UE according to current specifications.

Proposal 1: Introduce capability for eMTC to indicate that the UE is capable of acquiring and maintaining SIB33 in connected mode.

Proposal 2: Add SystemInformationBlockType33(-NB) to NOTE1 of 5.3.18.

# Text proposal

## RRC

------------------- Based on 36.331 V18.3.1 -------------------

5.3.18	T317 expiry

The UE in RRC_CONNECTED shall:

1>	if T317 expires and the UE is not performing GNSS measurement; or

1>	if indication is received that new GNSS position becomes valid and T317 has expired during the GNSS measurement; or

1>	if indication is received that new GNSS position becomes valid, and T317 has expired before the GNSS measurement, and timer T318 has been stopped upon the GNSS measurement:

2>	inform lower layers that the UL synchronisation is lost;

2>	start timer T318;

2>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) as specified in 5.2.2;

2>	if the UE acquires SystemInformationBlockType33 (SystemInformationBlockType33-NB in NB-IoT) as specified in 5.2.2:

3>	inform lower layers when UL synchronisation is restored upon successful acquisition of SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT);

3>	stop timer T318 when both SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) and SystemInformationBlockType33 (SystemInformationBlockType33-NB in NB-IoT) are acquired;

2>	else:

3>	upon successful acquisition of SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT):

4>	stop timer T318;

4>	inform lower layers when UL synchronisation is restored.

NOTE 1:	SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) and SystemInformationBlockType33 (SystemInformationBlockType33-NB in NB-IoT) may be broadcast on a different narrowband or different NB-IoT carrier than the one configured to the UE.

NOTE 2:	The exact time when UL synchronisation is restored (after SystemInformationBlockType31 or SystemInformationBlockType31-NB in NB-IoT is acquired) is left to UE implementation, which can be from the subframe indicated by epochTime and optionally before the subframe indicated by epochTime.

NOTE 3:	For UEs not capable of performing system information acquisition and GNSS measurement at the same time, if the UE cannot complete acquisition of SystemInformationBlockType31 (SystemInformationBlockType31-NB) before the start of GNSS measurement gap, acquisition of SystemInformationBlockType31 (SystemInformationBlockType31-NB) may be postponed until GNSS measurement is completed, and T318 is restarted after GNSS measurement is completed.

------------------- Based on 36.331 V18.3.1 -------------------