3GPP TSG-RAN WG2 Meeting #128		R2-2410491

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item: 	8.7.1

Source:		Sony

Title:	Discussion on LS from SA2 on PDU Set information

Document for:	Discussion & Decision

# Introduction

RAN2 (at RAN2#127bis [3] has received two LSes from SA2 regarding handling of PDU sets,

R2-2409525 [1] on Application-Layer FEC Awareness at RAN which is a response to the RAN2 LS on Application-Layer FEC Awareness at RAN

The following 2 questions were stated:

Question 1: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode based on the above principles. Please also provide feedback on applicability of RLC AM for XR services, also considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget as per RP-240791.

Question 2: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC UM based on the above principles.

R2-2409533 [2] on PDU Set Information Marking Support without QoS parameters

In this LS the Q1 was provided to RAN2

Q1:SA2 would like RAN2 to provide feedback (e.g. the usefulness) on the PDU Set based handling without PDU set QoS parameters.

In this contribution we discuss the response to these questions.

# Discussions

The content ratio information on the FEC coding in the application layer of a PDU Set gives the information on how much redundancy information is added to the sent data. This allows that some packets within the PDU set are lost during the transmission and still the information is correctly received.

When a number of redundancy bits are added in the FEC giving the content ratio (amount of information bits divided by the total amount of transmitted bits), number of packets (PDUs) which can be discarded during transmission can be estimated. Therefore when RAN is transmitting the PDU Set to the UE and there is congestion, meaning that not all PDUs in the PDU Set can be transmitted in time, the RAN should know a minimum number of packets, need to be transmitted and successfully received in time for the UE to be able to decode (in application layer) the information in the PDU Set.

In order for the FEC discarding to work with low delay, the transmission needs to be in TM mode so all PDUs are directly forwarded to higher layers and the application in the UE. Then the application can start decoding the data as soon as enough data is received. Otherwise, the UM or AM modes in RLC needs to be adjusted in order to not delay the whole PDU set based on some late or discarded PDUs.

Observation 1: It is feasible to use the content ratio information to decide whether some PDUs in a PDU Set can be discarded or alternatively the whole PDU Set can be discarded since it is estimated that the PDU Set will not be sent in time.

Proposal 1: Send a Response LS to SA2 describing that it is feasible for RAN to use the content ratio information to decide whether PDUs can be discarded.

Proposal 2: Describe that the discarding works in RLC TM mode. The UM and AM modes may need to be adapted, due to the reassembly of RLC PDUs in these modes and the retransmission in AM mode, to work without extra latency when discarding PDUs in RAN.  RAN2 is still working on RLC AM enhancements.

Regarding Q1 in [2]. SA2 would like RAN2 to provide feedback (e.g. the usefulness) on the PDU Set based handling without PDU set QoS parameter. To be able to support and decide on discarding of PDUs related to the PDU Set it is important. Sometimes PDUs need to be discarded due to congestion etc, then it is important to know if it is a portion of a PDU Set which is discarded since there is a risk the whole PDU Set will not be useful at the receiver when a given number of PDUs are discarded.

Observation 2: The handling of a PDU set can be improved in RAN even without PDU set QoS parameters since it can be improved during e.g. congestion where it is known that the PDUs are part of a larger message.

Proposal 3: Describe in a response LS to SA2 that it is useful in RAN to know that the information of PDU sets is useful also without the PDU Set QoS parameters.

# Summary

Observation 1: It is feasible to use the content ratio information to decide whether some PDUs in a PDU Set can be discarded or alternatively the whole PDU Set can be discarded since it is estimated that the PDU Set will not be sent in time.

Observation 2: The handling of a PDU set can be improved in RAN even without PDU set QoS parameters since it can be improved during e.g. congestion where it is known that the PDUs are part of a larger message.

Proposal 1: Send a Response LS to SA2 describing that it is feasible for RAN to use the content ratio information to decide whether PDUs can be discarded.

Proposal 2: Describe that the discarding works in RLC TM mode. The UM and AM modes may need to be adapted, due to the reassembly of RLC PDUs in these modes and the retransmission in AM mode, to work without extra latency when discarding PDUs in RAN.  RAN2 is still working on RLC AM enhancements.

Proposal 3: Describe in a response LS to SA2 that it is useful in RAN to know that the information of PDU sets is useful also without the PDU Set QoS parameters.

# References

- R2-2409525 SA2 LS on Application-Layer FEC Awareness at RAN which is a response to the RAN2 LS on on Application-Layer FEC Awareness at RAN,

- R2-2409533, SA2, LS on PDU Set Information Marking Support without QoS parameters

- Minutes RAN2#127bis