3GPP TSG-RAN WG2 #128	R2-2410499

# Orlando, USA, 18th – 22th November 2024

Agenda Item:	 8.1.2.3

Source:	Ericsson

Title:	LCM For Positioning

Document for:	Discussion

# 1	Introduction

RAN2#127bis reached the following agreements on the topic of LCM for the positioning Case 1.

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In this paper we discuss the below aspects for the 1st priority Case 1 (UE-based positioning with UE-side model, direct AI/ML positioning):

- LCM Remaining procedure

- New Method or update of existing method

- Stage 2 Draft CR

# 2	Discussion

## LCM Remaining procedures

One of the procedures needed for AI/ML LCM is that if UE is unable to derive location using AI/ML, the UE should indicate to the NW with some failure cause. Depending upon the cause code, NW will decide whether to abort, use any other positioning method (non-AI/ML).

UE should provide the cause code if AI/ML based result was not produced. This would let the NW decide on next step such as change of positioning method (non- AI/ML based procedure) or aborting the procedure.

#### NR-AI-ML-TargetDeviceErrorCauses

The IE NR-AI-ML-TargetDeviceErrorCauses is used by the target device to provide NR AI/ML positioning error reasons to the location server.

-- ASN1START

NR-AI-ML-TargetDeviceErrorCauses-r19 ::= SEQUENCE {

cause-r19		ENUMERATED {	assistanceDataInconsistentBetweenTrainingAndInference,

thereWereNotEnoughSignalsReceivedForUeBasedAI-ML,

resourceOrProcessingCapacityIssueForAIML,

battery-low,

...

},

...

}

-- ASN1STOP

UE provides error/cause code when AI/ML based result is not produced.

## New Method or Update of Existing method

In order to minimize specification impact, we think update of existing method as shown below for a potential stage2 update as below. The changes needed as compared to existing DL-TDOA UE based positioning method is shown in yellow highlights.

#### 8.X.1.1	UE-based positioning with UE-side model

##### 8.X.1.1.1	UE-based positioning with UE-side model Positioning procedure

UE-based positioning with UE-side model is based upon DL-TDOA positioning method.

Figure 8.X.1.1-1: UE Based DL-TDOA positioning method with UE side AI/ML model.

0.	The LMF may use the procedure in Figure 8.X.1.1-1 to obtain the TRP information required for DL-TDOA positioning.

1.	The LMF may request the positioning capabilities of the target device using the LPP Capability Transfer procedure described in clause 7.1.2.1. The UE includes capability for UE-based UE side direct AI/ML.

2.	The LMF sends an LPP Provide Assistance Data message to the target device as described in clause 8.10.3.1.2.1. The message includes any required assistance data for the target device to perform the necessary DL-PRS measurements and enquires if AD is applicable for the AI/ML inference.

3. The UE responds and indicates its applicabilty conditions such as PRS Configuration, validity area, time.

4.	The LMF sends an LPP Request Location Information message to request Location information using AI/ML model inference and any fallback configurations.

5.	The target device performs the channel measurements based upon DL-PRS and generates the result using the AI/ML model inference.

6.	The target device reports the location information to the LMF in a LPP Provide Location Information message and providing information whether the result has been derived using AI/ML model.

Note: If the UE does not indicate that the location was derived using AI/ML, it would imply that UE failed to use the AI/ML model for inference (e.g. additional conditions for using the model inference were not met, battery or processing issue etc.).

7.	The LMF may provide model performance feedback to the target device.

However, RAN2 need to wait for RAN1 before deciding upon new method or update of existing method. If UE can perform AI/ML inference without the same set of AD as for legacy DL-TDOA procedure then a new method is more apt.

The decision on new method or existing method depends upon whether the same AD as in legacy can be reused or not. RAN2 need to wait for RAN1 on AD before deciding.

Wait for RAN1 to provide the Assistance data parameter list before deciding on new method or existing method.

## Training Data Collection for Case 1

### 2.3.1 Using Legacy Procedure for AD request for training

As per RAN1 agreements (Only keeping option for Case 1):

Agreement For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

The training data collection for Case 1, which only requires PRS configuration, is similar to existing procedure where UE requests assistance data of PRS configuration for legacy positioning measurements. Thus legacy procedure/ways for UE to request PRS configuration including UE-initiated assistance data request and UE-initiated on-demand PRS procedure can be reused. The training data collection for Case 1 is part of procedure for AI/ML positioning, and the PRS configuration request for this should not be either prioritised or deprioritized compared with legacy positioning, thus UE does not include the purpose of request when sending request for PRS configuration.

For data collection for AI/ML models on the UE side, the UE may request assistance data to LMF using UE-initiated assistance data request procedure as specified in clause 8.12.3.1.2.2 of TS 38.305 or UE-initiated on-demand PRS procedure as specified in clause 7.6.2 of TS 38.305 without the need to include the purpose of request.

Legacy procedure can be reused for data collection for case1.

### 2.3.2 Generating Label for data collection and associated parameters

RAN1 Agreements Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope.

AI/ML is used in area where there can be NLOS. Further, the PRS symbols may not be transmitted with perfect sync and there may be Real Time Difference. The LMF should be able to guide what is the expected positioning error during training phase to the UE based upon related Integrity parameters (e.g., standard deviation of RTD) so that the UE can understand if the obtained positioning can be used as a label (ground truth) or should be discarded. Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for AI/ML training may provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for the case of AI/ML training can provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

LMF provides the guidance for trustworthy label based upon Positioning Integrity (maximum allowed Positioning error or uncertainty for a trustworthy label)

# Conclusion

In the previous sections we made the following observations:

Observation 1	UE should provide the cause code if AI/ML based result was not produced. This would let the NW decide on next step such as change of positioning method (non- AI/ML based procedure) or aborting the procedure.

Observation 2	The decision on new method or existing method depends upon whether the same AD as in legacy can be reused or not. RAN2 need to wait for RAN1 on AD before deciding.

Observation 3	For data collection for AI/ML models on the UE side, the UE may request assistance data to LMF using UE-initiated assistance data request procedure as specified in clause 8.12.3.1.2.2 of TS 38.305 or UE-initiated on-demand PRS procedure as specified in clause 7.6.2 of TS 38.305 without the need to include the purpose of request.

Observation 4	Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for the case of AI/ML training can provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE provides error/cause code when AI/ML based result is not produced.

Proposal 2	Wait for RAN1 to provide the Assistance data parameter list before deciding on new method or existing method.

Proposal 3	Legacy procedure can be reused for data collection for case1.

Proposal 4	LMF provides the guidance for trustworthy label based upon Positioning Integrity (maximum allowed Positioning error or uncertainty for a trustworthy label)

# References

Tdoc Number, Title, Source, Meeting, Date

Spec number, Title, Source, Version, Date