3GPP TSG-RAN WG2 Meeting #128	R2-2410524

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.12.3

Source:	Samsung

Title:	Discussion on UE-initiated Beam Reporting and CSI enhancement

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In RAN2#127bis meeting, RAN2 had some initial discussion on UE initiated/event-triggered beam reporting and CSI enhancement aspects.

UE initiated/event-triggered beam reporting - Xiaomi prefer to leave those procedure in PHY so that R2 spec is simplified. OPPO think this is the R1-centric feature and we’d better wait instead of complicating things. - CMCC think in R2 we can conclude on the NW control in terms of how UE report the BM. - Vivo prefer to capture the evaluation procedure of event trigger BM in MAC. Ericsson, QC, MediaTek agree. - Apple think in R1 it is FFS which layer is handling the event triggered BM so prefer to wait for R1’s progress. CATT, ZTE, LG E share this view. - HW think R2 should also discuss and think the procedure will not be transparent to R2 spec, and prefer to have common framework for event triggered BM and LTM. LG E think these two are separate and no need to couple. OPPO think we can discuss the solutions for them separately and can check further how to capture them in the spec. - ZTE prefer to wait for R1 for both RRC and MAC impact. Chair: from the high level discussions, there are different views regarding whether R2 should wait for R1 on all the procedures, also there are different views regarding how to handle LTM and event triggered BM (e.g., whether they share a common framework.) CSI enhancement - Ericsson think we just follow the R1 parameters.

In this paper, we share our views on UE initiated/event-triggered (UEI) beam reporting from RAN2 perspective and discuss some RRC parameters for CSI enhancements.

# Discussion

## 2.1 UEI beam reporting

Based on RAN1 progress up to last meeting, we summarize the current agreed procedure of UEI beam reporting from several aspects as follows. The remaining FFSs in RAN1 are not included, which may lead to additional procedure and details.

- Measurement object: intra/inter-cell SSB, and periodic CSI-RS for BM

  - The same RS type for RS measurement for current beam and new beam is ensured implicitly:

    - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled, i.e., RS for current beam is the QCL RS in the indicated TCI state;

    - otherwise, Scheme-2 is enabled, i.e., the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

- Trigger events

  - Event-2:

    - Current beam: QCL RS of indicated TCI state, or SSB QCLed to the indicated TCI state

    - New beam: explicitly configured in RS resource set associated to the report configuration

    - Basic trigger: L1-RSRP of at least one new beam becomes a threshold (RRC configured) better than the current beam

    - Report content: CRI/SSBRI and L1-RSRP (differential value) for N ≥ 1 beam(s) (and current beam if enabled by RRC) in the report instance, with at least one beam meeting the event

    - Optional trigger by UE capability: If within a configurable time window, the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable conter M (counter M is per new beam).

  - (Working assumption) Event-1: Quality of the current beam is worse than a certain threshold.

  - (Working assumption) Event-7: Quality (e.g., L1-RSRP) of at least one new beam becomes a threshold (RRC configured) better than the Q-th best beam among the activated TCI states

- Report procedure

- Mode A (report in UCI using DG)

- Step 1: Transmit report notification in PUCCH (FFS dedicated SR or new UCI in dedicated periodic PUCCH)

- Step 2: Receive DCI including CSI trigger state associated with UEI beam report configuration(s) and DG for UCI,

- Step 3: Transmit report in UCI using DG PUSCH

  - Mode B (report in UCI using type-1 CG)

    - Step 1: Transmit report notification in PUCCH (FFS dedicated SR or new UCI in dedicated periodic PUCCH)

    - Step 2: Transmit report in UCI using type-1 CG PUSCH, after at least X symbols

      - One-to-one mapping between the PUCCH resource and type-1 CG PUSCH resource: Only one periodic PUCCH resource and only one type-1 CG with the same periodicity can be associated with the UEI beam report configuration.

      - The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report, which can carry any other UCI but can NOT carry UL-SCH.

  - Support cross-CC report for both modes, the report notification in PUCCH and report in PUSCH should be in the same CG and can be from the same or different CC(s).

Based on RAN1 LS R1-2407285, the UEI beam reporting configuration includes at least

- Enable mode A/B,

- event-2 (threshold, time window, counter M) and report content (N, report current beam or not),

- new beam resource set for event-2,

- etc..

We propose to introduce a new IE under CSI-ReportConfig to include the above parameters for UEI beam reporting.

Proposal 1: Introduce a new IE under CSI-ReportConfig to include the UEI beam reporting configuration.

On the MAC aspect, in the last meeting we initially discussed RAN2 impacts for the following two issues.

- Issue-1: whether to use common framework for UEI beam reporting and LTM event-triggered L1 measurement reporting.

- Issue-2: whether to capture UEI beam report event evaluation/triggering procedure in MAC or in RAN1 specification.

For Issue-1, since UEI beam reporting and LTM event-triggered L1 measurement reporting are progressed in different WIs and will be continued separately as RAN plenary confirmed, we think there is no need to align the two. It will definitely bring more coordination efforts in order to build a common framework. What’s more important, the event evaluation/triggering of the two features are technically different in our view, which cannot share a common framework.

For UEI beam reporting, as summarized above,

- the basic trigger is that L1-RSRP of at least one new beam becomes a threshold (RRC configured) better than the current beam;

- the optional trigger up to UE capability is that in a configurable time window the L1-RSRP of one same new beam becomes a threshold (RRC configured) better than the current beam for at least M times ( counter M is per new beam).

- Only Event-2 has been agreed so far.

For LTM event-triggered L1 measurement reporting,

- TTT (like L3 event triggered MR) is used for event evaluation procedure.

- A list of events similar to L3 events are agreed.

Observation 1: Different events, event evaluation, and reporting trigger are agreed for UEI beam reporting and LTM event- triggered L1 measurement reporting.

For Issue-2, RAN1 has been discussed for several meetings on the details of event evaluation/triggering, including whether/how to reset the time/counter, whether to capture event evaluation/triggering in MAC or in RAN1 specification, whether to use dedicated SR or a new UCI as report notification in PUCCH, etc.. Considering the current RAN1 progress and their leading role on the whole feature, we think it is appropriate to wait for RAN1 progress. Although MAC handles time/counter operation as a convention, the complete procedure is still not clear for now and up to detailed discussion in RAN1. It is hard for RAN2 to start a parallel discussion, which may also complicate the work at the current stage. We expect RAN1 will make decision on which layer to handle the procedure in November meeting, so that RAN2 can discuss more based on it.

Observation 2: RAN1 is discussing details of event evaluation/triggering and will make decision on which layer to handle the procedure in November meeting. It is hard and not necessary for RAN2 to start a parallel discussion, which may also complicate the work at the current stage.

Proposal 2: RAN2 wait for more RAN1 progress on the event evaluation/triggering procedure.

## 2.2 CSI enhancements

We group several objectives in WID into CSI enhancements , including increased number of ports (up to 128) for CSI, inter-TRP calibration for CJT, and 3Tx for UL.

Here we discuss the CBSR (codebook subset restriction) parameters which are informed in particular by RAN1 LS R2-2404114.

[116bis] Agreement For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, refine the legacy CBSR as follows: - Only 1-bit hard restriction is supported (analogous to Rel-18 Type-II) - Moving (N1, N2) configuration out from CBSR IE and the CBSR can be optional configured - Send LS to RAN2, and subject to RAN2 consent - Group-based CBSR granularity where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction - FFS: Value(s) of X1 and X2 and detailed design/spec impact FFS: Whether/how to enable shared CBSR in RRC configuration for Type-I/-II codebooks with a same (N1,N2). [116bis] Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding CBSR design: - 1-bit hard restriction is supported (analogous to Rel-15 Type-I) - FFS: 3-bit scaling factor for soft restriction is supported with the scaling factor taken into account in CQI/PMI calculation - Moving (N1, N2) configuration out from CBSR IE and the CBSR can be optional configured - Send LS to RAN2, and subject to RAN2 consent - N1⋅O1⋅N2⋅O2X1⋅X2-bit CBSR where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction - FFS: Value(s) of X1 and X2 and detailed design/spec impact FFS: Whether/how to enable shared CBSR in RRC configuration for Type-I/II codebooks with a same (N1,N2).

Further, in RAN1 LS R1-2407285 more information on the relevant parameters are provided. For Rel-19 type-I single-panel codebook, the following parameters are required by RAN1.

As informed, CBSR bit string is optional. The length of the CBSR bit string is (N1O1N2O2)/(X1X2), where (N1, N2) and (X1, X2) take the defined values, and O1=O2=4 according to RAN1 agreement. Table1 below shows the CBSR bit string length for different cases for Rel-19 type-I single-panel codebook.

Table 1: CBSR bit string length for Rel-19 type-I single-panel codebook

For Rel-19 type-II and type-II Doppler codebook, the following parameters are required by RAN1.

As informed, CBSR bit string is optional. The length of the CBSR bit string is (N1N2)/(X1X2), where (N1, N2) and (X1, X2) take the defined values. Table 2 below shows the CBSR bit string length for different cases for Rel-19 type-II and type-II Doppler codebook.

Table 2: CBSR bit string length for Rel-19 type-II codebook

In Rel-15/16/17/18 (N1,N2) and CBSR configuration, the signaling of (N1,N2) and CBSR bit string are coupled since the length of CBSR bit string is a function of (N1,N2). Although CBSR is not a mandatory parameter for a configured codebook, it is always signaled with (N1,N2) since (N1,N2) is a mandatory parameter for a configured codebook. The cost of such signaling design is overhead, that is even if CBSR is not required, a bit string of all 0s is still signaled. In Rel-18, in order to make CBSR bit string optional, a CHOICE of no CBSR is used.

N1-N2-CBSR-r18 ::=     CHOICE {

two-one-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (8))},

two-two-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (27))},

four-one-r18           CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (16))},

three-two-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (35))},

six-one-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (24))},

four-two-r18           CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (43))},

eight-one-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (32))},

four-three-r18         CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (59))},

twelve-one-r18         CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (48))},

four-four-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (75))},

eight-two-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (75))},

sixteen-one-r18        CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (64))}

}

For Rel-19 codebook, as informed by RAN1 LS R2-2404114, (N1, N2) configuration should be moved out from CBSR IE. If we reuse the Rel-18 CHOICE structure, the CBSR bit string has to be signaled also as a CHOICE of (X1, X2), which leads to CHOICE embedded in 3 layers.

Alternatively, (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2). Note there are many (X1, X2) values not applicable to type-II and type-II Doppler codebook, but only applicable to type-I single-panel codebook. This can be directly reflected by signaling CBSR as a CHOICE of (X1, X2).

Proposal 3: For Rel-19 type-I single-panel, type-II, type-II Doppler codebook, n1-n2 is configured as a separate parameter not coupled with CBSR, and CBSR is optionally configured as a CHOICE of (X1, X2). Take the below signaling structure as baseline.

n1-n2-r19		ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeI-codebookSubsetRestriction-r19		typeI-X1-X2-CBSR-r19							 OPTIONAL    -- Need R

typeI-X1-X2-CBSR-r19 ::=     CHOICE {

one-one-r19            CHOICE {twentyfour BIT STRING (SIZE (384)), thirtytwo BIT STRING (SIZE (192)), sixtyfour BIT STRING (SIZE (1024))},

two-one-r19            CHOICE {twentyfour BIT STRING (SIZE (192)), thirtytwo BIT STRING (SIZE (256)), sixtyfour BIT STRING (SIZE (512))},

two-two-r19            CHOICE {twentyfour BIT STRING (SIZE (96)), thirtytwo BIT STRING (SIZE (128)), sixtyfour BIT STRING (SIZE (256))},

four-one-r19           CHOICE {twentyfour BIT STRING (SIZE (96)), thirtytwo BIT STRING (SIZE (128)), sixtyfour BIT STRING (SIZE (256))},

four-two-r19           CHOICE {twentyfour BIT STRING (SIZE (48)), thirtytwo BIT STRING (SIZE (64)), sixtyfour BIT STRING (SIZE (128))},

four-four-r19          CHOICE {twentyfour BIT STRING (SIZE (24)), thirtytwo BIT STRING (SIZE (32)), sixtyfour BIT STRING (SIZE (64))}

}

typeII-codebookSubsetRestriction-r19		typeII-X1-X2-CBSR-r19							 OPTIONAL    -- Need R

typeII-X1-X2-CBSR-r19 ::=     CHOICE {

one-one-r19            CHOICE {twentyfour BIT STRING (SIZE (24)), thirtytwo BIT STRING (SIZE (32)), sixtyfour BIT STRING (SIZE (64))},

two-one-r19            CHOICE {twentyfour BIT STRING (SIZE (12)), thirtytwo BIT STRING (SIZE (16)), sixtyfour BIT STRING (SIZE (32))},

two-two-r19            CHOICE {twentyfour BIT STRING (SIZE (6)), thirtytwo BIT STRING (SIZE (8)), sixtyfour BIT STRING (SIZE (16))},

four-one-r19           CHOICE {twentyfour BIT STRING (SIZE (6)), thirtytwo BIT STRING (SIZE (8)), sixtyfour BIT STRING (SIZE (16))},

four-two-r19           CHOICE {thirtytwo BIT STRING (SIZE (4)), sixtyfour BIT STRING (SIZE (8))}

}

# Conclusion

Based on the discussion, we have the following proposals.

Observation 1: Different events, event evaluation, and reporting trigger are agreed for UEI beam reporting and LTM event- triggered L1 measurement reporting.

Observation 2: RAN1 is discussing details of event evaluation/triggering and will make decision on which layer to handle the procedure in November meeting. It is hard and not necessary for RAN2 to start a parallel discussion, which may also complicate the work at the current stage.

Proposal 2: RAN2 wait for more RAN1 progress on the event evaluation/triggering procedure.

Proposal 3: For Rel-19 type-I single-panel, type-II, type-II Doppler codebook, n1-n2 is configured as a separate parameter not coupled with CBSR, and CBSR is optionally configured as a CHOICE of (X1, X2). Take the below signaling structure as baseline.

# References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.

- R2-2404114, LS to RAN2 on CBSR for Rel-19 MIMO

- R1-2407285, LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5