3GPP TSG-RAN WG2 #128	R2-2410544

# Orlando, US, 18th – 22nd November 2024

Agenda Item:	8.6.2

Source:	Ericsson

Title:	Security handling and DC aspects for inter-CU LTM

Document for:	Discussion

# 1	Introduction

In this contribution, we want to discuss two important remaining aspects for inter-CU LTM:

- Security handling for both MCG and SCG

- The handling of SCG during inter-CU LTM for the MCG

# 2	Discussion

## 2.1	Handling of SCG during inter-CU LTM at the MN

In the RAN2#127 meeting, the following agreement has been taken for how to handle the SCG when an inter-CU LTM cell switch at the MN is executed:

SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

Based on this agreement, if the UE has an SCG at the moment an inter-CU LTM cell switch is executed at the MN, the SCG can be released, reconfigured, or kept as it is. Further, according to the agreement that RAN2 took, this also implies that when an inter-CU LTM cell switch is executed, there is also the possibility to add an SCG. Therefore, we propose:

RAN2 to confirm that when an inter-CU LTM cell switch is executed, the SCG addition is supported.

Given that now the SCG can be either added, released, or reconfigured, it is evident that this behavior is basically the same that we have for the legacy DC procedures that have been specified since Rel-15. However, in Rel-18 there is a restriction on how the SCG is handled in case an intra-CU LTM cell switch is triggered. In fact, when this happen the SCG can either be kept or released.

In Rel-18, in case an intra-CU LTM cell switch is triggered. In fact, when this happen the SCG can either be kept or released.

Since this restriction has been lifted in Rel-19, it would be good to clarify that also when an intra-CU LTM cell switch is executed, how to handle the SCG part is up to the network implementation (e.g., it is either released, configured or reconfigured).

The SCG configuration can be changed when an intra-CU LTM cell switch is executed and how to handle the SCG is up to the network implementation.

One aspect that needs to be considered, however, is whether the SCG configuration in case of reconfiguration (or addition, if supported) can be a delta configuration or always a full configuration. In case we allow the SCG configuration to be a delta of the current UE configuration on when LTM is configured, the following scenario can happen:

- Step 1: UE has an SCG and MN LTM candidate configuration ID 1 is added which has an SCG configuration that is a delta of the current SCG configuration of the UE, and MN LTM candidate configuration ID 2 is added which releases the current SCG configuration at the UE.

- Step 2: UE performs an MN LTM cell switch to LTM candidate configuration ID 2 which then releases the SCG.

- Step 3: UE performs an MN LTM cell switch to LTM candidate configuration ID 1 which potentially can add the SCG, but since this was a delta of the previous SCG configuration of the UE (which has been released by LTM candidate configuration 2) the UE considers the SCG configuration is invalid and triggers the SCG failure configuration procedure.

According to the example, for a MN LTM candidate configuration a simple way to avoid issues in the handling of the SCG would be to only allow the SCG configuration to be always a full configuration. Thus, we propose:

If the MN LTM candidate configuration includes an SCG configuration (to reconfigure or add the SCG during an LTM cell switch procedure at the MN), this SCG configuration is always a full configuration.

## 2.2	Security handling

### 2.2.1	Security for the MCG at inter-CU LTM

At RAN2#125bis, RAN2 sent an LS to SA3 [1] on security handling for inter-CU LTM for non-DC cases. Among the options identified by RAN2 and after that also in SA3, the main options are the following:

- Use new information in MAC CE to deliver the security info. Whether the UE uses horizontal or vertical derivation is derived from this new information in MAC CE (which is currently, neither integrity protected nor ciphered).

  - NCC value to use is included as MAC CE parameter to be used at inter-CU LTM execution.

  - UE is preconfigured with a NCC value list and association to the index in a ciphered and integrity protected message (in RRC), and the index of NCC is included as MAC CE parameter.

- After every inter-CU LTM cell switch execution, for vertical derivation based security change, using RRC, the UE is provided with the NCC to be used for the next inter-gNB CU LTM switch. This implies that every inter-CU LTM switch which is vertically derived security key based, needs a prior RRC message to inform the UE which NCC to use for this inter-CU LTM switch.

We have now received a reply from SA3 [2]. According to their reply, we can observe the following:

SA3 expressed security concern with NCC unprotected in MAC CE.

In our understanding, the security threat of unprotected NCC in the MAC CE in option 1 cannot be ignored. As the threat and potential solutions would need further analysis by SA3, it also requires more work in at least SA3.

Delivering the NCC within a MAC CE requires additional work in SA3 to solve security concern with NCC in MAC.

In order to send an NCC in the MAC CE, this NCC need to be fetched by the source gNB from the AMF before the LTM cell switch. How this should be performed is not clear yet. It would also require more work and standard impact as it will likely need new inter-node signaling procedure(s). As it may also have N2 and core network impact, multiple working groups would need to be involved, such as RAN2, RAN3 and SA2 and SA3.

Delivering the NCC within a MAC CE requires work in RAN3/SA2 to define new procedure to pre-fetch NCC, NH pair from AMF to target gNB ahead of LTM cell switch.

Another concern we have on option 1, is that as it relies on sending the NCC in a MAC CE during network triggered LTM switch, it is not clear how it would work when the UE triggers the switch, such as a fast LTM recovery or conditional LTM.

It is not clear that delivering the NCC in a MAC CE will work with fast LTM recovery or conditional LTM.

Option 4, on the other hand is based on how L3 handover is designed. As it reuses existing procedures and principles the impact is much smaller, and it does not introduce security concerns.

After every inter-CU LTM cell switch execution that changes the PDCP anchor, providing to UE the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch has no security concerns.

This means that once the UE receives the NCC value after an inter-CU LTM cell switch, it would be ready to perform another inter-CU LTM cell switch with vertical security derivation without any issues. Further, since the LTM candidate cell configurations will not be added/released/modified (unless the network wants to), the principle of subsequent LTM is also respected and thus this is also in line with the WID.

Providing the NCC after every inter-CU LTM cell switch execution guarantees that subsequent LTM can be used as the LTM candidate cell configuration don’t need to be added/released/modified.

A further aspect is that providing the NCC after every inter-CU LTM cell switch execution means that the legacy path switch procedure over NG interface can be re-used. If a new NCC, NH pair is received during the path switch procedure, the NCC value is delivered to the UE using RRC signalling before the UE may perform another inter-CU LTM cell switch.

Providing the NCC after every inter-CU LTM cell switch execution means that existing path switch procedures may be re-used to receive NCC,NH pair from AMF.

Therefore, it seems that providing the NCC after every inter-CU LTM cell switch execution (Option 4 of the LS from SA3) is a reasonable option to be used for inter-CU LTM. Thus, we propose:

RAN2 to agree on providing the NCC after every inter-CU LTM cell switch execution (preferred option mentioned by SA3).

Next step is to discuss how to provide the NCC to the UE after every inter-CU LTM cell switch execution. Currently, the NCC is provided to the UE within the MasterKeyUpdate IE:

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

The field MasterKeyUpdate is currently included on the top level of the RRCReconfiguration message. The presence of MasterKeyUpdate triggers security key update and thus if we re-use it for LTM it means that additional standardization effort would be needed to clarify the existing UE behavior. This is because the UE, in case of LTM, shall not trigger the security refresh but rather wait for the next inter-CU LTM cell switch to happen. According to this, there are two possible alternatives:

- Alt A: reuse the existing field within candidate RRCReconfiguration, by adding a condition that for LTM the security key update is only triggered when securityCellSetId is different from servingSecurityCellSetId, or

- Alt B: introduce a new masterKeyUpdateLTM field for LTM, which is processed by UE only if key update is required, e.g., if securityCellSetId is different from servingSecurityCellSetId.

Therefore, according to this we propose:

A new MasterKeyUpdateLTM is specified in order to deliver the NCC to the UE after every inter-CU LTM cell switch procedure.

### 2.2.2	How to provide sk-counter(s) for the SCG

The security at the SCG is handled according to the reception of the sk-counter from the MN. In this case, in the RAN2#127 meeting the following agreement has been reached:

RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

And at the RAN2#127bis meeting, this agreement was made:

The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

In the reply LS [3], SA3 states they share RAN2’s view that the subsequent CPAC security key update mechanism defined in TS 33.501 can be taken as the baseline.

The next step is to discuss how the list of sk-counters are provided to the UE. If we look at the signalling in Rel-18, for subsequent CPAC the MN provides a list of sk-counters which are linked to a particular SN and that are used only when execution of a subsequent CPAC configuration lead to that an inter-SN change or SCG addition is performed:

-- ASN1START

-- TAG-CONDITIONALRECONFIGURATION-START

ConditionalReconfiguration-r16 ::=   SEQUENCE {

attemptCondReconfig-r16              ENUMERATED {true}              OPTIONAL,   -- Cond CHO

condReconfigToRemoveList-r16         CondReconfigToRemoveList-r16   OPTIONAL,   -- Need N

condReconfigToAddModList-r16         CondReconfigToAddModList-r16   OPTIONAL,   -- Need N

...,

[[

scpac-ReferenceConfiguration-r18     SetupRelease {ReferenceConfiguration-r18}          OPTIONAL,   -- Need M

servingSecurityCellSetId-r18         SecurityCellSetId-r18                              OPTIONAL,   -- Need M

sk-CounterConfiguration-r18          SK-CounterConfiguration-r18                        OPTIONAL    -- Need M

]]

}

CondReconfigToRemoveList-r16 ::=     SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigId-r16

SK-CounterConfiguration-r18      ::= SEQUENCE {

sk-CounterConfigToReleaseList-r18    SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SecurityCellSetId-r18    OPTIONAL,    -- Need N

sk-CounterConfigToAddModList-r18     SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfig-r18     OPTIONAL     -- Need N

}

SK-CounterConfig-r18 ::=             SEQUENCE {

securityCellSetId-r18                SecurityCellSetId-r18,

sk-CounterList-r18                   SEQUENCE (SIZE (1..maxSK-Counter-r18)) OF SK-Counter

}

SecurityCellSetId-r18 ::= INTEGER (1.. maxSecurityCellSet-r18)

-- TAG-CONDITIONALRECONFIGURATION-STOP

-- ASN1STOP

For subsequent CPAC, each list of sk-counters is associated to one or more candidate configurations.

Given that the assumption of CPAC is that the MN does not change, for LTM the approach is different as the list of sk-counter is for the case when an inter-CU LTM cell switch is executed, and the security needs to be refreshed at the SCG.

For MCG inter-CU LTM the approach is different as the list of sk-counters is for the case when an MCG inter-CU LTM cell switch is executed, and the security needs to be refreshed at the SCG.

Since in LTM we also need to handle the case when the UE performs multiple LTM cell switches back and forth between the two same MNs, in this case it would make sense that each MN provide its own list of sk-counters, and that this list is included in the MN RRC reconfiguration message, thus inside of the LTM-Config IE.

Each MN LTM candidate configuration includes a list of sk-counters.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In Rel-18, in case an intra-CU LTM cell switch is triggered. In fact, when this happen the SCG can either be kept or released.

Observation 2	SA3 expressed security concern with NCC unprotected in MAC CE.

Observation 3	Delivering the NCC within a MAC CE requires additional work in SA3 to solve security concern with NCC in MAC.

Observation 4	Delivering the NCC within a MAC CE requires work in RAN3/SA2 to define new procedure to pre-fetch NCC, NH pair from AMF to target gNB ahead of LTM cell switch.

Observation 5	It is not clear that delivering the NCC in a MAC CE will work with fast LTM recovery or conditional LTM.

Observation 6	After every inter-CU LTM cell switch execution that changes the PDCP anchor, providing to UE the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch has no security concerns.

Observation 7	Providing the NCC after every inter-CU LTM cell switch execution guarantees that subsequent LTM can be used as the LTM candidate cell configuration don’t need to be added/released/modified.

Observation 8	Providing the NCC after every inter-CU LTM cell switch execution means that existing path switch procedures may be re-used to receive NCC,NH pair from AMF.

Observation 9	For subsequent CPAC, each list of sk-counters is associated to one or more candidate configurations.

Observation 10	For MCG inter-CU LTM the approach is different as the list of sk-counters is for the case when an MCG inter-CU LTM cell switch is executed, and the security needs to be refreshed at the SCG.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that when an inter-CU LTM cell switch is executed, the SCG addition is supported.

Proposal 2	The SCG configuration can be changed when an intra-CU LTM cell switch is executed and how to handle the SCG is up to the network implementation.

Proposal 3	If the MN LTM candidate configuration includes an SCG configuration (to reconfigure or add the SCG during an LTM cell switch procedure at the MN), this SCG configuration is always a full configuration.

Proposal 4	RAN2 to agree on providing the NCC after every inter-CU LTM cell switch execution (preferred option mentioned by SA3).

Proposal 5	A new MasterKeyUpdateLTM is specified in order to deliver the NCC to the UE after every inter-CU LTM cell switch procedure.

Proposal 6	Each MN LTM candidate configuration includes a list of sk-counters.

# 4	References

- R2-2403929, LS on security handling for inter-CU LTM in non-DC cases, RAN2

- S3-244316/ R2-2409534, Reply LS on security handling for inter-CU LTM in non-DC cases, SA3.

- S3-244317/ R2-2409535, Reply LS on security key update of inter-CU SCG LTM, SA3