3GPP TSG-RAN WG2 #128					      		 R2-2410569

Orlando, USA – November 18 - 22, 2024		                   Revision of R2-2407206

Agenda item:	8.13.3

Source: 	Kyocera

Title: 	Control Plane under multihop L2 U2N relaying

Document for:	Discussion

# Introduction

In this contribution, we consider the some of the key issues for control plane under L2 U2N multihop relaying.

# Discussion

Based on the discussions in RAN2#127, the following agreements have been reached:

Agreements:

The following connections are assumed as a baseline to be needed:

-	From last Relay UE perspective:

A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

-	From intermediate relay UE perspective (including first relay UE):

A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

FFS what RRC states are supported for the intermediate relay UE.

-	From U2N Remote UE perspective:

A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

Furthermore, a CP email discussion [1] has raised many issues which remains to be resolved, some of which are discussed below.

2.1.	Connection establishment

For the discussion of connection establishment procedure, one of the issues during the email discussion A1.2 [1] was “whether connection establishment by the remote UE must wait for connection establishment of each intermediate relay UE, and in sequence (i.e., Last relay UE triggers and completes connection establishment, then Intermediate relay UE completes connection establishment, and so on)”.  Although, the delay may be tolerable for one additional relay UE compared to Rel-17, it should be considered whether such a delay is really necessary, esp. since RAN2 has agreed that the intent of this WI is to minimize the impact of hop count on the multi-hop relay mechanisms.  If PC5 connections are already established among the UEs, the remote UE’s RRC Setup Request can be sent prior to the relay UEs’ RRC connection establishment.  In particular, we assume at least the remote UE should be allowed to send RRC Setup Request to the intermediate relay UE, to trigger the intermediate relay UE to trigger its own RRC Setup Request if it’s in IDLE/INACTIVE, which is consistent with the Rel-17 U2N relay UE’s behavior.

- Proposal 1	The remote UE should be allowed to send RRC Setup Request to the intermediate relay UE, regardless of the RRC State of the intermediate relay UE.

The remaining issue is to consider is whether the intermediate relay UE should wait for the last relay UE (in IDLE/INACTIVE) to transition to RRC CONNECTED before forwarding the remote UE’s RRC Setup Request (in addition to the intermediate relay UE’s own RRC Setup Request).  This issue is described in Figure 1. We could assume the relay discovery and relay selection process has been completed successfully (Steps 1 and 2 in Figure 1).

Figure 1: RRC Setup procedure with multihop relay UEs.

Once the relay UEs are selected for the desired path towards the NW, the PC5 connections for all the sidelink hops must also be established. Note that it is still FFS whether it is required for the relay UEs to establish PC5 connections/RRC connections prior to discovery transmission, which should be left for discovery/relay reselection discussions. If needed, discovery transmissions will occur after step 5 in Figure 1. Then it is assumed the remote UE will send the RRC Setup Request via a preconfigured SL-RLC0 via the selected relay UEs towards the serving cell (Step 3).  With regards to Step 4 in Figure 1, we assume the intermediate relay UE will need to forward the remote UE’s RRC Setup Request to the U2N relay UE, but it will also need to send its own RRC setup Request to the cell via the U2N relay UE, if it’s not yet in RRC CONN.    If we follow the legacy-like procedure, the intermediate relay UE would first send its own RRC Setup Request to the U2N relay UE, and similarly for the U2N relay (as needed if it’s not in RRC CONN). It may also be assumed that the intermediate relay UE will not receive the RRC Setup from the gNB, until the U2N relay UE is RRC CONN.  This would suggest that the intermediate relay UE will only forward the remote UE’s RRC Setup Request to the gNB via the U2N relay UE once it is in RRC CONN.  However, it could also be considered whether such a restriction is really necessary as the intermediate relay UE in IDLE/INACTIVE can still send the remote UE’s RRC Setup Request to the U2N relay UE via a specified SL RLC Channel (similar to the one used by the remote UE) to reduce remote UE’s latency in RRC connection establishment.

- Proposal 2	The intermediate relay UE should be allowed to forward the remote UE’s RRC Setup Request in case while it is still in RRC IDLE/INACTIVE.

Another issue that should be considered as part of RRC connection establishment procedure, is whether the gNB needs to know which relay UEs are associated with the remote UE’s RRC connection establishment, for the case when all the associated relay UEs are already in RRC CONN.  For example, in Figure 2, Remote UE 1 is already in RRC CONN, and the indirect path consists of two intermediate relay UEs followed by the U2N relay UE. Remote UE 2, currently in RRC IDLE wants to set up RRC connection with the gNB and has selected Intermediate relay UE 1 to connect with the gNB.  The current RRC Setup procedure would allow the U2N relay UE to forward the remote UE’s RRC Setup Request to the gNB; however, the gNB may not know which relay UEs are involved in this remote UE’s RRC Setup Request.

Figure 2: RRC Setup Request while all relay UEs are already in RRC CONNECTED.

- Proposal 3	RAN2 should consider how the gNB should be informed of the relay UEs associated with the RRC Setup Request from the remote UE, if all the relay UEs associated with the indirect path is already in RRC CONN.

2.1.2	Failure condition handling

The failure conditions for multihop relay should follow the same principles and failure reporting as for the legacy U2N failure conditions.  Currently, the NotificationMessageSidelink message already allows for certain indications to be sent from the relay UE to the remote UE.  These include the case for Uu RLF, relay UE HO, relay UE cell reselection, RRC failure, including establishment and resume failures. Also, for U2U relay, it was further added that indication of SL-RLF may also be indicated, which is particularly useful for the multihop relay UE scenario which wasn’t an issue with the legacy U2N relay scenario.  Additionally, the SidelinkUEInformationNR message allows the U2N relay UE to report the SL-RLF to the gNB.

- Observation 	The existing indications in the failure NotificationMessageSidelink message and SidelinkUEInformationNR message can be largely reused for the multihop relay scenario.

For the multihop relay scenario, although there’s only one Uu interface, there are multiple PC5 hops, which depends on the number of relay UE in the indirect path.  Therefore, when the U2N relay UE detects SL-RLF or is notified of the SL-RLF from a PC5 connected intermediate relay UE, it should inform the gNB not only of a SL-RLF, but also which relay UE detected the SL-RLF.  This will prevent the gNB from releasing a relay UE that is still has good connection to the gNB and continue to provide relay service to other remote UEs.

- Proposal 4	U2N relay UE should inform the gNB of the intermediate relay UE that detected the SL-RLF.

The same condition may also be considered for the indication sent to the remote UE.  In case the NotificationMessageSidelink message is sent to the remote UE, the relay UE should also include in the message the relay UE that was the cause for the indication.

- Proposal 5	The remote UE should be informed via the NotificationMessageSidelink message which relay UE is the cause for the indication.

2.1.2	RRC Connection Re-establishment

For multihop relay scenario, the baseline assumption should be similar to the legacy U2N case, i.e., upon the detection of SL-RLF, the U2N Remote UE may perform relay reselection and perform RRC reestablishment. However, with multihop relay, it is still undecided which UE should perform RRC reestablishment as point out in A1.6 in the email discussion [1]. The particular, it should be decided whether the intermediate relay UE or the remote UE or both UEs should perform RRC reestablishment, since in Rel-17 it was already assumed that the U2N relay UE does not initiate RRC reestablishment.  To avoid complexity at the NW for handling “duplicated” RRC reestablishment requests, we should not allow multiple UEs to initiate RRC reestablishment simultaneously.  Then it should be considered if the intermediate relay UE or the remote UE should perform the RRC reestablishment.   We consider two options for which UE should perform RRC reestablishment due to Uu RLF.

- The remote UE will initiate RRC reestablishment upon Uu RLF.  The relay UEs should inform the remote UE of the failure via notification messages.

- The intermediate UE will initiate RRC reestablishment upon receiving notification message from the last relay UE due to Uu RLF. The intermediate relay UE does not send notification message to the remote UE until the reestablishment procedure is completed (either successful or failure to reestablish).

With Option a), the RRC reestablishment procedure will remain a responsibility of the remote UE, which should result in less specification impact.

With Option b), since the RRC reestablishment is initiated at the intermediate relay UE, it is likely closer to the cell and should have a better chance of successful reestablishment. Furthermore, in case multiple remote UEs are connected to the intermediate relay UE, the intermediate relay UE’s reestablishment would benefit all the PC5 connected remote UEs. In case the RRC reestablishment is unsuccessful, the intermediate relay UE should send a notification message to the remote UE, which would trigger the remote UE to perform relay reselection and RRC reestablishment.

- Proposal 6	RAN2 should decide whether the remote UE or the intermediate relay UE should initiate RRC reestablishment upon Uu RLF.

# Conclusion

In this contribution, we addressed the issues that may arise with multihop relay on the control plane.  RAN2 is kindly asked to take into account the observation and proposals below:

- Proposal 1	The remote UE should be allowed to send RRC Setup Request to the intermediate relay UE, regardless of the RRC State of the intermediate relay UE.

- Proposal 2	The intermediate relay UE should be allowed to forward the remote UE’s RRC Setup Request in case while it is still in RRC IDLE/INACTIVE.

- Proposal 3	RAN2 should consider how the gNB should be informed of the relay UEs associated with the RRC Setup Request from the remote UE, if all the relay UEs associated with the indirect path is already in RRC CONN.

- Observation 	The existing indications in the failure NotificationMessageSidelink message and SidelinkUEInformationNR message can be largely reused for the multihop relay scenario.

- Proposal 4	U2N relay UE should inform the gNB of the intermediate relay UE that detected the SL-RLF.

- Proposal 5	The remote UE should be informed via the NotificationMessageSidelink message which relay UE is the cause for the indication.

- Proposal 6	RAN2 should decide whether the remote UE or the intermediate relay UE should initiate RRC reestablishment upon Uu RLF.

# References

- R2-24xxxxx, “Report of [POST127][402][Relay] Multi-hop relay control plane”, Interdigital