3GPP TSG-RAN WG2 Meeting #128	R2-2410572

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:		8.2.2

Source:		Futurewei

Title:		Discussions on Energy Status Indication for Ambient IoT

Document for:		Discussion and Decision

# Introduction

In meeting #127-bis, RAN2 reached the following agreements regarding the energy status indication [1]:

Agreements on energy bit indication

Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can.  1 bit indication can be captured in the TR.    We will capture the use case in the TR.   FFS whether it is reader controlled.

In this contribution, we discuss how the energy status indication is used.

# Discussions

## Definition of the energy status indication

The device needs to be able to complete the D2R transmission that carries the energy status indication (ESI) for the ESI to be successfully conveyed to the reader. Therefore, the ESI should be determined by the device (and interpreted by the reader) as a predicted energy status of the device at a future point in time, e.g., an earliest possible time for the device to monitor for R2D transmission and to transmit in response to a matching R2D trigger after the device finishes the current D2R transmission.

In RAN1 meeting #118, RAN1 has reached the following agreements [2]:

Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

In Direction 1, since the reader does not provide information as to when the device may become available/unavailable, the earliest possible time for the device to monitor for R2D transmission and to transmit in response to the matching R2D trigger, if the device still has the energy to do so, is TD2R_min after the device finishes the D2R transmission carrying the ESI. Therefore, the ESI in the D2R transmission should be determined by the device (and interpreted by the reader) as a predicted energy status of the device at the time TD2R_min after the device finishes the D2R transmission.

In Direction 2, the reader provides information in R2D message, based on which the device may become available/unavailable. For example, if the A-IoT paging message indicates that a device is not an intended device to participate in any access round in the current paging round, then the device can make itself unavailable for communications with the reader until the end of the current paging round by transitioning to the OFF state (or the SLEEP state) to harvest energy. For another example, if the A-IoT paging message indicates that a device is an intended device to participate in access round #N in the current paging round, then the device makes itself available for communications with the reader (by being in the ON state) during the period of access round #N while unavailable for communications (by being in the OFF state or the SLEEP state to harvest energy) during the periods of other access rounds, for which the device is not qualified to contend, until the end of the current paging round. When this device wins the contention for an access occasion within access round #N, in Msg3 and/or D2R message(s) that the device sends to the reader in response to command(s), the device includes the ESI to indicate its predicted energy status at the end of the current paging round, taking into account the energy to be harvested during the remaining unavailability period (e.g., from the end of access round #N to the end of the current paging round).

Because the ESI is only 1 bit, it can indicate only one of two possible states of the device after the current D2R transmission (for Direction 1) or after the end of the current paging round (for Direction 2), e.g.,

- whether the device has sufficient energy or not to make another D2R transmission, if being triggered to do so;

- whether the device will remain available or become unavailable for communications with the reader;

- whether the device will remain in the ON state or transition to the OFF state (or the SLEEP state).

In our view, the device having sufficient energy to make another D2R transmission, the device remaining available for communications, and the device remaining in the ON state after the current D2R transmission (for Direction 1) or after the end of the current paging round (for Direction 2) may result in similar actions. Meanwhile, the device having insufficient energy to make another D2R transmission, the device becoming unavailable for communications, and the device transitioning to the OFF state (or the SLEEP state) after the current D2R transmission (for Direction 1) or after the end of the current paging round (for Direction 2) may result in similar actions.

Proposal 1. The 1-bit ESI is used for indicating whether the device has sufficient energy or not to make another D2R transmission, i.e., whether the device will remain in the ON state and available for communications or will transition to the OFF state (or the SLEEP state) and become unavailable for communications after the current D2R transmission (for Direction 1) or after the end of the current paging round (for Direction 2).

## In which D2R messages the energy status indication is included

In our view, the ESI is included the following D2R messages:

- Msg3 sent in a 3-step CBRA procedure and subsequent D2R data transmission(s) (Step C2 as in Figure 6.3.1-1 in [3]), if any; and

- any D2R transmissions sent in a CFRA procedure.

In a 3-step CBRA procedure, the device sends Msg1 with an intention to send Msg3 if it wins the contention. It would be odd from protocol point of view and wasteful from resource point of view for a device to send Msg1 while indicating (in the Msg1) that the device has insufficient energy to send Msg3. Meanwhile, there is risk of collision on Msg1. Adding additional information, such as the ESI, in Msg1 will increase the penalty of collision. Therefore, the ESI should not be included in Msg1 sent in a 3-step CBRA procedure. If the device determines that it does not have sufficient energy to send Msg3 if sending Msg1 and receiving matching Msg2, the device should not initiate Msg1 in the first place. If the reader receives a Msg1 in a 3-step CBRA procedure, the reader should consider that the device sending the Msg1 has sufficient energy to send Msg3.

2-step CBRA procedure is intended for a quick D2R transmission of small data (such as the device ID) because if the data size is large, the collision probability will increase and the penalty of collision will become heavy. And because the device performs only one D2R transmission in 2-step CBRA procedure, the chance that the device’s energy will be depleted after transmitting Msg1 is low. If the 2-step CBRA has resulted in a collision, the reader should fall back to 3-step CBRA procedure, where the ESI is provided in Msg3. Therefore, the need for including the ESI in Msg1 sent in a 2-step CBRA procedure is not very strong either.

Proposal 2. The ESI is included in the following D2R messages:

- Msg3 sent in a 3-step CBRA procedure and subsequent D2R data transmission(s) (Step C2), if any; and

- any D2R transmissions sent in a CFRA procedure.

Proposal 3. The ESI is not to be included in Msg1 sent in a 3-step CBRA procedure.

Proposal 4. In a 3-step CBRA procedure, if the device determines that it does not have sufficient energy to send Msg3 if sending Msg1 and receiving matching Msg2, the device should not initiate Msg1 in the first place.

Proposal 5. If the reader receives a Msg1 in a 3-step CBRA procedure, the reader considers that the device sending the Msg1 has sufficient energy to send Msg3.

Proposal 6. RAN2 consider that the ESI is not to be included in Msg1 sent in a 2-step CBRA procedure either.

## Whether and how the presence of the ESI is controlled

One way for determining the presence of the ESI in a D2R transmission is based on the D2R message type, e.g., by mandating the inclusion of the ESI in all Msg3 sent in a 3-step CBRA procedure and subsequent D2R data transmission(s) (Step C2), if any, and in all D2R transmissions sent in a CFRA procedure, as proposed in P2, and mandating the exclusion of the ESI in all Msg1, as proposed in P3 and P6 together. In this way, no additional signaling overhead is needed for managing the presence of the ESI.

Proposal 7. The ESI is included in all Msg3 sent in a 3-step CBRA procedure and subsequent D2R data transmission(s) (Step C2), if any, and in all D2R transmissions sent in a CFRA procedure, and not included in any Msg1. The presence of the ESI is determined based on the D2R message type. No additional signaling is needed for indicating the presence of the ESI.

If P7 is not agreeable, we prefer that the inclusion of the ESI is controlled by the reader, e.g., the reader indicating, in the R2D trigger, whether the ESI is to be included in the pending D2R transmission or not. No D2R signaling is needed for indicating the presence/absence of the ESI. Compared to the alternative where the device decides and indicates the presence/absence of the ESI in the D2R transmission, first, the R2D indication is more reliable than the D2R indication. Secondly, using the R2D indication removes the uncertainty as to whether the ESI bit is included or not, for the reader to schedule the D2R resource efficiently. Finally, using the R2D indication also saves the device’s energy from transmitting the presence bit.

Proposal 8. If P7 is not agreeable, the inclusion of the ESI in a D2R transmission is controlled by the reader, e.g., the reader indicating, in the R2D trigger, whether the ESI is to be included in the pending D2R transmission or not. No D2R signaling is needed for indicating the presence of the ESI.

# Conclusions

We propose the following:

Proposal 1. The 1-bit ESI is used for indicating whether the device has sufficient energy or not to make another D2R transmission, i.e., whether the device will remain in the ON state and available for communications or will transition to the OFF state (or the SLEEP state) and become unavailable for communications after the current D2R transmission (for Direction 1) or after the end of the current paging round (for Direction 2).

Proposal 2. The ESI is included in the following D2R messages:

- Msg3 sent in a 3-step CBRA procedure and subsequent D2R data transmission(s) (Step C2), if any; and

- any D2R transmissions sent in a CFRA procedure.

Proposal 3. The ESI is not to be included in Msg1 sent in a 3-step CBRA procedure.

Proposal 4. In a 3-step CBRA procedure, if the device determines that it does not have sufficient energy to send Msg3 if sending Msg1 and receiving matching Msg2, the device should not initiate Msg1 in the first place.

Proposal 5. If the reader receives a Msg1 in a 3-step CBRA procedure, the reader considers that the device sending the Msg1 has sufficient energy to send Msg3.

Proposal 6. RAN2 consider that the ESI is not to be included in Msg1 sent in a 2-step CBRA procedure either.

Proposal 7. The ESI is included in all Msg3 sent in a 3-step CBRA procedure and subsequent D2R data transmission(s) (Step C2), if any, and in all D2R transmissions sent in a CFRA procedure, and not included in any Msg1. The presence of the ESI is determined based on the D2R message type. No additional signaling is needed for indicating the presence of the ESI.

Proposal 8. If P7 is not agreeable, the inclusion of the ESI in a D2R transmission is controlled by the reader, e.g., the reader indicating, in the R2D trigger, whether the ESI is to be included in the pending D2R transmission or not. No D2R signaling is needed for indicating the presence of the ESI.

# References

R2_127bis_ChairNotes_24-10-18_13-00.

R1-2407479, Chair notes RAN1#118 (9.4 R19 Ambient IoT) v08 (eom), Ad-Hoc Chair (Huawei).

R2-2407984, TP for TR 38.769 update, Huawei, CMCC, T-Mobile USA.