3GPP TSG-RAN WG2 Meeting #128	R2-2410588

Orlando, USA, Nov. 18th - 22nd, 2024

Agenda Item:	 8.13.3

Source:	 Huawei, HiSilicon

Title:	 Control Plane Procedures for Multi-hop Relay

Document for:  Discussion and Decision

# 1	Introduction

During the RAN 2# 127 meeting we had discussed the control plane procedure for the multi-hop relay for the first time and had made the following agreements.

- The following connections are assumed as a baseline to be needed: - From last Relay UE perspective: - A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and - A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED. - From intermediate relay UE perspective (including first relay UE): - A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and - An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED. - FFS what RRC states are supported for the intermediate relay UE - From U2N Remote UE perspective: - A direct (non-relayed) PC5 connection with Intermediate Relay UE, and - An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED. - The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18)

In addition to this we had a [POST127][402][Relay] Multi-hop relay control plane. During the email discussion, two approach/architecture were considered:

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

In this contribution, we discuss the details of the key issues for the two approaches and provide comparison of the two approaches that would be useful in deciding to down select one of the two approaches during this meeting.

# 2	Discussion

## 2.1	Approach 1 (Intermediate Relay UEs are in RRC Connected)

Conclusion of the phase 1 of the email discussion had the following proposal for the further discussion as the baseline for Approach 1

Proposal 1 – In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED. Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED. FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE. - The U2N Remote UE, First Relay UE, Intermediate Relay UE, and Last Relay UE perform discovery procedure, and establish a PC5-RRC connection between each adjacent UE (U2N Remote UE<->First Relay UE, First Relay UE <-> Intermediate Relay UE, Intermediate Relay UE <-> Last Relay UE) using the NR sidelink PC5 unicast link establishment procedure. [FFS whether to support PC5-RRC connection establishment between some adjacent UEs after transmission of the first RRC message in step 2.] - The L2 U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the First Relay UE, using a specified PC5 Relay RLC channel configuration. The first Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the First Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment via the Intermediate Relay UE (using similar actions as a U2N Remote UE) upon reception of a message from U2N Remote UE on the specified PC5 Relay RLC channel. The Intermediate Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the Intermediate Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment via the Last Relay UE (using similar actions as a U2N Remote UE) upon reception of a message from the First Relay UE on the specified PC5 Relay RLC channel. The Last Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the Last Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment upon reception of a message from the Intermediate Relay UE on the specified PC5 Relay RLC channel. In each of the previous sub-steps, if a given relay UE and its parent relay UE both need to enter RRC_CONNECTED, the given relay UE cannot do so until the parent relay UE has completed its own RRC connection establishment. The Last Relay UE receives SRB0 relaying Uu Relay RLC channel configuration for the Intermediate Relay UE from gNB. The Intermediate Relay UE receives SRB0 relaying Relay RLC channel configuration for the First Relay UE from gNB. The gNB configures SRB0 (for U2N Remote UE) relaying RLC channel to the first Relay UE. The gNB responds with an RRCSetup message to U2N Remote UE. The RRCSetup message is sent to the U2N Remote UE using SRB0 relaying Last Relay RLC channel over Uu and the specified PC5 Relay RLC channels over each of the PC5 links. [FFS whether the Last Relay UE can send SUI on behalf of all other relay UEs.] - The gNB, Last Relay UE, Intermediate Relay UE and First Relay UE perform relaying channel setup procedure over Uu. According to the configuration from the gNB, the First Relay/U2N Remote UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the U2N Remote UE/First Relay UE over PC5, the Intermediate Relay/First Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the First Relay UE/Intermediate Relay UE over PC5 and the Last Relay UE/ Intermediate Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the Intermediate Relay UE/Last Relay UE over PC5. [FFS if each relay UE can establish RLC channel for relaying of SRB1 at the same time as its connection establishment in step 2]. - The RRCSetupComplete message is sent by the U2N Remote UE to the gNB via the First Relay UE, Intermediate Relay UE and the Last Relay UE using SRB1 relaying channels over PC5 and SRB1 relaying channel configured to the Last Relay UE over Uu. Then the U2N Remote UE is RRC_CONNECTED with the gNB. - The L2 U2N Remote UE and gNB establish security following the Uu security mode procedure and the security messages are forwarded through the First Relay UE, Intermediate Relay UE, and Last Relay UE. - The gNB sends an RRCReconfiguration message to the U2N Remote UE via the Last Relay UE, Intermediate Relay UE, and First Relay UE to setup the end-to-end SRB2/DRBs of the U2N Remote UE. The U2N Remote UE sends an RRCReconfigurationComplete message to the gNB via the First Relay UE, Intermediate Relay UE, and Last Relay UE as a response. In addition, the gNB may configure additional Uu Relay RLC channels between the gNB and Last Relay UE, and PC5 Relay RLC channels between each of the Intermediate Relay UE, First Relay UE, and U2N Remote UE for the relaying traffic. Proposal 2 – The figure and description above serves as a baseline connection establishment procedure for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted.

### 2.1.1	Analysis for Approach 1

Signalling flow for Connection Establishment Procedure

If we carefully look at the Work item description it can be seen that the main justification of the work item is that single hop U2N relays that have been developed in Rel-17 and Rel-18 have limited applicability because of the range limitation of a single hop sidelink relay and this needs to be enhanced to support multi-hop UE-to-Network Relay

## 3 Justification 3GPP RAN completed a work item “NR Sidelink Relay” in Rel-17 and a work item “NR sidelink relay enhancements” in Rel-18 in order to cover the enhancements and solutions necessary to support the UE-to-network Relay coverage extension which consider a wide range of V2X, Public Safety and commercial applications and services. The 5GS has been also enhanced to support Proximity Services in Rel-17 and Rel-18 as specified in TS 23.304, based on the service requirements defined in TS 22.278, TS 22.261 and TS 22.115. However, single hop U2N relays that have been developed in Rel-17 and Rel-18 have limited applicability because of the range limitation of a single hop sidelink relay. Thus, additional sidelink relay enhancements are needed to support some remaining (and new) requirements for Proximity Services captured in TS 22.278 and TS 22.261, e.g. multi-hop support for UE-to-Network Relay. A study item for ProSe phase 3 has been approved in SA in order to investigate further 5G system enhancements to support Proximity Services in Rel-19. RAN-side enhancements for sidelink relay, particularly multi-hop Layer-2 UE-to-Network relay, is necessary in accordance with the SA work.

It can be seen that Approach 1 is aligned with the work item description in justification section where the main intention is to extend the range limitation of the single hop sidelink relay based on the U2N relay legacy procedures. The Signalling flow for Connection Establishment Procedure for multi hop U2N relay using approach 1 and the corresponding steps for it are the natural extension of R17 Connection Establishment Procedure for single hop U2N relay procedure and should be adopted for the R19 Multi-hop procedures. Hence we have the following proposals

Observation 1: It can be seen that Approach 1 is aligned with the work item description in justification section where the main intention is to extend the range limitation of the single hop sidelink relay based on the legacy U2N relay procedures.

Proposal 1: The Signalling flow for Connection Establishment Procedure for multi hop U2N relay using approach 1 and the corresponding steps for it are the natural extension of R17 Connection Establishment Procedure for single hop U2N relay procedure and should be adopted for the R19 Multi-hop procedures.

Latency for Connection Establishment Procedure

Couple of companies have indicated during the email discussion that main drawback of Approach 1 is that latency of remote UE connection establishment increases as the number of hops increases with Approach 1. However, we believe latency increase is not specific to Approach 1 and increase in latency is also an issue for Approach 2 where Intermediate Relay UEs are in RRC IDLE/INACTIVE state. The data transmission from the remote UE needs to wait after the UE ID allocation/QoS split/bearer configuration for both the approaches. The difference is that there will be an increased Uu signalling in Approach 1, that is controlled by the network, while there will be increased SL signalling in Approach 2 among the relay UEs due to local ID assignment and QoS split.

The signalling flow for Connection Establishment Procedure for multi hop U2N relay using approach 1 in the figure above depicts the worst-case scenario where all the relay UEs participating in the multi hop connection establishment are in RRC IDLE/INACTIVE state and the remote UE triggers the connection establishment via these RRC IDLE/INACTIVE relay UEs. An important point to note is that this scenario might happen quite infrequently when the Remote UE triggers connection establishment while all the relay UEs are RRC IDLE/INACTIVE. Whereas a more frequent scenario in practice would be that the last and intermediate relay UEs are already in connected state and have established the connection to the network for their own traffic and then they start serving the remote UE. Furthermore, the latency problem can be easily solved by a simple solutions. For example, the remote UE can select RRC CONNCETED relay UE with high priority if the service that has stringent requirement, or we can have some restriction on the maximum hop number between the RRC IDLE/INACTIVE relay UEs and the network.

Hence we have the following observations.

Observation 2: Latency increase is not specific to Approach 1. The data transmission from the remote UE needs to wait after the UE ID allocation/QoS split/bearer configuration for both the approaches There will be an increased Uu signalling in Approach 1, that is controlled by the network,. For approach 2, there will be increased SL signalling among the relay UEs due to local ID assignment and QoS split.

Observation 3: The signalling flow for Connection Establishment Procedure for multi hop U2N relay using approach 1 depicts the worst-case scenario where all the relay UEs participating in the multi hop connection establishment are in RRC IDLE/INACTIVE state when the remote UE triggers the connection establishment via these RRC IDLE/INACTIVE relay UEs. However, this scenario might not happen quite frequently in practice.

Observation 4: A more frequent scenario in practice would be that the last and intermediate relay UEs are already in connected state and have established the connection to the network for their own traffic before they start serving the remote UEs

Proposal 2: Latency increase is not specific to Approach 1 and shall not be a show stopper for adopting Approach 1 for the multi hop relay in Rel 19. The latency can be improved for Approach 1 if it is adopted for multi hop relay.

Need for DRB Establishment for Relay UEs

Some companies have indicated that an intermediate relay UE needs to be configured with Uu SRAP configuration at least for the default DRB/DRB and SRB.

We don’t think we need any DRBs or default DRB for the relay UEs to be in RRC connected State when the remote UE triggers the connection establishment procedure while all the relay UEs are in RRC IDLE state. These relay UEs can be moved to RRC CONNECTED state by the network just to serve the remote UE without having their own data to be transferred to the network.

Alternatively, in other scenario,  intermediate relay UE may first get connected as a remote UE to transfer their own data via the last relay UE and go through the DRB establishment procedure and then start serving any remote or other intermediate relay UEs in the vicinity for relaying their data.

We don’t see any issues with these two scenarios

Proposal 3: No DRBs or default DRB needs to be established for the last or intermediate relay UEs to move them from RRC IDLE to RRC Connected State before serving the remote UE when the remote UE triggers the connection establishment procedure

Failures during connection establishment

Some companies have indicated that Remote UE or an intermediate relay UE may fail to set up its RRC_CONNECTION, due to its parent relay UE (s) taking too long time to setup their RRC_CONNECTIONS. We believe the failures can be resolved with extended timers for multi hop. T300 will have to be increased for multi-hop U2N relay operation in accordance with the hop count.

Proposal 4: Extend the T300 for multi-hop U2N relay operation in accordance with the hop count to avoid any failures during connection establishment procedure

## 2.2	Approach 2 (Intermediate Relay UEs are in RRC IDLE/INACTIVE)

Conclusion of the phase 1 of the email discussion had the following proposal for the further discussion as the baseline for Approach 2

Proposal 3 – In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in RRC_IDLE/RRC_INACTIVE when the U2N remote UE is in RRC_CONNECTED. Proposal 4 – In approach 2, any relay UE which happens to be in RRC_CONNECTED can obtain its relaying RLC channel configuration in dedicated signalling. - The U2N Remote UE, First Relay UE, Intermediate Relay UE, and Last Relay UE perform discovery procedure, and establish a PC5-RRC connection between each adjacent UE (U2N Remote UE<->First Relay UE, First Relay UE <-> Intermediate Relay UE, Intermediate Relay UE <-> Last Relay UE) using the NR sidelink PC5 unicast link establishment procedure. - The L2 U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the First Relay UE, using a specified PC5 Relay RLC channel configuration. If the First Relay UE is in RRC_CONNECTED, it sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. Otherwise, it obtains the configuration from SIB or preconfiguration, or from the remote UE’s serving gNB. If the Intermediate Relay UE is in RRC_CONNECTED, it sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. Otherwise, it obtains the configuration from SIB or preconfiguration. The Last Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the relay operation for the U2N Remote UE. If the Last Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment upon reception of a message on the specified PC5 Relay RLC channel. After the Last Relay UE's RRC connection establishment procedure and sending the SidelinkUEInformationNR message, gNB configures SRB0 relaying Uu Relay RLC channel to the Last Relay UE. The gNB responds with an RRCSetup message to U2N Remote UE. The RRCSetup message is sent to the U2N Remote UE using SRB0 relaying Last Relay RLC channel over Uu and the specified/preconfigured PC5 Relay RLC channels over each of the PC5 links. FFS which option to obtain the relaying configuration (SIB/preconfiguration or the remote UE’s serving gNB) is used by relay UEs which remain in IDLE/INACTIVE while the remote UE is in RRC_CONNECTED. - According to (pre)configuration, the First Relay/U2N Remote UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the U2N Remote UE/First Relay UE over PC5, the Intermediate Relay/First Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the First Relay UE/Intermediate Relay UE over PC5 and the Last Relay UE/ Intermediate Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the Intermediate Relay UE/Last Relay UE over PC5. - The RRCSetupComplete message is sent by the U2N Remote UE to the gNB via the First Relay UE, Intermediate Relay UE and the Last Relay UE using SRB1 relaying channels over PC5 and SRB1 relaying channel configured to the Last Relay UE over Uu. Then the U2N Remote UE is RRC_CONNECTED with the gNB. - The L2 U2N Remote UE and gNB establish security following the Uu security mode procedure and the security messages are forwarded through the First Relay UE, Intermediate Relay UE, and Last Relay UE. - The gNB sends an RRCReconfiguration message to the U2N Remote UE via the Last Relay UE, Intermediate Relay UE, and First Relay UE to setup the end-to-end SRB2/DRBs of the U2N Remote UE. The U2N Remote UE sends an RRCReconfigurationComplete message to the gNB via the First Relay UE, Intermediate Relay UE, and Last Relay UE as a response. In addition, the gNB may configure additional Uu Relay RLC channels between the gNB and Last Relay UE, and PC5 Relay RLC channels between each of the Intermediate Relay UE, First Relay UE, and U2N Remote UE for the relaying traffic. Proposal 5 – The figure and description above serves as a baseline connection establishment procedure for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted.

### 2.2.1	Analysis for Approach 2

Signalling flow for Connection Establishment Procedure

The Signalling flow for Connection Establishment Procedure for multi hop U2N relay using approach 2 and the steps for it are not inherited/aligned with the R17 Connection Establishment Procedure for single hop U2N relay procedure and requires multiple complex enhancements for supporting the R19 Multi-hop procedures.

Although the signalling flow for Approach 2 may seem simpler compared to Approach 1, there are many hidden issues which are currently unknown and will require more complex procedures among multiple entities.

Observation 5: The Signalling flow for Connection Establishment Procedure for multi hop U2N relay using approach 2 and the steps for it are not inherited/aligned with the R17 Connection Establishment Procedure for single hop U2N relay. Furthermore, it requires multiple complex enhancements for supporting the R19 Multi-hop procedures.

Allocation of local UE ID

If we rely on gNB to allocate UE ID, the allocation of UE ID will require the last relay UE to move into RRC CONNECTED state and reports all the UE context along the path to the network.

If we allow the last relay UE to establish the RRC connection, new trigger condition is needed since there is no RRC Setup Request message from the intermediate relay. In addition, it is unclear how the last relay UE will get all the other UEs information (e.g., L2 ID) and report to network especially considering there is no E2E PC5 RRC connection between non-adjacent UEs. Furthermore, it is unclear how to configure the SRAP entity for each intermediate Relay UE in RRC_IDLE/INACTIVE toward the Remote UE.

The risk for collusion of the UE ID exist even when the UE ID is allocated by gNB. For example, when the intermediate relay UE is in IDLE/INACTIVE, it may provide the relay service between different remote UE with different last relay UE. If the last relay UE are connected to the different gNBs, the intermediate relay UE may receive the same local ID for different remote UE from different gNBs.

If we depend on relay UE to allocate UE ID, the risk that the collision will happen is much higher compared to the case when it is allocated by gNB.

Observation 6: Allocation of local UE ID is extremely complex with Approach 2 and will require new trigger conditions at the last relay UE to move into RRC CONNECTED state and reports all the UE context along the path to the network.

Observation 7:- In comparison UE ID allocation for Approach 1 is straight forward and does not require any complex procedures. as in Approach 2.

RLC channel configuration for a relay UE

How to configure the RLC channel configuration for a relay UE that is in RRC_IDLE/RRC_INACTIVE for approach 2 is unclear. Different RRC state and coverage states needs to be discussed/captured separately. Additionally, whether the remote/relay UEs are in same or in different cell needs to be discussed/captured separately.

These solutions will be extremely complex and are unnecessary for R19.

Observation 8: How to configure the RLC channel configuration for a relay UE that is in RRC_IDLE/RRC_INACTIVE for approach 2 is unclear. Different RRC state and coverage states needs to be discussed/captured separately. Additionally, whether the remote/relay UEs are in same or in different cell needs to be discussed/captured separately.

Observation 9 : In comparison Signalling of RLC channel configuration for a relay UEs in Approach 1 is straight forward and does not require any complex procedures as in Approach 2.

Complex PC5 Procedures for QoS split and Local UE ID allocation

### Even if we assume that the relay UE will some how will be able to configure local UE ID or performs the QoS splitting for approach 2 , it will need to maintain the UE context of each UE/each link of the multi-hop there will be significantly complex new PC5 signalling procedures needed to for coordination/configuration/collision resolution/avoidance, especially considering there is no E2E PC5 connection between non-adjacent UEs. Design of such complex procedures to support Approach 2 is unnecessary and inefficient.

Additionally, for QoS split, since the intermediate relay UE can be in IDLE/INACTIVE or can be in a cell of different gNB i.e. not the one controlling the last relay UE, there is no central node to split QoS for each hop, which will be complex. For example, the gNB will first split the QoS into Uu hop part and the part between the last relay UE and the remote UE. Then the all the intermediate relay UEs need to participate in the QoS splitting procedure to determine the split QoS on each hop between the adjacent UEs, which can in turn introduce a large delay.

Observation 10: - Significantly complex new PC5 signalling procedures will have to be designed for coordination/ configuration/ collision resolution/avoidance/QoS Split for Approach 2.

Limitations for supporting Service Continuity Scenarios C and D

The WI objective states that

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

A serious limitation of approach 2 is that Service Continuity Scenario C and D cannot be supported with approach 2 as relay UEs can be in different cell/ different gNB and bringing them to connected state will be extremely complex requiring inter gNB signalling.

Path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell cannot be achieved by Approach 2 as the gNB has no control over the relay UE which might be connected to different gNB

Observation 11:  A serious limitation of approach 2 is that Service Continuity Scenario C and D cannot be supported with approach 2 as relay UEs can be in different cell/ different gNB and bringing them to connected state will be extremely complex.

Considering Observation 5 to 11 it is evident that procedures required to support Approach 2 are extremely complex and should not be adopted for Rel- 19 Multi hop relays.

Proposal 5: Considering observation 5 to 11, it is evident that the procedures required to support Approach 2 are extremely complex. Given the fact that Approach 1 is the natural extension of R17 U2N Relay and can support all of these procedures without any complexity, RAN 2 agrees not to adopt Approach 2 for Rel- 19 Multi hop relays

# 3	Conclusion

Approach 1 (Intermediate Relay UEs are in RRC Connected)

Signalling flow for Connection Establishment Procedure

Observation 1: It can be seen that Approach 1 is aligned with the work item description in justification section where the main intention is to extend the range limitation of the single hop sidelink relay based on the legacy U2N relay procedures.

Proposal 1: The Signalling flow for Connection Establishment Procedure for multi hop U2N relay using approach 1 and the corresponding steps for it are the natural extension of R17 Connection Establishment Procedure for single hop U2N relay procedure and should be adopted for the R19 Multi-hop procedures.

Latency for Connection Establishment Procedure

Observation 2: Latency increase is not specific to Approach 1. The data transmission from the remote UE needs to wait after the UE ID allocation/QoS split/bearer configuration for both the approaches There will be an increased Uu signalling in Approach 1, that is controlled by the network,. For approach 2, there will be increased SL signalling among the relay UEs due to local ID assignment and QoS split.

Observation 3: The signalling flow for Connection Establishment Procedure for multi hop U2N relay using approach 1 depicts the worst-case scenario where all the relay UEs participating in the multi hop connection establishment are in RRC IDLE/INACTIVE state when the remote UE triggers the connection establishment via these RRC IDLE/INACTIVE relay UEs. However, this scenario might not happen quite frequently in practice.

Observation 4: A more frequent scenario in practice would be that the last and intermediate relay UEs are already in connected state and have established the connection to the network for their own traffic before they start serving the remote UEs

Proposal 2: Latency increase is not specific to Approach 1 and shall not be a show stopper for adopting Approach 1 for the multi hop relay in Rel 19. The latency can be improved for Approach 1 if it is adopted for multi hop relay.

Need for DRB Establishment for Relay UEs

Proposal 3: No DRBs or default DRB needs to be established for the last or intermediate relay UEs to move them from RRC IDLE to RRC Connected State before serving the remote UE when the remote UE triggers the connection establishment procedure

Failures during connection establishment

Proposal 4: Extend the T300 for multi-hop U2N relay operation in accordance with the hop count to avoid any failures during connection establishment procedure

Approach 2 (Intermediate Relay UEs are in IDLE/INACTIVE)

Proposal 5: Considering observation 5 to 11, it is evident that the procedures required to support Approach 2 are extremely complex. Given the fact that Approach 1 is the natural extension of R17 U2N Relay and can support all of these procedures without any complexity, RAN 2 agrees not to adopt Approach 2 for Rel- 19 Multi hop relays

Signalling flow for Connection Establishment Procedure

Observation 5: The Signalling flow for Connection Establishment Procedure for multi hop U2N relay using approach 2 and the steps for it are not inherited/aligned with the R17 Connection Establishment Procedure for single hop U2N relay. Furthermore, it requires multiple complex enhancements for supporting the R19 Multi-hop procedures.

Allocation of local UE ID

Observation 6: Allocation of local UE ID is extremely complex with Approach 2 and will require new trigger conditions at the last relay UE to move into RRC CONNECTED state and reports all the UE context along the path to the network.

Observation 7:- In comparison UE ID allocation for Approach 1 is straight forward and does not require any complex procedures. as in Approach 2.

RLC channel configuration for a relay UE

Observation 8: How to configure the RLC channel configuration for a relay UE that is in RRC_IDLE/RRC_INACTIVE for approach 2 is unclear. Different RRC state and coverage states needs to be discussed/captured separately. Additionally, whether the remote/relay UEs are in same or in different cell needs to be discussed/captured separately.

Observation 9 : In comparison Signalling of RLC channel configuration for a relay UEs in Approach 1 is straight forward and does not require any complex procedures as in Approach 2.

Complex PC5 Procedures for QoS split and Local UE ID allocation

Observation 10: - Significantly complex new PC5 signalling procedures will have to be designed for coordination/ configuration/ collision resolution/avoidance/QoS Split for Approach 2.

Limitations for supporting Service Continuity Scenarios C and D

Observation 11:  A serious limitation of approach 2 is that Service Continuity Scenario C and D cannot be supported with approach 2 as relay UEs can be in different cell/ different gNB and bringing them to connected state will be extremely complex.

# 4	Reference

- R2-2410006, Report of [POST127][402][Relay] Multi-hop relay control plane.

- R2-2410305, Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection

- Session Chair Notes on Relay and Positioning RAN2 #127bis;